3GPP TSG RAN4 Meeting #80bis	R4-168344

Ljubljana, Slovenia, 10 – 14 October, 2016

Title: 	Impact of adding 3MHz BW into B3 in CA_1A-3A, CA_3A-40A, and CA 3A-41A

Source: 	Huawei, HiSilicon

Agenda item:	8.2.2

Document for:	Other (approval)

# 1	Introduction

This contribution discusses the impact of adding 3MHz CC BW into three 1UL/2DL CA cases, CA_1A-3A, CA_3A-40A, and CA_3A-41A.

# 2	Discussion

All these CA combinations have low-proximity characteristics i.e. the cross-band isolation is not sufficient to meet 0dB MSD. The well-known practice in RAN4 has been to use analysis results from all interested companies as basis for deciding appropriate MSD for each BW in each CA combination. Now that the MSD’s defined in TS36.101 is based on multiple analyses it is fairly difficult to provide analysis that matches well for the case when a new BW is added.

We did analyze the MSD for each of these CA combinations without 3MHz BW on B3. It took a while to make the MSD results to match adequately well with the current numbers in the specification because, as said earlier, they are based on numbers from several companies. After the results matched well-enough then we analyzed MSD for 3MHz B3 BW and in addition analyzed the impact to B40/B41 MSD due to the slightly larger IP2 noise. This was a very extensive exercise but we believe it was important in order to make the analysis better represent “all” analysis that were used to derive the original CA REFSENS numbers.

We emphasize that the purpose for these analyses is only to find out the MSD for 3MHz B3 CC and to check if the slightly larger IP2 noise impacts B40 and B41 MSD’s. The purpose of this analysis is not to change any of the numbers in specification outside of the two aspects.

The architectures for these combinations are the same as they were when the original CA REFSENS values were calculated.

CA_1A-3A:

The following parameters were used [1]:

| Cross-band isolation | 50dB |
| --- | --- |
| Band 1 PA noise | -122 dBm/Hz |
| IIP2 | 48 dB |
| RFIC TX noise@RX | -148.5 dBc/Hz |
| Rx LO phase noise | -150 dBc/Hz |
| B1 Front-end loss | 6.2dB |
| B3 Front-end loss | 6.2dB |
| Thermal noise at LNA | -169.5dBm/Hz |

The respective REFSENS results are:

| CA_1A-3A4 | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | -100 | -97 | -95.2 | -94 |
|  | 3 | -95 | -94 | -91.5 | -90 | -88.9 |

The results are very well aligned with the numbers in TS36.101

| CA_1A-3A4 | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | -100 | -97 | -95.2 | -94 |
|  | 3 |  | -94 | -91.5 | -90 | -89 |

The only open issue for this CA combination is the REFSENS value for 3MHz B3. We propose to use -95dBm.

Proposal1: For CA_1A-3A, the REFSENS value for 3MHz B3 CC shall be -95dBm

| CA_1A-3A4 | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | -100 | -97 | -95.2 | -94 |
|  | 3 | -95 | -94 | -91.5 | -90 | -88.9 |

CA_3A-40A:

The following parameters were used [2], [3]

| PA spectrum regrowth Rx noise | -135dBm/Hz |
| --- | --- |
| RFIC IIP2 | 48dB |
| B40 Front-end loss | 7.2dB |
| B3 Front-end loss | 6.7dB |
| B3 Duplexer Tx attenuation @ B40 | 31dB |
| B3 Duplexer Rx attenuation @ B40 | 40dB |
| B40 band filter attenuation @ B3 Tx | 29dB |
| B40 band filter attenuation @ B3 Rx | 28dB |
| B3 RX LO phase noise | -154dBc/Hz |
| B40 RX LO phase noise | -154dBc/Hz |
| RFIC TX noise@RX | -151dBc/Hz |
| Diplexer ISO @ B40 | 10dB |
| Diplexer ISO @ B3 | 13dB |
| Thermal noise at LNA | -170dBm/Hz |

The respective REFSENS results are:

| CA_3A-40A | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 40 |  | -95.3 | -92.7 | -91.1 | -90.2 |
| CA_3A-40A | 3 | -95.3 | -94.1 | -91.4 | -89.8 | -88.7 |
|  | 40 |  | -100 | -97 | -95.2 | -94 |

The results are again fairly well aligned with the current numbers in TS36.101.

| CA_3A-40A | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 |  | -97 | -94 | -92.2 | -91 |
|  | 40 |  | -95.4 | -92.9 | -91.3 | -90.2 |
| CA_3A-40A | 3 |  | -94.2 | -91.2 | -89.5 | -88.3 |
|  | 40 |  | -100 | -97 | -95.2 | -94 |

We propose to use -95.3dBm for 3MHz B3 REFSENS.

Proposal2: For CA_3A-40A, the REFSENS value for 3MHz B3 CC shall be -95.3dBm.

| CA_3A-40A | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 40 |  | -95.3 | -92.7 | -91.1 | -90.2 |
| CA_3A-40A | 3 | -95.3 | -94.2 | -91.2 | -89.5 | -88.3 |
|  | 40 |  | -100 | -97 | -95.2 | -94 |

CA_3A-41A:

The following parameters were used [4], [5]:

| PA spectrum regrowth Rx noise (dBm/Hz) | -135dBm/Hz |
| --- | --- |
| RFIC IIP2 (dBm) | 48dB |
| B41 FE Loss (dB) | 7.6dB |
| B3 FE Loss (dB) | 6.7dB |
| B3 Duplexer Tx attenuation @ B41 (dB) | 40dB |
| B3 Duplexer Rx attenuation @ B41 (dB) | 40dB |
| B41 band filter attenuation @ B3 Tx | 26dB |
| B41 band filter attenuation @ B3 Rx | 22dB |
| B3 Rx LO phase noise (dBc/Hz) | -154dBc/Hz |
| B41 RX LO phase noise (dBc/Hz) | -154dBc/Hz |
| RFIC TX noise @RX | -151dBc/Hz |
| Diplexer ISO @ B41 (dB) | 15dB |
| Diplexer ISO @ B3 (dB) | 15dB |
| B41 thermal noise at LNA | -169.2dBm/Hz |
| B3 thermal noise at LNA | -170dBm/Hz |

The respective REFSENS results are:

|  | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A5 | 3 | [-95.3] | [-93.9] | [-90.9] | [-89.1] | [-87.9] |
|  | 41 |  | -97.5 | -94.5 | -92.7 | -91.5 |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 41 |  | [-93.3] | [-90.9] | [-89.5] | [-88.4] |

The results are again fairly well aligned with the current numbers in TS36.101.

|  | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A5 | 3 |  | [-94] | [-91] | [-89.2] | [-87.9] |
|  | 41 |  | -97.5 | -94.5 | -92.7 | -91.5 |
|  | 3 |  | -97 | -94 | -92.2 | -91 |
|  | 41 |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] |

We propose to use -95.3dBm for 3MHz B3 REFSENS.

Proposal3: For CA_3A-41A, the REFSENS value for 3MHz B3 CC shall be -95.3dBm.

|  | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A5 | 3 | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] |
|  | 41 |  | -97.5 | -94.5 | -92.7 | -91.5 |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 41 |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] |

There is some impact in IP2 noise when B3 uses 3MHz UL, but the impact is so small it can be considered negligible versus the lengthy discussions that would likely be needed in order to change the B40 and B41 CA REFSENS numbers.

Proposal4: B40 and B41 CA REFSENS number do not need to be changed due to slightly higher IP2 noise.

# 3	Conclusion

The requirements associated with adding 3MHz BW into B3 in CA_1A-3A, CA_3A-40A, and CA_3A-41A were  analyzed. The following proposals were made.

Proposal1: For CA_1A-3A, the REFSENS value for 3MHz B3 CC shall be -95dBm

| CA_1A-3A4 | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | -100 | -97 | -95.2 | -94 |
|  | 3 | -95 | -94 | -91.5 | -90 | -88.9 |

Proposal2: For CA_3A-40A, the REFSENS value for 3MHz B3 CC shall be -95.3dBm.

| CA_3A-40A | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 40 |  | -95.3 | -92.7 | -91.1 | -90.2 |
| CA_3A-40A | 3 | -95.3 | -94.2 | -91.2 | -89.5 | -88.3 |
|  | 40 |  | -100 | -97 | -95.2 | -94 |

Proposal3: For CA_3A-41A, the REFSENS value for 3MHz B3 CC shall be -95.3dBm.

|  | Band | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A5 | 3 | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] |
|  | 41 |  | -97.5 | -94.5 | -92.7 | -91.5 |
|  | 3 | -98.7 | -97 | -94 | -92.2 | -91 |
|  | 41 |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] |

Proposal4: B40 and B41 CA REFSENS number do not need to be changed due to slightly higher IP2 noise.

# 4	References

[1] R4-162947, “TP for 36.851:  B1 + B3 reference sensitivity”, Qualcomm Inc., RAN4#71

[2] R4-152254, “MSD analysis for 3+40”, Huawei, Hisilicon, RAN4#74bis

[3] R4-153036, “CA_B3_B40 self-desensitization analysis”, Mediatek, Ran4#75

[4] R4-155041, “B3+B41 carrier aggregation”, Qualcomm Inc, RAN4#76

[5] R4-152873, “Updated UE RF analysis for 3+41”, Huawei, Hisilicon, RAN4#75