3GPP TSG RAN4 Meeting #80bis	R4-168354

Ljubljana, Slovenia, 22 – 26 August, 2016

Title: 	TP for TR36.714-00-02 on MSD for 3+7+8 3DL 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This is a TP on MSD requirements and test points for 3+7+8 3DL 2 UL CA.

# 2	Text proposal

This CA combination has several intermodulation products falling on top of own RX, see table below. Please note that this combination is the very first example when two intermodulations, and in addition, third harmonic fall on top of each other.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |

Table 1 Example test points

In addition this combination has also a scenario where IMD2 and IMD3 from B3+B8 both hit on top of B7 but they do not overlap. The example below depicted one possible scenario.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 10 | 50 | B7 | 2650 | 20 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |

We use the following RF front-end architecture in our analysis. Another alternative would be common triplexer based architecture. There is harmonic trap assumed in B8 path for this CA case because harmonic trap for was assumed for B8+B7 1UL/2DL CA and for B3+B7+B8 1UL/3DL CA.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP2 (dBm) | IP3 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 68 |
| Diplexer | 115 | 86 |
| Quadplexer | 113 | 74 |
| PA forward mixing | 31 | 31.5 |
| PA reverse mixing | 38 | 28 |
| LNA | 5 | -5 |

Table 2 General linearity parameters

B8 duplexer TX rejection at B7		30

Quadplexer B8 TX rejection    		45

Quadplexer B3 TX rejection			45

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation PAout-RX				65

RFIC Pout B3/B8	/B7					0

Quadplexer X-band Iso B3->B7		50

Diplexer isolation						15

PA gain								28

Based on these parameters we get the following MSD. Please note that the CF is the same as in [1]. For the case where B3+B8 IMD2, IMD3, and B8 third harmonic fall on top of each other at B7 DL the impact of the third harmonic is negligible because it is more than 20dB below the MSD due to IMD2 and IMD3.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 32.0 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 22.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

We did also calculate IMD values for the case when IMD2 and IMD3 hit on top of B7 DL but do not overlap. The results are as follows. Please note these IMD’s don’t include CF’s that will in this case be fairly significant.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | IMD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3+7+8 | B3 | IMD2 | fB3 + fB8 | 1762.5 | 10 | 50 | B7 | 2650.0 | 20 | 29.0 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1762.5 | 10 | 50 | B7 | 2650.0 | 20 | 19.2 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 10 | 50 | B7 | 2650 | 20 | 29.5 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |

Then finally we consider which of the possible test points shall be in standard. MSD for B8 is automatically standardized as it is the only one for B8. For B7 there are three possible testpoints; IMD2, IMD3 and case where these two overlap together with H3. According to WF the treatment of the case where IMD’s overlap is FFS. For this band combination, the MSD for B7 shall be verified only in case where IMD2, IMD3 and H3 overlap.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |