3GPP TSG RAN WG4 Meeting #80bis	R4-168554

Ljubljana, SI, 10 - 14 Oct 2016

Agenda Item:	8.25.3.1.2

Source: 	Nokia

Title: 	Initial eMTC RSTD Simulation Results

Document for:	Discussion

# Introduction

Positioning is important for supporting many eMTC applications. In Rel-13, only limited positioning functionalities are supported by eMTC UEs [1]. For Rel-14, the new WI on Further Enhanced MTC includes following objective to improve eMTC positioning [2]:

Positioning [RAN4, RAN1]

- E-CID: RSRP/RSRQ measurement

- E-CID: UE Rx-Tx time difference measurement

- OTDOA: core requirements

- OTDOA: consider improvements of accuracy, UE complexity and power consumption

In RAN4#80, OTDOA RSTD measurement performance with Rel-13 specifications was discussed in [3]. In addition, simulation assumptions for investigating eMTC RSTD performance was approved [4]. In this paper, we provide initial simulation results based on the simulation assumptions.

# Simulation Assumptions

Simulation assumptions for investigating eMTC RSTD performance was approved [4]. The simulation scenarios and assumptions in this paper are shown in the following Table 1.

Table 1. Simulation assumptions for RSTD measurement

| Parameter |  | Value |
| --- | --- | --- |
| Cell layout |  | - 3 cells at distinct locations - the distances between each cell and target UE are identical - Same ISD between cells |
| Cell ID scenarios |  | (0, 1, 2), (0, 6, 12) (baseline) |
| Network synchronization |  | - Synchronous with time shifts <0,0, CP/2> |
| Duplex modes |  | FDD |
| Cyclic prefix |  | Normal |
| DRX |  | OFF |
| Carrier frequency |  | 2 GHz |
| Carrier bandwidth |  | 10 MHz |
| Channel model |  | AWGN |
| SINR for three cells, [dB] |  | (Reference cell, neighbour cell 1, neighbour cell 2) = (-6,-13,-13) |
| Number of transmit antennas | PRS | 1 |
|  | CRS | 1 |
| Number of receive antennas |  | 1 |
| MPDSCH transmission |  | No MPDSCH transmission in PRS transmission occasions |
| Number of consecutive positioning subframes |  | 6 |
| Number of positioning occasions for a positioning fix |  | 1 |
| PRS pattern |  | 6-reuse in frequency, vshift = mod(PCI,6) |
| PRS transmission bandwidth |  | 10 MHz |
| Measurement bandwidth |  | 6 RB, 25RB (Note: 24RB was used in [4].) |
| Sampling errors |  | Random number in half of sampling intervals |

# eMTC RSTD Simulation Results

In this section, the simulation results are provided for FDD system with SNR=(-6,-13,-13) for both 1.4MHz and 5MHz eMTC systems. The number of consecutive PRS subframes for a RSTD measurement is 6 for all simulations.

Figures 1 - 4 show the simulation results with AWGN channel for the following scenarios:

- Case 1: 1.4MHz, Cell IDs(0, 6, 12) , 6PRS SFs, AWGN

- Case 2: 1.4MHz, Cell IDs(0, 1, 2) , 6PRS SFs, AWGN

- Case 3: 5MHz, Cell IDs(0, 6, 12) , 6PRS SFs, AWGN

- Case 4: 5MHz, Cell IDs(0, 1, 2) , 6PRS SFs, AWGN

The results shows that under AWGN channel without considering implementation margin, for all above cases the RSTD performance is better than the current performance requirements of ±15 Ts for 1.4MHz (6PRBs) measurement bandwidth, which is defined based on 2Rx antenna and 6 PRS subframes, and ±6 Ts for 5MHz (25 PRBs) measurement bandwidth, which is defined based on 2Rx antenna and 2 PRS subframes,

Figure 1: 1.4MHz, Cell IDs(0, 6, 12), 6PRS SFs, AWGN

Figure 2: 1.4MHz, Cell IDs(0, 1, 2), 6PRS SFs, AWGN

Figure 3: 5MHz, Cell IDs(0, 6, 12), 6PRS SFs, AWGN

Figure 4: 5MHz, Cell IDs(0, 1, 2), 6PRS SFs, AWGN

# Conclusions

In this paper, we provided eMTC RSTD simulation results for AWGN channel according to the agreed simulation assumptions. The simulation results show that

- With AWGN channel, for all cases the RSTD performance is better than the current performance requirements of ±15 Ts for 1.4MHz (6PRBs) measurement bandwidth, and ±6 Ts for 5MHz (25 PRBs) measurement bandwidth.

# References

- 3GPP TS 36.133

- RP-161464, “Revised WID for Further Enhanced MTC for LTE”, Ericsson

- R4-165458, “Discussion on eMTC OTDOA RSTD Measurements”, Nokia

- R4-166819, “Simulation assumption on RSTD”, Huawei, HiSilicon

- 3GPP TS 36.355, E-UTRA: LTE Positioning Protocol (LPP)