3GPP TSG-RAN WG4 Meeting #80BIS	R4-168744

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda Item:	6.3.4.1.3

Source: 	CATT

Title: 	simulation results of NPDSCH Demodulation for NB-IoT

Document for:	Discussion

- Introduction

In RAN4#80 Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward [1] and further simulation assumptions for NB-IoT NPDSCH demodulation evaluations are given [2]. Interested companies are encouraged to provide the simulation results including impairment error using the assumption parameters suggested. In this contribution, we provide our simulation results to show the BLER performance for NPDSCH.

- NPDSCH Simulation Assumptions and Results

For NPDSCH, five test cases have been agreed in RAN4 [3]. In this contribution, the main simulation parameters for Simulation Case 1-3 of In-band configuration are accordingly configured as in Table 1 and Table 2.  And the main simulation parameters for Simulation Case4-5 of Standalone/Guard-band configuration are accordingly configured as in Table 3 and Table 4. And the TBS table for NPDSCH is also given in Table 5.

Table 1: NPDSCH Minimum performance for In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.5 FDD | 1 | EPA5 | 2x1 Low | 70% | TBD |
| 2 | 200kHz | Anchor | R.NB.5 FDD | TBD | EPA5 | 2x1 Low | 70% | TBD |
| 3 | 200kHz | Non-anchor | R.NB.5-1 FDD | TBD | ETU1 | 2x1 Low | 70% | TBD |

Table 2 NPDSCH Reference Channel with 2 TX Antennas

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.5 FDD | R.NB.5-1 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 2 | 10 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 4/0 | 4/0 |
| Target Coding Rate |  | 1/3 | 1/3 |
| Coding Rate |  | 0.4 | 0.4 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 56 | 56 |
| For Sub-Frame 0,5 | Bits | N/A | 56 |
| For Sub-Frame 4,9 | Bits | Note 3 | 56 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 4 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) | Bits | 200 | 200 |
| For Sub-Frame 0,5 | Bits | N/A | 200 |
| For Sub-Frame 4,9 | Bits | Note 5 | 200 |
| Max. Throughput averaged over 2 frame for 1 repetition (Note 6) | Mbps | 36.4kb/s | 56kb/s |
| UE Category |  | NB1 | NB1 |
| Note 1: For in-band, the first 3 symbols are used for LTE PDCCH and the number of LTE CRS ports is 4 Note 2: 6 when mod 2 = 0, otherwise 8. is specified in TS36.211 [4]. Note 3: N/A when mod 2 = 0, otherwise 56. Note 4: N/A when mod 2 = 0, otherwise 1. Note 5: N/A when mod 2 = 0, otherwise 200. Note 6: For repetition number N, the maximum throughput averaged over 1 frame equals to (Max. Throughput averaged over 1 frame for 1 repetition) / N. |  |  |  |

Table 3: Minimum performance for NPDSCH under Standalone/Gurard-band with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 4 | 200kHz | Anchor | R.NB.6 FDD | TBD | EPA5 | 1 | 70% | TBD |
| 5 | 200kHz | Non-anchor | R.NB.6-1 FDD | TBD | ETU1 | 1 | 70% | TBD |

Table 4: NPDSCH Reference Channel with 1Tx Antenna

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.6 FDD | R.NB.6-1 FDD |
| Carrier Type |  | Anchor | Non-Anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | 10 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For a four continuous sub-frame group made up of Sub-Frames 1,2,3,6,7,8 and 9 (if NB-SSS is not present) | Bits | 616 | N/A |
| For a four continuous sub-frame group made up of Sub-Frames 0,1,2,3,4,5,6,7,8 and 9 |  | N/A | 392 |
| Transport block CRC | Bits | 24 | 24 |
| Number of Code Blocks |  |  |  |
| For a four continuous sub-frame group made up of Sub-Frames 1,2,3,6,7,8 and 9 (if NB-SSS is not present) |  | 1 | N/A |
| For a four continuous sub-frame group made up of Sub-Frames 0,1,2,3,4,5,6,7,8 and 9 | Bits | N/A | 1 |
| Available Binary Channel Bits per Sub-frame |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 2 | 320 |
| Max. Throughput averaged over 1 frame for 1 repetition (Note 3) | Mbps | 100kbit/s | 98kbit/s |
| UE Category |  | NB1 | NB1 |
| Note 1: 6 when mod 2 = 0, otherwise 8. is specified in TS36.211 [4]. Note 2: N/A when mod 2 = 0, otherwise 320. Note 3: For repetition number N, the maximum throughput averaged over 1 frame equals to (Max. Throughput averaged over 1 frame for 1 repetition) / N. |  |  |  |

Table 5	TBS table for NPDSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

The Throughput vs. SNR for Simulation Case 1 is shown in Fig.1.

Fig.1. Throughput vs. SNR for Simulation Case 1

For Simulation Case 1, the goal is to seek performance alignment between companies by assuming RL=1.

Observation 1: For Simulation Case 1, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR is around 1.7dB.

Proposal 1: For Simulation Case 1, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR is around 1.7dB.

The Throughput vs. SNR for Simulation Case 2 and Case3 are shown in Fig.2. and Fig.3.

Fig.2. Throughput vs. SNR for Simulation Case 2

Fig.3. Throughput vs. SNR for Simulation Case 3

The alignment simulation results of case 1-3 are shown below:

| Test number | Carrier Type | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | SNR(dB) |  |  |  |
| 1 | Anchor | EPA5 | 2x1 Low | 70% | RL1 |  |  |  |
|  |  |  |  |  | 1.7 |  |  |  |
| 2 | Anchor | EPA5 | 2x1 Low | 70% | RL8 | RL16 | RL32 |  |
|  |  |  |  |  | -5.2 | -7.3 | -9.3 |  |
| 3 | Non-anchor | ETU1 | 2x1 Low | 70% | RL64 | RL128 | RL192 | RL256 |
|  |  |  |  |  | -10.5 | -12.1 | -13.2 | -14.1 |

The impairment simulation results of case 1-3 are shown below:

| Test number | Carrier Type | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | SNR(dB) |  |  |  |
| 1 | Anchor | EPA5 | 2x1 Low | 70% | RL1 |  |  |  |
|  |  |  |  |  | 3.5 |  |  |  |
| 2 | Anchor | EPA5 | 2x1 Low | 70% | RL8 | RL16 | RL32 |  |
|  |  |  |  |  | -3.4 | -5.5 | -7.5 |  |
| 3 | Non-anchor | ETU1 | 2x1 Low | 70% | RL64 | RL128 | RL192 | RL256 |
|  |  |  |  |  | -8.7 | -10.3 | -11.4 | -12.3 |

Observation 2: For Simulation Case 2, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR for RL=8 is around -5.2dB; the corresponding SNR for RL=16 is around -7.3dB; the corresponding SNR for RL=32 is around -9.3dB.

Observation 3: For Simulation Case 3, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR for RL=64 is around -10.5dB; the corresponding SNR for RL=128 is around -12.1dB; the corresponding SNR for RL=192 is around -13.2dB; the corresponding SNR for RL=256 is around -14.1dB.

Proposal 2: For the Simulation Case 2 with target SNR = -6dB, the impairment simulation result shows that the repetition level RL=32 is sufficient.

Proposal 3: For Simulation Case 3 with target SNR = -12dB, the impairment simulation result shows the repetition level RL=256 is sufficient.

The Throughput vs. SNR for Simulation Case 4 and 5 are shown in Fig.4.and  Fig.5.

Fig.4. Throughput vs. SNR for Simulation Case 4

Fig.5. Throughput vs. SNR for Simulation Case 5

The alignment simulation results of case 4-5 are shown below:

| Test number | Carrier Type | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | SNR(dB) |  |  |  |
| 4 | Anchor | EPA5 | 1 | 70% | RL16 | RL32 | RL64 |  |
|  |  |  |  |  | -4.6 | -7.3 | -9.9 |  |
| 5 | Non-anchor | ETU1 | 1 | 70% | RL64 | RL128 | RL192 | RL256 |
|  |  |  |  |  | -8.8 | -11.7 | -13.1 | -14.2 |

The impairment simulation results of case 4-5 are shown below:

| Test number | Carrier Type | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | SNR(dB) |  |  |  |
| 4 | Anchor | EPA5 | 1 | 70% | RL16 | RL32 | RL64 |  |
|  |  |  |  |  | -2.8 | -5.5 | -8.1 |  |
| 5 | Non-anchor | ETU1 | 1 | 70% | RL64 | RL128 | RL192 | RL256 |
|  |  |  |  |  | -7.0 | -9.9 | -11.3 | -12.4 |

Observation 4: For Simulation Case 4, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR for RL=16 is around -4.6dB; the corresponding SNR for RL=32 is around -7.3dB; the corresponding SNR for RL=64 is around -9.9dB.

Observation 5: For Simulation Case 5, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR for RL=64 is around -8.8dB; the corresponding SNR for RL=128 is around -11.7dB; the corresponding SNR for RL=192 is around -13.1dB; the corresponding SNR for RL=256 is around -14.2dB.

Proposal 4: For the Simulation Case 4 with target SNR = -6dB, the impairment simulation result shows that the repetition level RL=64 is sufficient.

Proposal 5: For Simulation Case 5 with target SNR = -12dB, the impairment simulation result shows the repetition level RL=256 is sufficient.

- Conclusion

In this contribution, we provided simulation results to show the BLER performance for NPDCCH and then share our view of recommendations on the repetition levels for different simulation cases.

Proposal 1: For Simulation Case 1, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR is around 1.7dB.

Proposal 2: For the Simulation Case 2 with target SNR = -6dB, the impairment simulation result shows that the repetition level RL=32 is sufficient.

Proposal 3: For Simulation Case 3 with target SNR = -12dB, the impairment simulation result shows the repetition level RL=256 is sufficient.

Proposal 4: For the Simulation Case 4 with target SNR = -6dB, the impairment simulation result shows that the repetition level RL=64 is sufficient.

Proposal 5: For Simulation Case 5 with target SNR = -12dB, the impairment simulation result shows the repetition level RL= 256 is sufficient.

- Reference

[1] R4-166656, “Way forward on NB-IoT UE demodulation requirements”, Ericsson, Huawei, HiSilocon, ZTE, Qualcomm, Intel, Samsung ,AUG  2016.

[2] R4-165352, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE, AUG 2016.

[3]  R4-167114, “CR for NPDSCH demod requirements”, Huawei, AUG 2016.