3GPP TSG-RAN WG4 (Radio) Meeting #80bis	R4-168903

Ljubljana, Slovenia, 10 – 14 October, 2016

Agenda Item:	8.11.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei

Title: 	TP for TR 37.871: Necessary changes to the test requirements for Multi-Band Base Station testing with three or more bands

Document for:	Approval

# 1.	Introduction

The Multi-Band (MB) Base Station (BS) testing with three or more bands work item was approved at RAN#70 [1]. The objectives of the work item include the following:

Specify multi-band BS testing for all the identified band combinations. In particular, to decide:

1.	The RF bandwidth position of the middle band(s) within the frequency range(s) supported by the BS in the middle band(s).

2.	The multi-band combinations to be tested out of the supported ones for BS that supports multiple multi-band combinations.

3.	Other aspects of multi-band BS testing that are necessary to complete the specification.

The first topic above has been concluded at RAN4#80, with the approval of the text proposal in [2]. In this contribution, we provide a text proposal into the TR [3] of this work item to conclude this topic.

# 2.	Text proposal

<Start of change>

## 6.3	Multi-band combinations to be tested

One of the potential usages for BS to support multiple multi-band combinations is when a higher maximum output power can be declared when fewer bands are configured for operation. In such case, it should be advantageous to test all supported multi-band combinations with different declared parameters (e.g. different maximum output power when three bands and two-out-of-three bands are configured for operation), in order to ensure that the BS will perform satisfactorily with the different declared parameters.

On the other hand, it may not enhance the test coverage to test for every supported multi-band combination with identical declared parameters (e.g. same maximum output power when any two-out-of-three bands are configured for operation). In this case, it should be sufficient to test only the most stringent multi-band combinations out of those with identical parameters, which should normally be the multi-band combination covering the widest radio bandwidth among the supported ones. Therefore, the following principles should be used to test BS that supports multiple multi-band combinations:

1)	The supported multi-band combination covering the widest radio bandwidth should be tested.

2)	Among the remaining supported multi-band combinations, the following ones should also be tested:

-	Those with a larger rated total output power (per band or per band combination).

-	Those with a larger total number of supported carriers (per band or per band combination).

-	Those with a larger Maximum Base Station RF Bandwidth (per band).

<End of change>

# References

[1]	RP-152205, “New Work Item proposal: Multi-Band Base Station testing with three or more bands”, Alcatel-Lucent.

[2]	R4-166948, TP for TR 37.871: Necessary changes to the test requirements for Multi-Band Base Station testing with three or more bands, Nokia, Alcatel-Lucent Shanghai Bell, Huawei.

[3]	RP-160776, “TR 37.871 V1.0.0”, Nokia.