3GPP TSG-RAN WG4 Meeting #77					             R4-157003

Anaheim, US, 16th – 20th Nov 2015

Agenda Item:	7.48.1

Source: 	Intel Corporation

Title: 	On RSTD report mapping

Document for:	Approval

# Introduction

In RAN4#76bis, the potential benefit to increase the granularity of RSTD report mapping is discussed. There are two proposed options to be considered [1]

- Option 1: UE reports according to Rel-9 measurement report mapping and, when a higher resolution is supported, reports additionally a non-zero “delta”

  - ”delta” refers to a relative measurement value with respect to the Rel-9 measurement report mapping (delta=0 corresponds to Rel-9 measurement report)

  - “delta” values are specified in a new table in 36.133

- Option 2: The RSTD quantization with scaling impact to 36.133 are expected to include no big change, only some clarification is as below:

  - UE reports according to Rel-9 measurement report mapping

  - 36.133, subclause 9.1.10.3: “RSTD Measurement Report Mapping” is modified to clearly state that the mapping of the scaled measurement quantity is defined in Table 9.1.10.3-1 as such:

  - The RSTD value that is reported corresponds to a scaled RSTD measurement value, which equals to the measured RSTD value multiplied with a scaling factor [details defined in 36.355].

  - The reporting range of scaled RSTD is defined from -15391Ts to 15391Ts with 1Ts resolution for absolute value of scaled RSTD less or equal to 4096Ts and 5Ts for absolute value of  scaled RSTD greater than 4096Ts.

  - The mapping of scaled measurement quantity is defined in Table 9.1.10.3-1.

Even though there have been a lot of discussion on how to increase the reported RSTD granularity, the corresponding OTDOA gain however has not been thoroughly studied, especially when a realistic time alignment error at eNB is considered. In this contribution, the potential benefit to increase RSTD reporting granularity is discussed.

# Issues to be considered on OTDOA performance

It is well known that OTDOA performance depends on the follow factors

- RSTD estimation/reporting accuracy.

- Number of reported RSTD

- Knowledge of the geographical coordinates of reference and measured eNB

- Time alignment error (TAE) between reference and measured eNBs

Among the aforementioned issues, the knowledge of eNB coordination can be assumed accurate enough at eNB side since the related information most likely keeps unchanged. The remaining three issues are related to the quantity and quality of RSTD reporting, and the TAE at eNB side.

Frames of LTE signals present at the eNB antenna ports are not perfectly aligned. As a result, TAE requirements are defined in 36.104 as follow

### 6.5.3	Time alignment error

This requirement applies to frame timing in TX diversity, MIMO transmission, carrier aggregation and their combinations.

Frames of the LTE signals present at the BS transmitter antenna port(s) are not perfectly aligned in time. In relation to each other, the RF signals present at the BS transmitter antenna port(s) experience certain timing differences.

For a specific set of signals/transmitter configuration/transmission mode, time alignment error (TAE) is defined as the largest timing difference between any two signals.

#### 6.5.3.1	Minimum Requirement

For MIMO or TX diversity transmissions, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 130 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 260 ns.

For inter-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 1.3 μs.

It has been clearly defined that <65ns (~2Ts) TAE for intra-frequency cases and <260ns (~8Ts) for inter-frequency but inter-band. In case of inter-band scenarios, TAE between eNB can be as large as 1.3us (~39Ts). Obviously, the reported RSTD by UE includes not only the receive time difference due to different propagation delay between the reference and measured eNBs, but also the TAE between the reference and measured eNBs. Since TAE is unknown at eNB and LPP server, TAE and real RSTD cannot be distinguished in positioning server. The aforementioned TAE requirements should be based on the assumption that GPS is equipped. Therefore, it is supposed to be even more challenging for indoor eNB to maintain this level of TAE.

To understand better on the reported RSTD, it can be represented as follow

RSTDreported=RSTDgeni+∆est+∆quanti+TAE

- RSTDgeni:RSTD purly due to the propogation delay difference

- ∆est:RSTD estimation error at UE (within +/- 4Ts when the measurement BW>15MHz)

- ∆quanti:quantization error due to discrete RSTD report mapping (<0.5Ts based on existing report mapping)

- TAE:time alignment error between the reference and measured eNBs (<2Ts for intra-freq, <8Ts for inter-freq/intra-band), <39Ts for intre-band)

Obviously, the existing RSTD minimum requirement with wideband and TAE requirements are much larger than the quantization error due to the existing report mapping. Intuitively, the RSTD report mapping should not be the bottleneck to limit the OTDOA performances.

Observation 1: The existing RSTD minimum requirement with wideband and TAE requirements are much larger than the quantization error due to the existing report mapping. The RSTD report mapping should not be the bottleneck to limit the OTDOA performances.

Observation 2: TAE between the measured and reference eNBs can impact OTDOA performances much more than other imperfection issues.

# TAE impact on OTDOA performance

In RAN4 SI “Feasibility of positioning enhancements for E-UTRA” [2] and RAN1 SI “Study on indoor positioning enhancements for UTRA and LTE” [3], it has been investigated how to improve OTDOA performance. RAN4’s SI focuses more on RSTD performance under wide and narrow measurement bandwidth conditions. The system level OTDOA performance is not specifically studied, especially with realistic TAE assumption. This is reasonable since system level OTDOA study may be out of the scope of RAN4’s concern. In RAN1’s indoor positioning SI, some system level OTDOA evaluations have been captured with zero TAE assumed. However, there is no sufficient system level OTDOA study to understand the benefit of increasing RSTD report mapping granularity. Meanwhile, the impact of TAE is not addressed either in both RAN4 and RAN1 SI.

Observation 3: No system level OTDOA study with realistic TAE has been done in both RAN1 and RAN4 SI. The related gain of increasing RSTD report mapping granularity is not well justified.

To understand the OTDOA performance with more realistic scenarios assumed, system level simulation results are provided with both increased RSTD report mapping granularity and TAE assumed. SLS assumptions are provided in the appendix. Rather than using the minimum TAE requirements, a relatively optimistic 50ns and 100ns of TAE are assumed with the implementation margin considered at eNB. In the legend of Figure 1, “Indoor 0.5, 50” represents the scenario with minimum RSTD report mapping granularity of 0.5Ts and TAE<50ns.

When zero TAE is assumed, the gain due to finer the RSTD report mapping from 1Ts to 0.5Ts is around 1 and 2 meters at eighty and fifty percentiles, respectively. When a quite mild TAE of 50ns is assumed, the gap between 1Ts and 0.5Ts RSTD report mapping granularity becomes very trivial and can be neglected. When TAE is increased to 150ns which is still less than the minimum TAE requirement for intra-band continuous CA cases, the OTDOA performance becomes identical with1Ts and 0.5Ts RSTD report mapping granularity.

Observation 4: With quite mild TAE assumed, OTDOA performance cannot be improved with increased RSTD report mapping granularity. Even with zero TAE, the performance enhancement due to improved RSTD reporting granularity is limited.

Meanwhile, the performance degradation due to TAE is quite significant. For example, with 0.5Ts of RSTD report granularity, >10 meters of OTDOA degradation is observed when the maximum TAE is increase from 0 to 50ns. When the maximum TAE is around 150ns, it results in more than 20 meters of OTDOA degradation.

Observation 5: System level study shows significant OTDOA performance degradation due to coarse TAE between eNBs. It suggests that improving TAE requirements should be much more effective than just refining RSTD reporting granularity in terms of OTDOA performance enhancement.

Proposal: RSTD report mapping enhancement should not be considered until the minimum requirement of TAE becomes satisfactory.

# Conclusion

In this contribution, we discuss about OTDOA performances and the concerned issues. It is observed

Observation 1: The existing RSTD minimum requirement with wideband and TAE requirements are much larger than the quantization error due to the existing report mapping. The RSTD report mapping should not the bottleneck to limit the OTDOA performances.

Observation 2: TAE between the measured and reference eNBs can impact OTDOA performances much more than other imperfection issues.

Observation 3: No system level OTDOA study with realistic TAE has been done in both RAN1 and RAN4 SI with. The related gain of increasing RSTD report mapping granularity is not well justified.

Observation 4: With quite mild TAE assumed, OTDOA performance cannot be improved with increased RSTD report mapping granularity. Even with zero TAE, the performance enhancement due to improved RSTD reporting granularity is limited.

Observation 5: System level study shows significant OTDOA performance degradation due to coarse TAE between eNBs. It suggests that improving TAE requirements should be much more effective than just refining RSTD reporting granularity in terms of OTDOA performance enhancement.

Based on the observation above, it is proposed

Proposal: RSTD report mapping enhancement should not be considered until the minimum requirement of TAE becomes satisfactory.

# References

[1] R4-156684	“Way forward on reducing quantization errors”, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia Networks, CATT, Verizon, CMCC, China Telecom, ZTE

[2] 3GPP TR 36.855: "Feasibility of positioning enhancements for E-UTRA".

[3] 3GPP TR 37.857: “Study on indoor positioning enhancements for UTRA and LTE”

# Appendix – System Level Evaluation Assumptions

In this section, we provide summary of all simulation assumptions used for current analysis. The relevant set of system level evaluation assumptions as well as system level parameters are provided in Table 1.

Table 1. Summary of system level evaluation assumptions for positioning and timing estimation.

| Positioning method |  |
| --- | --- |
| OTDOA Positioning Method | 2D ML positioning problem is formulated and solved by iterative algorithm based on Taylor series expansion method [2]. The error weighting matrix is proportional to instantaneous PRS SINR measured over wideband. Initial guess is the average coordinate of the 5 nodes with unique locations (selected according to largest SINR). |
| Cell ID Planning | Sectors of Macro cell use orthogonal PRS patterns, unique Cell IDs are assigned when feasible. |
| PRS muting information | 8-bit muting pattern with a single activated PRS occasion is used. |
| Max number of cells for RSTD measurements | Maximum number of sources is restricted to 20. Sources with SINR < -13dB are excluded from considerations, assuming high likelihood of inaccurate timing measurements. |
| Reference cell | Serving cell (max received power criterion). |
| Timing estimation |  |
| Timing estimation | Search window is restricted to [+/- 200 Ts] relative to the serving cell. Threshold based algorithm for the first arrival path detection with non-coherent combining across UE antennas. Low interference subframes are assumed (w/o data and control signals) Only PRS processing for timing estimation of neighboring cells. |
| PRS info | 10 MHz, 20MHz and 40MHz (2 CCs) are used for PRS processing. No boosting. No up-sampling is applied. |
| Cyclic Prefix | Extended cyclic prefix was used in analysis. |
| TAE assumption | Gaussian distribution with rms X ns and truncated at [+X ns; -X ns]. X=[0, 50, 150]. |