3GPP TSG-RAN WG4 Meeting #77 	R4-157124

Anaheim, U. S. A.  16 – 20 November 2015

Source:	DISH Network

Title:	TP for Sections 8 (conclusion) for TR 36.870

Work item:	FS_LTE_AWS_3_4

Agenda item:	9.4.2

Document for:	Approval

Abstract

This document provides a TP for Sections 8 (Conclusion). (pCR)

## Introduction

The core objectives of this SI [1] are:

- Analyze the feasibility of specifying two duplex spacings in the band (1695-1710 MHz UL & 1995-2020 MHz DL) in order to provide deployment flexibility in protecting federal satellite receiver sites and addressing market license variations.

- Identify duplex spacings depending on the outcome of the analysis

## Conclusion

In the study item we have shown that;

- The specification allows  the use of other TX channel to RX channel carrier centre frequency separation; such a scheme is not precluded and is intended to form part of a later release [2]

- The proposed AWS-3/4 band plan has a large 285 MHz duplex gap and therefore the performance differences between the two TX/RX carrier/duplex spacing are expected to be insignificant. This means the minimum performance requirements for the band can remain consistent regardless of the TX/RX frequency or duplex spacing choice.

- The actual performance and conformance test cases for the additional TX/RX spacing (295MHz) would be limited to the one specific deployment configuration (Scenario 2b).

- Even though there are two duplex spacings, one duplexer covers the whole DL spectrum. There is no need to modify the baseband, mixer, LO, PA, and RF front-end. Supporting the additional duplex spacing can be done by software, which provides tuning control to the RX LO.

- In TS36.331, the signalling support of additional duplex spacing is already available. Therefore, from the signalling point of view, it is possible to specify two TX/RX carrier spacings for a single band [3]

- Supporting more than one duplex spacing in a band would be a proactive 3GPP feature, which offers needed flexibility to operators, as more “unconventional” spectrum (unpaired FDD, impaired spectrum and etc.) is being made available for LTE. For example, the FCC will be mandating band plans with two duplex spacings if the final band plan exceeds 35+35 MHz allocation as stated in the recently published 600 MHz incentive auction Report and Order

Therefore we have addressed the objectives of the study item which was to analyze the feasibility of specifying two duplex spacings in the band (1695-1710 MHz UL & 1995-2020 MHz DL) in order to provide deployment flexibility in protecting federal satellite receiver sites and addressing market license variations.

Separately to the required objectives of the study item, we have concluded that maintaining a single carrier/duplex spacing for the AWS-3/4 band arrangements would require the usage of 3DL CA, noting the current specifications do not support 3DL intra-band contiguous CA configurations whose aggregate bandwidths are less than 40MHz.

Hence, it is concluded that:

- It is feasible to specify two duplex spacings in the proposed band (1695-1710 MHz UL & 1995-2020 MHz DL) and

- It is identified that two duplex spacings of 300 MHz and 295 MHz are required to provide deployment flexibility in protecting federal satellite receiver sites and addressing market license variations.

3.0		Reference

[1]	RP-151626, “New SI: New AWS-3/4 Band Plan”, DISH Network, Hughes Network Systems, Nokia Network, Huawei, Intel, Samsung, Ericsson, 3GPP RAN#69, Phoenix, U. S. A., September 2015

[2] 	3GPP TS36.101, v13.1.0

[3] 	R4-155587, “On Two Duplex Spacings in the AWS-3/4 Band”, Intel Corporation, 3GPP RAN4 #76bis, Sophia Antipolis, France, October 2015

-- Start of text proposal ----

# 8	Conclusions

In the study item we have shown that;

- The specification allows  the use of other TX channel to RX channel carrier centre frequency separation; such a scheme is not precluded and is intended to form part of a later release [TS36.101]

- The proposed AWS-3/4 band plan has a large 285 MHz duplex gap and therefore the performance differences between the two TX/RX carrier/duplex spacing are expected to be insignificant. This means the minimum performance requirements for the band can remain consistent regardless of the TX/RX frequency or duplex spacing choice.

- The actual performance and conformance test cases for the additional TX/RX spacing (295MHz) would be limited to the one specific deployment configuration (Scenario 2b).

- Even though there are two duplex spacings, one duplexer covers the whole DL spectrum. There is no need to modify the baseband, mixer, LO, PA, and RF front-end. Supporting the additional duplex spacing can be done by software, which provides tuning control to the RX LO.

- In TS36.331, the signalling support of additional duplex spacing is already available. Therefore, from the signalling point of view, it is possible to specify two TX/RX carrier spacings for a single band [R4-155587]

- Supporting more than one duplex spacing in a band would be a proactive 3GPP feature, which offers needed flexibility to operators, as more “unconventional” spectrum (unpaired FDD, impaired spectrum and etc.) is being made available for LTE.  For example, the FCC will be mandating band plans with two duplex spacings if the final band plan exceeds 35+35 MHz allocation as stated in the recently published 600 MHz incentive auction Report and Order

Therefore we have addressed the objectives of the study item which was to analyze the feasibility of specifying two duplex spacings in the band (1695-1710 MHz UL & 1995-2020 MHz DL) in order to provide deployment flexibility in protecting federal satellite receiver sites and addressing market license variations.

Separately to the required objectives of the study item, we have concluded that maintaining a single carrier/duplex spacing for the AWS-3/4 band arrangements would require the usage of 3DL CA, noting the current specifications do not support 3DL intra-band contiguous CA configurations whose aggregate bandwidths are less than 40MHz.

Hence, it is concluded that:

- It is feasible to specify two duplex spacings in the proposed band (1695-1710 MHz UL & 1995-2020 MHz DL) and

- It is identified that two duplex spacings of 300 MHz and 295 MHz are required to provide deployment flexibility in protecting federal satellite receiver sites and addressing market license variations.

-- End of text proposal ----