3GPP TSG-RAN WG4 Meeting #77                                                                  R4-157370

# Anaheim, CA, US, 16 – 20 Nov., 2015

Source:	LG Electronics

Title:	Simulation results of cell identification with LAA

Agenda item:	7.13.4

Document for:	Discussion

Introduction

In this paper, we present the simulation results of cell identification for LAA with agreed simulation assumption[1]. Based on the simulation results, we provide our view on the cell identification.

Simulation assumption

The simulation assumptions for cell identification on unlicensed carrier are summarized in Tables 1-4 below.

Table 1: Simulation parameters for cell identification on unlicensed carrier

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Cell Identifier | - | known to UE | known to UE | unknown to UE, to be identified |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| PSS and SSS PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Carrier frequency | GHz | 5 | 5 | 5 |
| PSS/SSS transmission bandwidth | Number of RB’s | 6 | 6 | 6 |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| Relative Delay of 1st Path (synchronous) | ms | 0 | 0 | CP/2 |
| Es/Noc | dB | -0.25 dB | 1 dB | {-1,0,1,2,3,4} dB (Es/Iot= {-6,-5,-4,-3,-2,-1} dB) |
| PSS Sequence ID | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| SSS Sequence ID | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| PSS/SSS configuration |  | Rel-12 PSS/SSS configuration |  |  |
| Propagation Condition | - | AWGN, EPA5, ETU30 |  |  |
| Noc Model | - | AWGN |  |  |
| DRS occasion periodicity (M) | ms | A DRS occasion is transmitted every 160 ms |  |  |
| DRS occasion duration (N) | - | A DRS occasion for a cell comprises 1 subframe (N=1). |  |  |
| DRS availability | - | DRS is present in all DRS occasions |  |  |
| L1 measurement period | - | i*160 ms, where - i=1 (single shot) - i=2 and i=3 (multi-shot) |  |  |
| NOTE : Noc value doesn’t include the three simulated eNB signals’ power |  |  |  |  |

Table 2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or asynchronous (by signaling) | No |
| DRX | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence id. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| UE receive antennas | 2 (uncorrelated) |

Table 3: Cell Id Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |

Table 4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |

Performance Metrics

- Cell identification delay (T_identify)

  - The cell identification delay is the time required by the UE to fully identify the cell (Cell 3 in this case) by detecting PSS/SSS sequences, excluding the L1 RSRP measurement period.

- Cell identification probability for a given number of 1 ms DRS occasions.

Simulation results

Table 5 shows  the number of shot and the probability of detection corresponding to the number of shot  . In the table, it is highlighted with green color for above 90% detection probability.

Table 5: Probability of detection

| case,#of shot |  | Probability of detection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mBW | AWGN |  |  |  |  |  | EPA5 |  |  |  |  |  | ETU30 |  |  |  |  |  |
|  | Es/Iot | -6 | -5 | -4 | -3 | -2 | -1 | -6 | -5 | -4 | -3 | -2 | -1 | -6 | -5 | -4 | -3 | -2 | -1 |
| C1 | 1 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.90 | 0.93 | 0.96 | 0.97 | 0.98 | 0.99 | 0.63 | 0.73 | 0.80 | 0.86 | 0.91 | 0.93 |
|  | 2 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.85 | 0.92 | 0.96 | 0.98 | 0.99 | 1.00 |
|  | 3 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.95 | 0.98 | 1.00 | 1.00 | 1.00 | 1.00 |
| C2 | 1 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.88 | 0.93 | 0.96 | 0.97 | 0.98 | 0.99 | 0.62 | 0.73 | 0.80 | 0.85 | 0.89 | 0.94 |
|  | 2 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.86 | 0.92 | 0.95 | 0.98 | 0.99 | 1.00 |
|  | 3 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.95 | 0.98 | 0.99 | 1.00 | 1.00 | 1.00 |
| C3 | 1 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.91 | 0.93 | 0.96 | 0.97 | 0.98 | 0.99 | 0.62 | 0.73 | 0.80 | 0.85 | 0.90 | 0.93 |
|  | 2 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.86 | 0.92 | 0.95 | 0.98 | 0.99 | 1.00 |
|  | 3 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.95 | 0.99 | 0.99 | 1.00 | 1.00 | 1.00 |
| C4 | 1 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.89 | 0.94 | 0.95 | 0.97 | 0.98 | 0.99 | 0.62 | 0.72 | 0.80 | 0.86 | 0.90 | 0.92 |
|  | 2 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.85 | 0.92 | 0.97 | 0.98 | 0.99 | 0.99 |
|  | 3 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.95 | 0.98 | 1.00 | 1.00 | 1.00 | 1.00 |

Based on these results, we can observe as follows.

- Observation 1: Cell identification can be performed with one shot with above 90 % detection probability at SINR of -1dB for AWGN, EPA5 and ETU30.

From the observation, we propose as below

- Proposal 1: Cell identification delay with LAA should be specified with one shot at side condition of -1dB.

Conclusion

In this paper, we provided simulation results of cell identification with agreed simulation assumptions. From simulation results, we could observe as follows.

- Observation 1: Cell identification can be performed with one shot with above 90 % detection probability at SINR of -1dB for AWGN, EPA5 and ETU30.

From the observation, we propose as below

- Proposal 1: Cell identification delay with LAA should be specified with one shot at side condition of -1dB.

- Reference

- R4-156644, “Simulation assumptions for cell identification with LAA”, Ericsson