3GPP TSG-RAN WG4 Meeting #77	R4-157460

# Anaheim, US, 16-20 Nov. 2015

Title:	Use case and requirement for HPUE

Source:	CMCC

Agenda Item:	9.3.3

Document for:	Discussion

- Introduction

HPUE TR 36.886 shows the justification of introducing power class 2 UE in the network. In this paper, some detailed scenarios are given.

- High power UE use case and requirement

Limited coverage scenario in rural area

In rural area, it is usually not continuously covered by E-UTRA network. For example, only a few BSs will cover main area of a town, or only one BS severs a small village. Subscribers use CPE or MiFi to connect to the network. In such case, operators could increase BS transmit power to achieve better downlink performance yet don’t have to worry about the inter cell interference. However, uplink is always the bottleneck, especially for TDD with less UL subframes. Power Class 3 UE with directional antenna is also proposed for this scenario, but it is not suitable for mobile or nomadic case and directional antenna is too complex to install for common users. Therefore, high power UE is more suitable for limited coverage scenario in rural area.

Observation#1：high power UE is suitable for limited coverage scenario in rural area.

Large penetration loss in urban area

In urban area, E-UTRA network is always fully covered. However, there are also some coverage holes for indoor cases. In urban area, mobile connections always happen indoor, and outdoor to indoor coverage is a common case when operator cannot have indoor coverage in a certain building. We always assume only 10-20dB penetration loss in simulation or deterministic calculation, but practical penetration loss is larger. Several walls instead of one have to be penetrated, and higher frequency band make the case even worse. Therefore, in such case high power UE could help increase the cell coverage.

Observation#2：high power UE is suitable for large penetration loss scenario in urban area.

High definition VoLTE case

Table 1 shows VoLTE link budget for practical Band 41 network with TDD configuration 2.

Table 1. Band 41 VoLTE link budget

|  |  | Control Channel |  | AMR 12.2k |  | AMR 23.85k |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PDCCH | PUCCH | PUSCH | PDSCH | PUSCH | PDSCH |
| Transmission Power | TX Power (dBm) | 46 | 23 | 23 | 46 | 23 | 46 |
|  | RBs | 8 CCE | 6 | 5 | 12 | 5 | 20 |
|  | Tx Power/Subcarrier (dBm) | 15.2 | 4.4 | 5.2 | 15.2 | 5.2 | 15.2 |
| System Cost | Transmission Line Loss (dB) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
|  | Antenna Gain (dBi) | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 |
|  | Antenna Diversity Gain(dB) | 3.0 | 9.0 | 9.0 | 3.0 | 9.0 | 3.0 |
|  | RLC Segmentation Gain | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Fast Fade Margin (dB) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Shadow Fading Margin (dB) | 11.6 | 11.6 | 11.6 | 11.6 | 11.6 | 11.6 |
|  | Interference Margin (dB) | 5.0 | 2.0 | 2.0 | 5.0 | 2.0 | 5.0 |
|  | Penetration Loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
|  | OTA(dB) | 6 | 6 | 6 | 6 | 6 | 6 |
|  | MS Body Loss (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
|  | Terminal Loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Total System Cost (dB) | 8.1 | -0.9 | -0.9 | 8.1 | -0.9 | 8.1 |
| Sensitivity | Thermal Noise (kT) (dBm/Hz) | -174.0 | -174.0 | -174.0 | -174.0 | -174.0 | -174.0 |
|  | Noise Figure (dB) | 9.0 | 5.0 | 5.0 | 9.0 | 5.0 | 9.0 |
|  | Noise Power (dBm) | -123.2 | -127.2 | -127.2 | -123.2 | -127.2 | -123.2 |
|  | SNR | -5.9 | -11.8 | -1.5 | -5.0 | 1.4 | -5.0 |
|  | Sensitivity - Composite | -129.1 | -139.0 | -128.7 | -128.2 | -125.8 | -128.2 |
|  | Maximum allowable Path Loss | 136.2 | 144.3 | 134.8 | 135.3 | 131.9 | 135.3 |

We can see that, AMR 12.2k doesn’t cause the imbalance between UL and DL. However, for the high definition voice of AMR 23.85, DL coverage is approximate 3dB better than UL.

Observation#3：high power UE with 3dB higher maximum transmit power is suitable for high definition VoLTE case.

- Conclusion

Observation#1：high power UE is suitable for limited coverage scenario in rural area.

Observation#2：high power UE is suitable for large penetration loss scenario in urban area.

Observation#3：high power UE with 3dB higher maximum transmit power is suitable for high definition VoLTE case.