3GPP TSG-RAN WG4 Meeting #77	R4-157673

# Anaheim, CA, USA, 16-20 November, 2015

Title:	Reply LS on power boosting in-band and guard-band operation for NB-IoT

Release:	Release 13

Work Item:	NB_IOT-Core

Source:	RAN4

To:	RAN1

Contact Person:

#### Name:	Ali Behravan

E-mail Address:	ali.behravan at ericsson.com

Attachments: None

- Description:

RAN4 would like to ask RAN1 regarding the questions in R1-156249 regarding the feasibility of in-band and guard-band power boosting for NB-IOT.

After discussing the issue in RAN4, we have the following answers to the questions:

Q1: Does RAN4 consider the assumption on 6dB power boosting is feasible.

A1:   6dB power boosting for NB-IOT with 15kHz subcarrier spacing is feasible, and there is no impact on either EVM or spectral emission of the wide-band LTE and the NB-IOT. However in-band operation of NB-IOT with 3.75kHz subcarrier spacing results in unacceptable increase in EVM of both LTE and NB-IOT regardless of the power boosting level.

Q2: If the answer is No on Q1, RAN1 would like to know the appropriate value(s) that can be assumed for power boosting for NB-IoT performance evaluation in inband scenarios.

A2: Assuming 15kHz subcarrier spacing, power boosting up to 12dB is feasible. However for subcarrier spacing of 3.75kHz no power boosting is feasible.

Q3: What level of boosting is feasible for transmissions within the LTE guard band, for the two cases of shared and separate PAs, and for approximately 0 and 200kHz offsets from the edge of the LTE carrier?

A3:   For NB-IOT with 15kHz subcarrier spacing, power boosting up to 6dB is feasible with minor impact on the out-of-band emission and no impact on EVM for both 0Hz and 180kHz carrier spacing. For NB-IOT with 3.75kHz, maximum power boosting of 3dB might be feasible for carrier spacing of 180kHz with some impact on EVM. However for 0 Hz carrier spacing the impact on EVM is very large and operation is not feasible.

- Actions:

RAN4 asks RAN1 to consider above answers in their evaluation.

- Date of Next TSG-RAN1 Meetings:

TSG-RAN4 NB-IOT AH		January 20-22 			EU

TSG-RAN1 Meeting#84		February	15-19			Malta, Malta