3GPP TSG-RAN WG4 (Radio) Meeting #77	R4-157864

Anaheim, CA, US, 16 – 20 November 2015

Agenda Item:	7.42.2.4

Source: 	Alcatel-Lucent, U.S. Cellular

Title: 	TP for TR 36.854-13: BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12

Document for:	Approval

# 1.	Introduction

10 WI proposals to support the LTE Advanced (LTE-A) 4DL/1UL Carrier Aggregation (CA) with Bands 2, 4, 5 and 12 were approved in RAN#68 [1-5] and RAN#69 [6-10]. The required BS studies of harmonics and IMD products for the 10 WI proposals were provided in RAN4#76 [11-12] and RAN4#76bis [13-17].

In this contribution, we provide a text proposal to complete the BS studies in the 4DL/1UL TR 36.854-13 [18].

# 2.	Text Proposal

<Start of change>

#### 6.26.1.2	Co-existence studies for CA_2A-5A-12B

The 3rd IMD products caused in the BS by transmitting of CA_2A-5A-12B (i.e. 4DL CA with one component carrier on bands 2 and 5 and two carriers on band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR36.853-12 and TR36.853-13.

<New clause>

## 6.XX	LTE Advanced Carrier Aggregation of Band 2 and Band 4 and Band 12 and Band 12 (1 UL)

### 6.XX.1 	List of specific combination issues

#### 6.XX.1.1	Channel bandwidths per operating band for CA

Table 6.XX.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-4-12-12 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz |  |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |

Table 6.XX.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_2A-4A-12A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.XX.1.2	Co-existence studies for CA_2A-4A-12A-12A

The 3rd IMD products caused in the BS by transmitting of CA_2A-4A-12A-12A (i.e. 4DL CA with one component carrier in band 2 and 4 and two non-contiguous carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

## 6.YY	LTE Advanced Carrier Aggregation of Band 4 and Band 4 and Band 12 and Band 12 (1 UL)

### 6.YY.1 	List of specific combination issues

#### 6.YY.1.1	Channel bandwidths per operating band for CA

Table 6.YY.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_4-4-12-12 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |

Table 6.YY.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 |  |  | 5 | 10 |  | 15 |  | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz |  |  | MHz | MHz |  | MHz |  | MHz | [MHz] |  |
| CA_4A-4A-12A-12A | 4 | See 36.101 V13.0.0 table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  | Yes |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.YY.1.2	Co-existence studies for CA_4A-4A-12A-12A

The 3rd IMD products caused in the BS by transmitting of CA_4A-4A-12A-12A (i.e. 4DL CA with two non-contiguous component carriers in band 4 and two non-contiguous carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

## 6.ZZ	LTE Advanced Carrier Aggregation of Band 4 and Band 5 and Band 12 and Band 12 (1 UL)

### 6.ZZ.1 	List of specific combination issues

#### 6.ZZ.1.1	Channel bandwidths per operating band for CA

Table 6.ZZ.1.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_4-5-12-12 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |

Table 6.ZZ.1.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_4A-5A-12A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

#### 6.ZZ.1.2	Co-existence studies for CA_4A-5A-12A-12A

The 3rd IMD products caused in the BS by transmitting of CA_4A-5A-12A-12A (i.e. 4DL CA with one component carrier in band 4 and 5 and two non-contiguous carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

<End of change>

# References

[1]	RP-150728, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 4, Band 5 and Band 12”, U.S. Cellular.

[2]	RP-151018, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 5 and Band 12”, U.S. Cellular.

[3]	RP-151019, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 5, Band 12 and Band 12”, U.S. Cellular.

[4]	RP-150731, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 4 and Band 5”, U.S. Cellular.

[5]	RP-150732, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 12 and Band 12”, U.S. Cellular.

[6]	RP-151208, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 4, Band 4, Band 5 and Band 12”, U.S. Cellular.

[7]	RP-151521, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 4, Band 5, Band 12 and Band 12”, U.S. Cellular.

[8]	RP-151210, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 4, Band 4 and Band 5”, U.S. Cellular.

[9]	RP-151522, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 4, Band 4, Band 12 and Band 12”, U.S. Cellular.

[10]	RP-151523, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 4, Band 12 and Band 12”, U.S. Cellular.

[11]	R4-153995, “Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12”, Alcatel-Lucen, U.S. Cellular.

[12]	R4-153996, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (5 + 12 + 12)”, Alcatel-Lucen, U.S. Cellular.

[13]	R4-156397, “Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12 (RAN#69)”, Alcatel-Lucen, U.S. Cellular.

[14]	R4-156398, “TP for TR 36.852-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 12”, Alcatel-Lucen, U.S. Cellular.

[15]	R4-156399, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 12 + 12)”, Alcatel-Lucen, U.S. Cellular.

[16]	R4-156400, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 12 + 12)”, Alcatel-Lucen, U.S. Cellular.

[17]	R4-156401, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (5 + 12 + 12)”, Alcatel-Lucen, U.S. Cellular.

[18]	R4-157452, “TR 36.854-13 v0.5.0”, Nokia Networks.