3GPP TSG-RAN4 Meeting #77	                                             R4-158144

Anaheim, United States, 16th Nov 2015 - 20th Nov 2015

Source: 	Acorn Technologies, Huawei, HiSilicon

Title:  	Corrections, clarifications and enhancements to TS 36.133 RSTD test cases

Agenda Item:	7.48.4

Document for:	Discussion/Decision

# Discussion

3GPP LTE OTDOA has recently been evaluated in the indoor positioning study item that resulted in TR 37.857 “Study on Indoor Positioning Enhancements for UTRA and LTE” [1].  It was concluded that OTDOA has the potential to meet the FCC E911 requirements published in [5].  From section 6.1.3 in the TR:

“The simulation results for the OTDOA simulation assumptions show that horizontal

positioning accuracy for indoor positioning can be achieved within the 50 meter threshold.”

OTDOA performance depends on accurate RSTD reporting from the UE.  3GPP TS 36.133 specifies RSTD measurement procedure requirements in section 8.1.2.5, accuracy requirements in section 9.1.10, and a variety of test cases in sections A.8.12 and A.9.8.  (In this discussion, we study these sections noting that the analysis applies to the related sections for inter-frequency and CA measurements.)

Incorporating the teachings of the recent TR, this contribution revisits TS 36.133 RSTD requirements, making observations and proposals for improvements.  This discussion paper accompanies the TS 36.133 CR document R4-158145.

## The use of multipath fading channel models

The simulation study in the TR uses the 3D Urban Macro (3D-UMa) and 3D Urban Micro (3D-UMi) channel models [1].  These channel models have an RMS delay spread that ranges from 65ns to 363ns [8].

The 36.133 test cases use the ETU channel model in section A.8.12, and the AWGN channel model in section A.9.8.  The ETU channel model has an RMS delay spread of 991ns, and the AWGN channel model (no multipath, no fading) has an RMS delay spread of 0ns.  Thus relative to the TR, the ETU channel is overly pessimistic while the AWGN channel is overly optimistic.

The standard channel models EPA, EVA and ETU are used extensively in TS 36.101 [7] and commonly supported in testing equipment.  The RMS delay spread of EPA is 43ns, while the RMS delay spread for EVA is 357ns.  Therefore, among these common models, EVA has an RMS delay spread that is closest to the worst-case of the channel models used in the TR.  This observation is illustrated in Figure 1.  Adopting EVA in 36.133 RSTD test cases would better conform to the channel models used in the TR in terms of RMS delay spread, with minimal impact to common testing methods (eg, EVA is extensively used in 36.101 and commonly supported in testing equipment).

Observation 1: Relative to TR 37.857, the ETU channel is overly pessimistic while the AWGN channel is overly optimistic.

Observation 2: EVA is most in-line with the worst-case channel models in the TR in terms of RMS delay spread.

Figure 1: Current 36.133 RSTD test cases use AWGN (overly optimistic) or ETU (overly pessimistic).  The EVA channel model is more in line with the 3D-UMa and 3D-UMi models adopted in TR 37.857.

## Clarification in test cases

The test cases in section A.9.8 are for RSTD measurement accuracy and are directly linked to the RSTD measurement performance requirements in section 9.1.10.  To pass one of these test cases, the UE must, for example, achieve a 5Ts accuracy when a 50 resource block PRS bandwidth is available.  These test cases are for the AWGN channel model with no multipath and no fading and a resulting RMS delay spread of 0ns.

The test cases in section A.8.12 are for RSTD measurement reporting delay and are directly linked to the RSTD measurement procedure requirements in section 8.1.2.5.  To pass one of these test cases, the UE must, for example, report RSTD measurements within 2560ms after the start of the first PRS occasion in the test.  However, the test case is not directly linked to the RSTD measurement performance requirements in section 9.1.10.  Instead, a correct event is defined to be when the UE reports any value from the RSTD measurement report mapping table in section 9.1.10.3:

The rate of the correct events for each neighbour cell observed during repeated tests shall be at

least 90%, where the reported RSTD measurement for each correct event shall be within the

RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

This reporting range of -15391Ts to +15391Ts does not consider the RSTD measurement performance requirement from section 9.1.10 (ie, 5Ts with 50 resource block PRS bandwidth).  But, the RSTD measurement procedure defined in section 8.1.2.5 does have a direct link to the RSTD measurement performance requirement in section 9.1.10:

The RSTD measurement accuracy for all measured neighbor cells i shall be fulfilled according

to the accuracy as specified in the sub-clause 9.1.10.1.

Therefore, A.8.12 needs clarification.  Figure 2 illustrates these observations for A.8.12.1.

Figure 2: Current TS 36.133 RSTD measurement reporting delay test cases (eg, A.8.12) do not consider measurement accuracy performance (eg, 9.1.10).

Proposal 1: Clarify accuracy requirements for RSTD test cases in section A.8.12.

## Summary of 36.133 CR (R4-158145)

The accompanying CR document R4-158145 implements Proposal 1. Figure 3 illustrates the outcome of incorporating these changes.  Figure 4 shows example text in the CR.  The changes to the TS are straightforward, and the outcome is improved clarity.

Figure 3: Outcome of proposed 36.133 CR (R4-158145) clarifying requirements for section A.8.12

Figure 4: Example text from 36.133 CR (R4-158145).

# Conclusion

This contribution studies the current state of 3GPP TS 36.133 in light of the recent teachings of the TR 37.875.  RAN4 is kindly asked to consider the following observations and proposals.

Observation 1: Relative to TR 37.857, the ETU channel is overly pessimistic while the AWGN channel is overly optimistic.

Observation 2: EVA is most in-line with the worst-case channel models in the TR in terms of RMS delay spread.

Proposal 1: Clarify accuracy requirements for RSTD test cases in section A.8.12.

Proposal 2: Approve the TS 36.133 CR in Tdoc R4-158145.

# References

[1]	3GPP TR 37.857, “Study on Indoor Positioning Enhancements for UTRA and LTE”

[2]	3GPP TR 36.855, “Feasibility of positioning enhancements for E-UTRA.”

[3]	3GPP TS 36.355, “LTE Positioning Protocol (LPP)”

[4]	3GPP TS 36.133, “Requirements for support of radio resource management”

[5]	FCC Fourth Report and Order, FCC 15-9, February 2015

[6]	R4-103439, “E-UTRAN FDD Intra Frequency RSTD Measurement Accuracy test case”, Huawei, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #56, August 2010

[7]	3GPP TS 36.101, “User Equipment (UE) radio transmission and reception”

[8]	TR 36.873, “Study on 3D channel model for LTE”