# 3GPP TSG-RAN WG4 Meeting #80Bis						           R4-167436

# Ljubljana, Slovenia, October, 10-14, 2016

Agenda Item:		8.1.2

Source: 		CMCC, Qualcomm Incorporated

Title: 	TP on TR 36.714-00-00 for CA_40D and CA_41D

Document for:		Approval

- Introduction

Intra-band contiguous CA of 3UL in B40 and B41 were approved in Ran#72 meeting [1]. This TP is based on the analysis in [2].

- Reference

- RP-160822, Revised WID: LTE Advanced intra-band CA Rel-14 for xDL/yUL including contiguous and non-contiguous spectrum

- R4-166106, Discussion on RF requirements for 3UL intra-band contiguous CA of CA_40D and CA_41D, CMCC

- Text Proposal

<Start of Text Proposal for TR 36.714-00-00>

- 6.9	CA_40D_3UL_BCS0

- 6.9.5 Transmit modulation quality

Requirements for bandwidth class B and C can be reused.

- 6.9.6 Spectrum emission mask

Table 6.9.6-1 shows the SEM requirement for bandwidth class D.

Table 6.9.6-1: General E-UTRA CA spectrum emission mask for Bandwidth Class D

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB | 50RB+75RB+100RB | 75RB+75RB+75RB | 50RB+100RB+100 RB | 75RB+75RB+100RB | 75RB+100RB+100RB | 100RB+100RB+100 RB | Measurement bandwidth |
| (MHz) | (44.6MHz) | (45MHz) | (49.7MHz) | (49.85MHz) | (54.65MHz) | (59.6MHz) |  |
|  0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
|  1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
|  5-44.6 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  44.6-45 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  45-49.6 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.6-49.7 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.7-49.85 |  | -25 | -25 | -13 | -13 | -13 | 1 MHz |
|  49.85-50 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
|  50-54.65 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
|  54.65-54.7 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
|  54.7-54.85 |  |  |  | -25 | -25 | -13 | 1 MHz |
|  54.85-59.6 |  |  |  |  | -25 | -13 | 1 MHz |
|  59.6-59.65 |  |  |  |  | -25 | -25 | 1 MHz |
|  59.65-64.6 |  |  |  |  |  | -25 | 1 MHz |

- 6.9.8 Spurious emission band UE co-existence

Table 6.9.8-1 shows the spurious emission band UE co-existence for CA_40D.

Table 6.9.8-1: Spurious emission band UE co-existence Requirements

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_40D | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 27, 32, 33, 34, 38, 39, 41, 42, 43, 44, 65, 67 | FDL_low | - | FDL_high | -50 | 1 |  |

- 6.10	CA_41D_3UL_BCS0

- 6.10.3 Spurious emission band UE co-existence

Table 6.10.3-1 shows the spurious emission band UE co-existence for CA_41D.

Table 6.y.3-1: Spurious emission band UE co-existence for CA_41D

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41D | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 65, 66,70 | FDL_low | - | FDL_high | -50 | 1 |  |

<End of Text Proposal for TR 36.714-00-00>