3GPP TSG-RAN4 Meeting #80bis                                           R4-167737

# Ljubljana, Slovenia, October 10-14 2016

Source: 	SONY Mobile

Title: 		Measured TRP/TRS results for some Sony phones

Agenda Item: 	8.13.1

Document for:	Discussion

# 1.		Introduction R4-125719

A mix of Sony smart-phones launched to various markets during 2015/16 has been measured for LTE-bands TRP/TRS according to R4 #80 WF [1] and to the CTIA test plan for BHH and H testing conditions. RX BW is 10 MHz. All phones are measured with PDA hands (w<72 mm).

2.	Results for Beside Head and Hand.

| Band | BHH dBm | Phone |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F | G | H | J |
| 3 | TRP | 13.9 | 15.8 | 15.4 | 13.6 | 13.1 | 12.2 | 14.1 | 13.3 | 14.0 |
|  | TRS | -89.5 | -89.8 | -90.9 | -91.1 | -88.4 | -88.3 | -90.1 | -91.0 | -90.7 |
| 7 | TRP | 12.8 | 14.9 | 14.4 | 11.8 | 13.8 | 15.0 | 14.5 | 16.1 | 13.4 |
|  | TRS | -86 | -90.9 | -90.2 | -89.9 | -90.2 | -88.3 | -87.6 | -87.6 | -90.6 |
| 20 | TRP | 9.6 | 11.0 | 10.7 | 9.0 | 9.9 |  |  |  |  |
|  | TRS | -86.4 | -86.8 | -86.6 | -86.7 | -86.5 |  |  |  |  |
| 1 | TRP | 16.9 | 16.9 | 14.6 | 12.8 | 14.6 |  |  |  |  |
|  | TRS | -89.7 | -91.8 | -89.2 | -88.3 | -92.2 |  |  |  |  |
| 4 | TRP | 14.7 | 15.7 | 15.2 | 14.0 |  |  |  |  |  |
|  | TRS | -90.5 | -91.5 | -89.8 | -86.6 |  |  |  |  |  |
| 8 | TRP | 10.3 | 10.7 | 9.8 | 10.4 | 7.1 |  |  |  |  |
|  | TRS | -86.1 | -86.2 | -87.3 | -85.2 | -83.9 |  |  |  |  |
| Notes | #LTE bands | 13 | 13 | 17 | 19 | 6 | 9 | 9 | 13 | 11 |
|  | CA Interband | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes | Yes |
|  | Metal frame and/or back | Yes | No | Yes | Yes | No | Yes | Yes | Yes | Yes |
|  | Primary market | EU | EU | EU | EU | Global | JP | JP | JP | JP |

3.	Results for Hand only.

Data is unfortunately available for three phones only.

| Band | Hand dBm | Phone |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F | G | H | J |
| 3 | TRP | 15.7 | 15.9 |  |  | 15.5 |  |  |  |  |
|  | TRS | -92.9 | -91.4 |  |  | -94.8 |  |  |  |  |
| 7 | TRP | 15.6 | 14.7 |  |  | 14.5 |  |  |  |  |
|  | TRS | -90.1 | -92.5 |  |  | -95.2 |  |  |  |  |
| 20 | TRP | 15.3 | 14.2 |  |  | 13.0 |  |  |  |  |
|  | TRS | -90.3 | -88.8 |  |  | -89.0 |  |  |  |  |
| 1 | TRP |  |  |  |  |  |  |  |  |  |
|  | TRS |  |  |  |  |  |  |  |  |  |
| 4 | TRP |  |  |  |  |  |  |  |  |  |
|  | TRS |  |  |  |  |  |  |  |  |  |
| 8 | TRP |  |  |  |  |  |  |  |  |  |
|  | TRS |  |  |  |  |  |  |  |  |  |
| Notes | # LTE bands | 14 | 14 | 18 | 20 | 6 | 9 | 9 | 13 | 11 |
|  | CA Interband | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes | Yes |
|  | Metal frame and/or back | Yes | No | Yes | Yes | No | Yes | Yes | Yes | Yes |
|  | Primary market | EU | EU | EU | EU | Global | JP | JP | JP | JP |

4.	Conclusion

In line with the WF [1] TRP/TRS measurement data for 9 different 2015/16 year Sony models have here been provided.

5.	References

[1].	R4-16 7201, Way forward on LTE TRP/TRS framework improvement and work plan.