3GPP TSG-RAN WG4 Meeting #80bis	      		R4-167738

Ljubljana, Slovenia, 10 – 14 October, 2016

Source: 	Sony

Title:		RX requirements for new UE category with single receiver based on Category 1

Agenda Item:	8.27.2

Document for:	Discussion

# Introduction

A new WI “Requirements for a new UE category with single receiver based on Category 1 for LTE” was decided in RANP #73, September 2016 [1].

Objective for RAN4:

RAN4 has to develop the receiver core requirements. A lot of the work can leverage the requirements developed for Category 0 UEs since many of the requirements do not depend on the data rate capabilities of the UEs. The detailed objectives are:

- Specify RF receiver core requirements for Category 1 UE with a single receiver [RAN4]

  - Previous work for Category 0 UE should be leveraged

- Specify RRM core requirements for Category 1 UE with a single receiver for intra-frequency and inter-frequency mobility [RAN4]

  - RLM and intra-frequency mobility requirements (measurements) for Category 0 UE could be reused

In this contribution we will discuss REFSENS for the new UE category with single receiver based on Category 1 based on REFSENS for UE category 0 full duplex.

# Discussion

For rel-12 MTC cat 0 there was a consensus of 2.5dB total relaxation for FDD and TDD for single RX with exception for Band 13 and Band 20 for 15, 20MHz BW [2], [3].

Due to the large similarities with cat 0 it is proposed to re-use the same REFSENS relaxation principle for the new UE category with single receiver, as used in rel-12 cat 0 MTC.

- Proposal	Re-use the same REFSENS relaxation principle for the new UE category with single receiver based on cat 1, as used in rel-12 cat 0 MTC.

From similarities with cat 0 it is found that Adjacent Channel selectivity, In-band blocking, Out-of-band blocking, Narrow band blocking, Spurious response and Wide band Intermodulation all relates to REFSENS and could be reused just adding a note referring to the RESENS table for the new category.

Specifications for Maximum input signal, Spurious emissions and Receiver image could be directly reused from cat 1.

# Conclusion

In this contribution we have made an early assessment of the RX parameters for the new UE category with single receiver based on Category 1 and found the following:

- Proposal	Re-use the same REFSENS relaxation principle for the new UE category with single receiver based on cat 1, as used in rel-12 cat 0 MTC.

Adjacent Channel selectivity, In-band blocking, Out-of-band blocking, Narrow band blocking, Spurious response and Wide band Intermodulation all relates to REFSENS and could be reused just adding a note referring to the RESENS table for the new category.

Specifications for Maximum input signal, Spurious emissions and Receiver image could be directly reused from cat 1.

# References

RP-161898	“New Work Item Proposal: Requirements for single receiver UE based on Category 1 for LTE”, Qualcomm Incorporated

R4-147954	“Reference sensitivity WF for MTC”, Vodafone

R4-151196	“Introduce additional bands of LC MTC”, Intel, Verizon, AT&T, T-Mobile, Qualcomm, Ericsson, ALU, MediaTek, LGE, Sequans