3GPP TSG RAN WG4 Meeting #80bis	R4-168618

Ljubljana, SI, 10 - 14 Oct 2016

Agenda Item:	8.24.4.2.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	NB-IoT UE Rx-Tx Simulation Assumptions

Document for:	Approval

# Introduction

In RAN4#80, NB-IoT E-CID positioning requirements were discussed (e.g., [1]). During the discussion, it was proposed to have simulation investigation on the NB-IoT UE Rx-Tx timing measurement performance.  In this paper, we propose the simulation assumptions for investigating NB-IoT UE Rx-Tx timing measurement performance.

# Simulation Assumptions

In order to progress NB-IoT UE Rx-Tx timing measurement performance, we propose to use the following table as the baseline for the discussion of simulation investigation of the NB-IoT UE Rx-Tx timing measurement accuracy.

Table 1: Simulation parameters for NB-IoT UE Rx-Tx timing measurement accuracy

| Parameter |  | Simulation set | Comment |
| --- | --- | --- | --- |
| Channel model |  | AWGN, ETU 1Hz, EPA 1Hz | Models stationary device |
| System bandwidth |  | 180kHz (1 PRB) |  |
| Antenna configuration | In-band Guard-band | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern |
|  | Stand-alone | 1TX, 1RX |  |
| Power boosting for NB-IoT | In-band Guard-band | 0 dB, 6 dB | NB-IoT is transmitted at 0dB or 6dB higher power than the MBB LTE cell to which it is associated. |
|  | Stand-alone | N/A |  |
| Cyclic prefix configuration |  | Normal CP | CP lengths 10 and 9 samples for first and following symbols in a slot, respectively, at 1.92MS/s. |
| NB-IoT load condition |  | Fully loaded, modelled by OCNG | OFDM symbols 3-13. |
| LTE cell load condition | In-band | Fully loaded, modelled by OCNG | OFDM symbols 0-2. (Control region) |
|  | Guard-band Stand-alone | N/A |  |
| Interference |  | AWGN |  |
| Frequency error |  | ±18 kHz | Models wake-up after eDRX inactivity |
| SNR range |  | -20 to 10 dB | Cover extreme coverage enhancement and up to fair coverage scenarios |
| NB-PSS periodicity |  | Transmitted in subframe #5 every 10ms | According to RAN1 |
| NB-SSS periodicities |  | Transmitted in subframe #9 every 20ms | According to RAN1 |
| Algorithms for cell detection |  | Implementation specific |  |

Proposal 1: Use the above table 1 as baseline for discussing NB-IoT UE Rx-Tx timing measurement accuracy simulation assumptions.

# Summary

In this paper, it is proposed that:

Proposal 1: Use Table 1 in this paper as the simulation assumption for investigating NB-IoT UE Rx-Tx timing measurement performance.

# References

- R4-165456, eNB-IoT E-CID Positioning, Nokia

- 3GPP TS 36.133

- RP-161324, “New work item proposal: Enhancements of NB-IoT”, Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm