3GPP TSG-RAN WG4 (Radio) Meeting #80bis  	R4-168953

Ljubljana, Slovenia, 10 – 14 October, 2016

Agenda item:	10.3.1

Source: 	NTT DOCOMO, INC.

Title: 	WF on Simulation assumptions of Co-existence study for WP5D

Document for:	Approval

Introduction

The contribution summarizes updated simulation assumptions of Co-existence study for WP5D. Updated assumptions are highlighted in yellow.

Network layout model

Urban macro

Single operator layout

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | Case 1: 500m Case 2: 300m | Note 1, 2 |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | Case 1: 80% Case 2: 20% | Note 1, 2 |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa | Specified in TR38.900 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note: If we find any issue, then we can revisit parameters. Other cases are not precluded in addition to Case 1 and 2. Note 2: Case 2 is used in scenario No. 11 described in section 11. |  |  |  |

Multi operators layout

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Dense urban

Single operator layout

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 28.9 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi | Specified in TR38.900 |
| Shadowing correlation |  | Between cite: 0.5 | Table 6.3.2.1 in TR37.809 |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with ISD = 200m is assumed. |  |  |  |

Multi operators scenario

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

Indoor

Single operator layout

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | 1 m | Specified in TR38.900 |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office | Specified in TR38.900 |
| Shadowing correlation |  | NA |  |

Multi operators layout

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

Propagation model

Path loss

The pathloss models are summarized in Table 7.4.1-1 and the distance definitions are indicated in Figure 7.4.1-1 and Figure 7.4.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 7.4.1-1.

| Figure7.4.1-1: Definition of d2D and d3D for outdoor UEs | Figure 7.4.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Table 7.4.1-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1.

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

O-to-I penetration loss

The pathloss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 7.4.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 7.4.3-1. Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 7.4.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 7.4.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

Transmission power control model

For downlink, no power control scheme is applied.

For uplink, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

- CL-xile = 88 + 10*log10(200/X)

  - X: UL transmission BW

- Gamma = 1

Note: we need discuss KPI based on simulation results.

Received signal power model

The following model is applied.

RX_PWR = TX_PWR – pathloss + G_TX + G_RX

where:

RX_PWR is the received signal power

TX_PWR is the transmitted signal power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | note 1 | note 1 | note 1 |
| UE Noise figure in dB | note 1 | note 1 | note 1 |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: At least the following NF are used for simulation assumptions for the RAN4#81 meeting for both UE and BS. 30GHz: 9 and 11 dB, 45GHz: 11 and 13 dB, 70GHz: 13 and 15 dB. |  |  |  |

Simulation description

Adopt following simulation steps. If companies find issues in the following simulation steps, we can revisit in RAN4#80bis. Note: detailed simulation description is captured in Section 5.1.5 TR25.942.

- Aggressor and victim network are generated.

- UE associations: UE are associated to base station based on coupling loss.

  - Associations are made assuming a single element at both UE and BS.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This done for both victim and aggressor networks.

- SINR Throughput are measured in the victim systems without considering ACI, i.e. ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- SINR and throughput are computed considering ACI: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based degradation cause by ACI: LossACI=1-ThputACIThputSINGLE.

Evaluation metric

Assume scaled Shannon's formula specified in TR36.942 with following parameters.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(UL) & 64QAM 0.93 (UL) |

ACIR range to be evaluated

ACIR range: 5 ~ 45dB, with 5dB as step. Note that: we do not have simulation per 1dB step.

Antenna configuration

Assumptions captured in page 8 - 11 of [R4-168794] are assumed as BS antenna configurations.

Assumptions captured in [R4-167123] are assumed as UE antenna configurations without an additional 3dB gain. Note: R4-167750 provides further explanation.

Co-existence scenarios

Evaluate following scenarios for WP 5D.

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Indoor hotspot |
| 2 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Urban macro |
| 3 | NR, 200MHz | NR, 200MHz | 30 GHz | DL to DL | eMBB | Dense urban |
| 4 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Indoor hotspot |
| 5 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro |
| 6 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Dense urban |
| 7 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Indoor hotspot |
| 8 | NR, 200MHz | NR, 200MHz | 70 GHz | DL to DL | eMBB | Dense urban |
| 9 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Indoor hotspot |
| 10 | NR, 200MHz | NR, 200MHz | 70 GHz | UL to UL | eMBB | Dense urban |
| 11 | NR, 20MHz | NR, 20MHz | 30 GHz | UL to UL | eMBB | Urban macro |

If companies want to provide results for 45GHz for indoor hotspot and dense urban micro, it is optional.