3GPP TSG-RAN WG4 Meeting #78	 R4-160076

# Malta, Malta, 15- 19 Feb, 2016

Agenda Item:	6.8.1

Source: 	Samsung

Title: 	Impairment results for FDD case

Document for:	Discussion

1	Introduction

A new work item “CRS Interference Mitigation for LTE Homogenous Deployments” was approved in RAN#66. As described in WI scope, DM-RS based transmission modes (including TM9 and TM10) should be covered on this WI.

In this contribution, we provide impairment results for CRS-IM demodulation test cases.

- Analysis

Test case list

For CRS-IM gain test, such test cases were agreed for non-TM10 and TM10 mode.

Table 1: Test case list

| Test case | MCS |
| --- | --- |
| TM4 | MCS= 16 |
| TM9 | MCS= 14 |
| TM10 (single CSI Process) | MCS [14, or 16] |
| TM10 (multiple CSI process) | MCS [14, or 16] |

Alignment simulation results

Simulation results for TM10 (single CSI process and multiple CSI process) with MCS 14 and 16 were given figure below.

Impairment result

The reference SNR points for 70% relative throughput were summarized in table 2 below:

Table 2: Impairment results for FDD case

| Test case | MCS | SNR [dB] |
| --- | --- | --- |
| TM4 | 16 | 9.7 |
| TM9 | 14 | 10.4 |
| TM10 (single CSI Process) | 14 | 10.8 |
| TM10 (multiple CSI process) | 14 | 10.8 |

- Conclusion

In this contribution, we provide impairment results for CRS-IM demodulation test cases.