3GPP TSG-RAN WG4 Meeting #78	                                                                         R4-160079

St. Julian’s, Malta, 15 – 19 February, 2016

Source: 	Samsung

Title: 	Simulation Results for 3/4-Layer PDSCH Demodulation with 4RX

Agenda Item:	6.8.2.1

Document for:	Discussion

Introduction

In the RAN4 #77 meeting, there had been the discussion on the 3/4-layer PDSCH demodulation requirements, and the following agreement had been captured in the Ad-Hoc meeting minutes [1]:

Configurations for alignment and impairment simulation  Test 1: 3layer, TM3, 4x4 low, EVA70 • MCS=18  Test 2: 4 layer, TM4,4x4 low, EPA5, followed wideband PMI • MCS=14  Test 3: 4 layer, TM9,4x4 low, EPA5, followed wideband PMI • MCS=14  For simulation alignment assume 6% EVM.

In the email thread on RAN4 reflector, RAN4 has agreed the following detailed simulation assumption to be used for alignment purpose [2], which has been captured in the Appendix of this contribution.

In this contribution, we will provide our simulation results for 3/4-layer PDSCH demodulation tests for alignment based on the agreed simulation assumption.

4RX PDSCH demodulation Tests for 3/4-Layer

Based on the detailed simulation assumption agreed on RAN4 email reflector, we would like to provide FDD simulation results for alignment purpose. The simulation results for TM3 with 3-layer, TM4 with 4-layer and TM9 with 4-layer are illustrated in the following Figure 1 to Figure 3, respectively.

Figure 1.  PDSCH Demodulation Performance for TM3 with 3-Layer

Figure 2. PDSCH demodulation performance for TM4 with 4-Layer

Figure 3. PDSCH demodulation performance for TM9 with 4-Layer

Based on our simulation results, we can get the reference SNR value corresponding to 70 percentage of maximum throughput, which has been summarized in the following table:

Table 1. Reference Demodulation SNR Point for Test Cases

| Test cases | TM3, 3-Layer | TM4, 4-Layer | TM9, 4-Layer |
| --- | --- | --- | --- |
| Reference SNR Value | 12.8dB | 12.3dB | 16.0dB |

Conclusion

In this contribution, we provide our simulation results for 3/4-layer PDSCH demodulation, based on the detailed simulation assumption agreed on RAN4 email reflector. For the reference SNR values corresponding to 70 percentage of maximum throughput for three test cases, we summarized in the following table:

Table 1. Reference Demodulation SNR Point for Test Cases

| Test cases | TM3, 3-Layer | TM4, 4-Layer | TM9, 4-Layer |
| --- | --- | --- | --- |
| Reference SNR Value | 12.8dB | 12.3dB | 16.0dB |

References

R4-158390, Ad hoc minutes for 4Rx RRM and demodulation.

RAN4 email discussion: simulation assumption for rank 3/4 PDSCH demodulation test.

Appendix- Simulation Assumption

The detailed description including FRC is provided in the RAN4 email reflector, which has been provided here for convenience.

# FDD TM3/TM4 test configuration

| Parameter |  | Unit | TM3 test | TM4 test |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | 4 |
| PDSCH rank |  |  | 3 | 4 |
| Number of HARQ processes |  |  | 8 | 8 |
| HARQ redundancy version |  |  | {0,0,1,2} | {0,1,2,3} |
| CFI |  |  | 2 | 2 |
| Precoding granularity |  | PRB |  | 50 |
| PMI delay |  | ms |  | 8 |
| Reporting interval |  | ms |  | 1 |
| Reporting mode |  |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  |  | FFFF000000000000 |
| Antenna configuration |  |  | 4x4 | 4x4 |
| Propagation channel |  |  | EVA70 Low Correlation | EPA5 Low Correlation |
| Note 1: . |  |  |  |  |

# FRC for FDD TM3/TM4 tests

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.xx FDD (TM3) | R.14 FDD (TM4) |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks (Note 4) |  | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 |
| Modulation |  | 64QAM | 16QAM |
| Target Coding Rate |  | 0.43 | 1/2 |
| Information Bit Payload (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 16416 (CW0) 32856 (CW1) MCS 18 | 25456 MCS 14 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 15264 (CW0) 30576 (CW1) MCS 17 | 22920 MCS 13 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 |  | 3 (CW0) 6 (CW1) | 5 |
| For Sub-Frame 5 |  | N/A | N/A |
| For Sub-Frame 0 |  | 3 (CW0) 5 (CW1) | 5 |
| Binary Channel Bits (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 38400 (CW0) 76800 (CW1) | 51200 |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 36096 (CW0) 72192 (CW1) | 48128 |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 14.659 (CW0) 29.342 (CW1) | 22.657 |
| UE Category |  | ≥ 2 | ≥ 2 |

# FDD TM9 test configuration

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | 4 layer precoding based on WB PMI feedback |
| Random beamforming granularity |  |  | 3 PRB in frequency domain 1 ms in time domain |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 0 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Number of HARQ processes |  |  | 8 |
| HARQ redundancy version |  |  | {0,1,2,3} |
| CFI |  |  | 2 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 8 |
| Reporting interval |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | FFFF000000000000 |
| Antenna configuration |  |  | 4x4 |
| Propagation channel |  |  | EPA5 Low Correlation |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

# FRC for FDD TM9 test

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.yy FDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.57 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 25456 MCS 14 |
| For Sub-Frames 2,3,7,8 | Bits | 25456 MCS 14 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 21384 MCS 14 |
| Number of Code Blocks (Note 4) |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 5 |
| For Sub-Frames 2,3,7,8 | Code blocks | 5 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 4 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 43200 |
| For Sub-Frames 2,3,7,8 |  | 41600 |
| For Sub-Frame 5 | Bits | N/A |
| For Sub-Frame 0 | Bits | 35424 |
| Max. Throughput averaged over 1 frame | Mbps | 22.503 |
| UE Category |  | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |