3GPP TSG-RAN Working Group 4 Meeting #78	R4-160230

Malta, 15th February – 19th February 2016

Source:	DISH Network

Title:	TP for TR 36.714-02-01: Section 6.X.3 CA_2A-66A Quadplexer Data

Agenda item:	8.2.1

Document for:	Approval

## 1	Introduction

In this contribution, quadplexer data for CA_2A-66A is presented.

## 2	Discussion

Simulated quadplexer data supporting inter-band carrier aggregation between Bands 2 and 66 is shown in Table 1.  The data provided is all under extreme temperature condition (ETC) unless noted otherwise.

Table 1.  Simulated quadplexer data for CA_2A-66A

|  | Band 2 |  |  |  | Band 66 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TX IL, Max | RX IL, Max | TX Isolation, Min | RX Isolation, Min | TX IL | RX IL | TX Isolation, Min | RX Isolation, Min |
| Vendor A1 | 1.7 dB | 2.5 dB | > 60 dB | > 60 dB | 1.5 dB | 2.2 dB | > 60 dB | > 60 dB |
| Vendor B | 3.5 dB | 3.5 dB | 55 dB | 55 dB | 2.7 dB | 2.7 dB | 55 dB | 55 dB |
| Vendor C | 3.9 dB | 4.2 dB | 59 dB2 | 55 dB2 | 2.3 dB | 2.3 dB | 58 dB2 | 59 dB2 |

1  Bands 2/25 co-banded

2  Typical values

## 3 				Conclusions

It is proposed that the above quadplexer data be included in Section 6.X.3 of TR 36.714-02-01 [1].

## References

R4-xxxxx, “Skeleton TR 36.869 v0.0.2,” Qualcomm, 3GPP RAN4 #78, Malta, February 2016

## << Unchanged sections omitted >>

## << Start of changed sections >>

## 6.X	CA_2A-66A_BCSn

### 6.X.1	Operating bands for CA

<Text will be added.>

### 6.X.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 6.X. 2	Co-existence studies

<Text will be added.>

### 6.X. 3	∆TIB and ∆RIB values

Simulated quadplexer data supporting inter-band carrier aggregation between Bands 2 and 66 is shown in Table 6.X.3-1.  The data provided is all under extreme temperature condition (ETC) unless noted otherwise.

Table 6.X.3.-1.  Simulated quadplexer data for CA_2A-66A

|  | Band 2 |  |  |  | Band 66 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TX IL, Max | RX IL, Max | TX Isolation, Min | RX Isolation, Min | TX IL | RX IL | TX Isolation, Min | RX Isolation, Min |
| Vendor A1 | 1.7 dB | 2.5 dB | > 60 dB | > 60 dB | 1.5 dB | 2.2 dB | > 60 dB | > 60 dB |
| Vendor B | 3.5 dB | 3.5 dB | 55 dB | 55 dB | 2.7 dB | 2.7 dB | 55 dB | 55 dB |
| Vendor C | 3.9 dB | 4.2 dB | 59 dB2 | 55 dB2 | 2.3 dB | 2.3 dB | 58 dB2 | 59 dB2 |

1  Bands 2/25 co-banded

2  Typical values

### 6.X. 4	REFSENS requirements

<Text will be added if it’s necessary.>

## << End of changed sections >>