3GPP TSG-RAN WG4 Meeting #78  R4-160155					R4-160242
St George's Bay, Malta, 15-19 February, 2016

Agenda item:	6.3.3 - Harmonization [LTE_MIMO_OTA-Core]

Source:	Rohde&Schwarz

Title:	Harmonization Analysis

Document for:	Discussion

As outlined in 37.977, three different harmonization options were selected for closer harmonization analyses, i.e., Options C, D, and G. These options vary in the averaging methods and the channel models considered. For the AC methodologies (RTS, MPAC), a set of three principal UE orientations (Portrait with 45o tilt-P45, Portrait with 90o tilt-P90, and Landscape with 45 o tilt-L45) were chosen to study harmonization with the 3D isotropic RC methodologies (RC, RC+CE). One key goal of the work item was to determine a harmonized MU between combinations of methodologies, h, to determine the outcome of harmonization.

The work on the measurement uncertainty (MU) resulted in typical MU budgets, m, for the respective methodologies considered (RC, RC+CE, RTS, and MPAC) as listed in Table 1.

| Method | MU [m] |
| --- | --- |
| RC | 1.86 |
| RC+CE | 2.59 |
| RTS | 2.08 |
| MPAC | 2.65 |

Table 1: Typical MU budgets for each methodology

Residual errors, r, were determined from the harmonization test campaign performed at CATR after applying a fixed offset per band to each methodology.

During the MIMO OTA ad-hoc meeting (RAN4#77AH-OTA), it was proposed to add another term to the harmonized MU term, i.e, the ADTF accuracy, x, listed in Table 2.

| Method | ADTF accuracy (REG AVG) [x] | ADTF accuracy (INV AVG) [x] |
| --- | --- | --- |
| RC | 1.3 | 1.2 |
| RC+CE | 1.6 | 1.6 |
| RTS | 0.6 | 0.6 |
| MPAC | 0.9 | 0.9 |

Table 2: ADTF accuracy terms for each methodology

The harmonized MUs, h, and the residual errors from the harmonization campaign, r, for the three harmonization options (C, D, and G), is shown in Table 3.

| Option | Band | RC&MPAC |  | RC+CE&MPAC |  | RTS&MPAC |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Residual Error at 70% [r] | Harmonized MU [h] | Residual Error at 70% [r] | Harmonized MU [h] | Residual Error at 70% [r] | Harmonized MU [h] |
| C | Band 13 | 0.9 | 4.4 | 0.3 | 4.5 | 0.1 | 3.6 |
|  | Band 7 | 0.6 | 4.1 | 0.5 | 4.7 | 0.4 | 3.9 |
|  | Band 41 | 0.2 | 3.8 | 0.4 | 4.6 |  |  |
| D | Band 13 | 1.2 | 4.7 | 0.6 | 4.8 | 0.2 | 3.7 |
|  | Band 7 | 0.6 | 4.1 | 0.5 | 4.7 | 0.4 | 3.9 |
|  | Band 41 | 0.3 | 3.8 | 0.4 | 4.5 |  |  |
| G (3 orientations) | Band 13 | 4.1 | 7.6 | 0.9 | 5.1 | 0.9 | 4.4 |
|  | Band 7 | 3.2 | 6.7 | 0.6 | 4.8 | 0.7 | 4.2 |
|  | Band 41 | 3.5 | 7.0 | 1.1 | 5.3 |  |  |

Table 3: Results for the harmonized MU and residual errors from the harmonization campaign (without the bias term from the additional UE testing).

The results for the harmonized MU, h, are displayed in Figure 1.

The calculations are provided in the Excel spreadsheet submitted with this document.

Figure 1: Harmonized MU, h, for the three different harmonization options: (a) Opt C, (b) Opt D, (c) Opt G.