3GPP TSG-RAN WG4 Meeting #78           						     							  R4-160538

# Malta,  15-19 Feb. 2016

Title:	MSR RF requirement for NB-IoT

Source:	CMCC

Agenda Item:	6.23.4

Document for:	Approval

- Introduction

In Budapest Ad-Hoc meeting, a way forward [1] was agreed for single RAT NB-IoT RF reuquirment including standalone and in/guard-band operation. In this paper, MSR requirement for NB-IoT is proposed.

- Discussion

  - Practical demand on involving NB-IoT in MSR Specification

So far GSM and UTRA are still widely used in operators’ network.  If one operator deploy GSM or UTRA in one band, and also want NB-IoT operates on the same band, it is more convenient for the operator to deploy or upgrade Multi-Standard Radio BS for the certain case.

Observation#1: it is necessary to introduce RF requirement for MSR including NB-IoT.

Proposal#1: MSR specification for NB-IoT should also be finished in the WI.

  - Band Category for MSR including NB-IoT

Multi-Standard Radio including NB-IoT can be categorized as following.

Case#1: (UTRA/E-UTRA) + GSM + standalone NB-IoT

Case#2: in-band/guard-band NB-IoT + UTRA

Case#3: in-band/guard-band NB-IoT + GSM

Note that NB-IoT can have higher spectrum power density by using Case 1 than Case 3, so case 3 is not a common case in practical network. There are two alternatives to introduce NB-IoT MSR in TS 37.104

Alternative#1: introduce new Band Category (e.g. BC4, BC5 and etc) in TS 37.104 for NB-IoT MSR

Alternative#2:  revise the definition of current Band Category taking following table as an example

| BC | Original | After |
| --- | --- | --- |
| BC1 | E-UTRA + UTRA | E-UTRA(in/guard-band NB-IoT) + UTRA |
| BC2 | E-UTRA/UTRA + GSM | (UTRA/E-UTRA) + GSM + standalone NB-IoT |

New Band Category may make the specification more complex, so we slightly prefer alternative 2.

Proposal#2: change the definition of Band Category to introduce NB-IoT in MSR specification.

  - RF requirement for MSR including NB-IoT

In TS 37.104 most of the requirements are referenced to other specification including TS 36.104, TS 25.104, TS 25.105 and TS 45.005. For NB-IoT in MSR specification, most of the RF requirements can also be directly referenced to standalone and in/guard-band  NB-IoT requirement in TS 36.104 in that way.

It was agreed that for unwanted emission requirement GSM or MSR mask for standalone and E-UTRA mask for in/guard-band. It is possible to reuse BC2 requirement for Case#1, and reuse BC1 requirement for Case#2

Proposal#3: reuse MSR BC2 unwanted emission requirements for MSR of standalone NB-IoT + GSM + (UTRA/E-UTRA)

Proposal#4: reuse MSR BC1&3 unwanted emission requirements for MSR of	In-band/guard-band NB-IoT + UTRA

- Conclusion

Observation#1: it is necessary to introduce RF requirement for MSR including NB-IoT.

Proposal#1: MSR specification for NB-IoT should also be finished in the WI.

Proposal#2: change the definition of Band Category to introduce NB-IoT in MSR specification.

Proposal#3: reuse MSR BC2 unwanted emission requirements for MSR of standalone NB-IoT + GSM + (UTRA/E-UTRA)

Proposal#4: reuse MSR BC1&3 unwanted emission requirements for MSR of	In-band/guard-band NB-IoT + UTRA

- Reference

[1] R4-77AH-IoT-0144, Way forward on BS RF requirements, Ericsson