# 3GPP TSG-RAN WG4 Meeting #78                                                            	R4-160548

St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Title: TP for TR 36.714-04-01 on  ΔTIB,c and ΔRIB,c for 4DL CA_8B-41C

Source: 	CMCC

Agenda Item:	8.4.1

Document for:	Approval

1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation (4DL/1UL) of Band 8, Band 8, Band41 and Band 41 was approved in RAN#70.

In this contribution, ΔTIB,c and ΔRIB,c values are proposed for LTE Advanced Carrier Aggregation (4DL/1UL) of Band 8, Band 8, Band41 and Band 41.

2.	Discussion

As shown in the table 2-1, the 3rd harmonic of band 8 will fall into DL of band 41.

# Table 2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |

However, no harmonic issues were found due to operators’ spectrum holdings. Therefore, it is not necessary to introduce harmonic trap filter for this specific combination.

For the UE which supports CA_8B-41C the ΔTIB,c is defined for applicable bands in table 2-2.

# Table 2-2:  ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8B-41C | 8 | 0.3 |
|  | 41 | 0.3 |

For the UE which supports CA_8B-41C the ΔRIB,c is defined for applicable bands in table 2-3.

# Table 2-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8B-41C | 8 | 0 |
|  | 41 | 0 |

3    Text Proposal

# <Start of change>

6           4 Band Carrier Aggregation with Single UL: Specific Band Combination Part

### 6.X	CA_8B-41C_BCS0

6.X.3         ∆TIB and ∆RIB values

For the UE which supports CA_8B-41C, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 6.X.3-1 and 6.X.3-2 respectively.

Table 6.X.3-1: ΔTIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8B-41C | 8 | 0.3 |
|  | 41 | 0.3 |

Table 6.X.3-2: ΔRIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8B-41C | 8 | 0 |
|  | 41 | 0 |

<End of change>