3GPP TSG-RAN WG4 Meeting #78 	R4-160878

Malta, February 15 – 19, 2015

Agenda item:	6.7.1.1

Source: 	Sony

Title: 	MPR and A-MPR suggestion for eMTC based on OOBE requirements

Document for:	Approval

# Summary

This contribution provides measurements results of A-MPR based on the 3GPP TS 36.101 [1] OOBE requirements.

# Introduction

3GPP TS 36.101 assume maximal transmission bandwidth of 6RB in eMTC devices. As devices keep operating in networks with equal or larger bandwidth, it is reasonable to assume that the OOBE specification will not change, aka eMTC device operating in 10MHz network will have to meet the same OOBE specifications as any other device in a same network.

We suggest reviewing the A-MPR rules in order to reduce PA back off and align requirements for different network configurations. We also observe slight relaxation of the required back off comparing to CAT1 requirements.

Device output power has a significant impact on the device cost, increase of the required output power is translated into higher of PA saturation point and linearity which typically lead to higher operating voltage, larger transistor sizes and increase in device maximal current which may require larger battery capacity or higher discharge rate batteries. Unfortunately those physical qualities are determined by the worst case operating point of the system. Therefore it is important to reduce the differences between this worst case operating point to other similar conditions in order to reduce PA linearity requirement on one hand and to improve the overall system performance on the other hand.

In addition, we know that cell radius (and MCL) in the UL is determined by device output power at its maximal possible output power, therefore the main factor in the cell coverage size is the power, while the allocation size has no impact on the radius. Therefore, if cell coverage is considered, it is essential to maintain the maximal output power for each possible network configuration (Nrb), the actual allocation size is less important in order to determine the cell size.

# Description of suggested maximal allowable relaxation

This section lists all requirements for the eMTC MPR (QPSK).

Table 3-1: New eMTC MPR

| Channel bandwidth (MHz) | Resource Blocks (Nrb) | MPR (dB) |
| --- | --- | --- |
| 1.4 | > 2 | ≤ 2 |
| 3 | > 2 | ≤ 2 |
| 5 | > 2 | ≤ 1 |
|  | > 4 | ≤ 2 |
| 10 | > 5 | ≤ 1 |

Higher MPR values for lower bandwidth (1.4 and 3MHz ) are essential in order to allow more efficient power amplifiers in commonly used network with system bandwidth higher than 5MHz

The section lists all requirements for the eMTC A-MPR in order to meet FCC requirements in case networks with 1.4 and 3MHz will be deployed. In order to meet NS_06/07 emission mask, we currently don’t foresee A-MPR requirements for network with bandwidth higher than 5MHz.

Table 3-2: New eMTC A-MPR

| Channel bandwidth (MHz) | Resource Blocks (Nrb) | A-MPR (dB) |
| --- | --- | --- |
| 1.4 | = 2 | ≤ 1 |
| 3 | = 2 | ≤ 1 |

# Description of measurements method

Using a widely available LTE PA we have verified maximum output power for all possible combination of Nrb, Lrb for RBstart=0 while meeting SEM, ACLR and EVM requirements.

The table above was derived by adjusting the output power (and therefore MPR) while meeting SEM and ACLR requirements for different network configurations (Nrb) and different allocation Lrb.

# Measurements results

This section presents measurements results using a typical LTE PA for different output power with different resource block allocation for system bandwidths of 1.4,3,5,10, 15, and 20 MHz. Pmax in the figures below refers to maximal linear power for LTE without MPR. The measurements were performed for allocation as close as possible to system bandwidth edge. MPR for allocation inside the channel are FFS

Figure ‎5-1: OOBE margin to spec with 1 resource blocks allocation (EVM limited)

Figure ‎5-2 : OOBE margin to spec with 2 resource blocks allocation (general SEM mask)

Figure ‎5-3 : OOBE margin to spec with 2 resource blocks allocation (Fails NS_06/NS_07 SEM mask)

Figure ‎5-4 : OOBE margin to spec with 3 resource blocks allocation

Figure ‎5-5 : OOBE margin to spec with 4 resource blocks allocation

Figure ‎5-6: OOBE margin to spec with 5 resource blocks allocation

Figure ‎5-7: OOBE margin to spec with 6 resource blocks allocation

# Conclusion

The above clearly shows that the legacy MPR and A-MPR tables can not be re-used for eMTC devices. We therefore suggest defining new dedicated MPR and A-MPR tables in order to balance performance for different configurations/ allocations. The power reduction may improve the PA efficiency, especially in 5,10,15 and 20MHz networks. The PA efficiency improvement is translated to up to 30% power saving in the PA, which translates to up to 26% reduction in peak power.

We suggest MPR scheme to cover generic requirements and A-MPR for “NS_06/NS_07” emission mask.

There is no power reduction for single RB and therefore there is no impact on cell coverage.

# References

- 3GPP TS 36.101 V13.2.1 (2016-01)