3GPP TSG-RAN WG4 Meeting #78	R4-161030

# St. Julian’s, Malta, 15 – 19 February, 2016

Agenda Item:	6.2	Base Station (BS) RF requirements for Active Antenna System (AAS)

6.2.3	Conformance requirements

Source:	Kathrein

Title:	Text proposal for chapter 10.3.1.1 (Test methods)

Document for:	Approval

# Introduction

Several test methodologies have been proposed in TR 37.840 [1]. The TR 37.842 [2] has to describe in sub-clause 10 the potential test methodologies for the AAS testing in detail.

In [3] a novel method for a one dimensional compact range chamber has been proposed. The method uses a one dimensional probe antenna for the generation of a quiet zone, where the radiated wave has a quasi plane wave character. In [4], we discussed the measurement uncertainties of our proposed method for multi column antennas in detail. We showed, that EIRP and EIS can be determined with very high accuracy for multi column antennas.

It is proposed to approve this TP and include it in the TR.

# References

[1]	TR 36.840,  Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station

[2]	TR 37.842,  Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

[3]	R4-75AH-AAS-0004, One dimensional Compact Range Chamber, Kathrein

[4]	R4-157666, Multi-Column Antennas in One-Dimensional Compact Range Chamber, Kathrein

# Text proposal TR 37.842

10.3.1 Radiated transmit power

10.3.1.1 Test methods

##### 10.3.1.1.1 Compact Range Chamber

<Title to be updated once the test method is identified>

###### 10.3.1.1.1.1 Description

<Texts to be added>

###### 10.3.1.1.1.2 Procedure

<Texts to be added>

###### 10.3.1.1.1.3 Uncertainty budget format

<Texts to be added>

##### 10.3.1.1.2 One Dimensional Compact Range Chamber

<Title to be updated once the test method is identified>

###### 10.3.1.1.2.1 Description

The principle of the measurement set up is shown in Figure xx. It closely resembles the standard compact range chamber. But for the generation of the plane wave in the quiet zone the proposed method uses a special one dimensional probe instead of the standard set up comprising a concave mirror and a feed horn. The main advantage is a reduced chamber size and a simplified probe system replacing the standard mirror feed horn set up.

Figure xx: Proposed compact measurement set up for AAS: one dimensional compact range chamber

The probe antenna is set up of a one dimensional array of radiators, fed by a special feed system. Optimized amplitude and phase settings in the feed system result in a plane wave in vertical direction. Using a probe antenna of ~4 m height, a quiet zone of ~2.5 m height can be achieved at a distance of ~4 m. In the horizontal direction the far field Fraunhofer criterion is fulfilled for antenna widths less than ~ 0.6m (exact value depends on wavelength). As a result the quiet zone has an elliptical shape with cylindrical wave fronts that allow to directly measure EIRP, EIS and antenna patterns for standard active (and also passive) base station antennas. As no near field to far field transformation has to be carried out, it is not required to measure a whole set of azimuth and elevation angles.

As an example in Figure yy shows the measurement results for the new method compared to standard near field measurement results. The antenna pattern measured with this first implementation of the new probe antenna matches good to the standard antenna measurement data.

Figure yy: Results achieved with the proposed method

As usual the whole setup is placed into an anechoic chamber, preventing interference with e.g. operating mobile networks.

Regarding the measurement of multi column antennas, the cylindrical wave character occurring in the set up leads to amplitude and phase deviations at the different columns compared to plane wave incident. The maximum phase deviation occurs when measuring the AUT in broadside direction.

Figure zz: Phase deviation vs. array width D

According to the Fraunhofer far field criterion r≥2D2λ a maximum phase deviation of Δϕ=22.5° between center and edge of the array is tolerable. Applying this value, Figure zz shows that the maximum array width, which can be measured, lies between approx. two and four wavelength, depending on the frequency.

The measurement is limited to antennas with a maximum number of columns of:

Nmax≤1+2rλ 				(1)

Table zz below summarizes the results for a measurement distance of r = 4 m and typical operating frequencies. The values clearly show that the measurement of practical relevant multi column antennas is possible.

| Frequency | Max. number of columns | Array width D |
| --- | --- | --- |
| 700 MHz | 5 | 0.86 m |
| 1800 MHz | 7 | 0.5 m |
| 2600 MHz | 9 | 0.46 m |

Table zz: Maximum number of columns that can be measured with the one dimensional compact range and corresponding array width D

The number of columns that can be measured at an AUT-probe distance of r = 4 m increases with frequency:

- Based on the Fraunhofer far field criterion for λ\2 column spacing: Nmax≤1+2rλ

  - Nmax=5 columns @ 700 MHz

  - Nmax=7 columns @ 1800 MHz

  - Nmax=9 columns @ 2600 MHz

The only deviations compared to far field measurements are observed at the nulls of the antenna patterns. Therefore, EIRP and EIS can be determined with very high accuracy.

###### 10.3.1.1.2.2 Procedure

<Texts to be added>

###### 10.3.1.1.2.3 Uncertainty budget format

<Texts to be added>