3GPP TSG-RAN WG4 Meeting #78 	R4-161346

St Julian’s, Malta, 15 – 19 February 2016

Source:	DISH Network

Title:	TP for TR36.749v.0.0.0 Section 8.1.3.1 (UE RFSENS)

Work item:	LTE_AWS_3_4

Agenda item:	8.9

Document for:	Approval

## Introduction

This document provides a TP for TR36.749v.0.0.0 Section 8.1.3.1 (UE RFSENS). (pCR)

## Introduction

<Text to be added>

#### 2.1	UE REFSENS

The reference sensitivity is impacted by the duplex filter insertion loss in the Rx path since this directly increases the overall noise figure of the receiver.  .

The AWS-3/4 band plan would require to support two Tx to RX spacing, however due to the large duplex gap the performance differences between the two TX/RX carrier (295/300MHz) are expected to be insignificant. This means the minimum performance requirements for the band can remain consistent regardless of the TX/RX frequency or duplex spacing choice [3]. Changes to TS 36.101 are proposed in Table 8.1.2.1-1 and Table 8.1.3.1-2

Table 2.1-1: Reference sensitivity QPSK PREFSENS (Table 7.3.1-1 of TS 36.101)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 4 | -104,7 | -101,7 | -100 | -97 | -95,2 | -94 | FDD |
| [XX] | [ TBD] | [ TBD] | [ TBD] | [TBD ] | [TBD] |  | FDD |

NS_03 need to be signalled for REFSENS

Table 2.1-2: Network signalling value for reference sensitivity (Table 7.3.1-3 of TS 36.101)

| E-UTRA Band | Network Signalling value |
| --- | --- |
| Band [XX] | NS_03 |

Editor’s note: how to address the requirements for the DL-only part needs further consideration.

##### 2.1.1	Uplink configuration

Given the large 295/300 MHz Tx-Rx separation for Band [XX], the uplink configuration for reference sensitivity can be fully allocated. The proposed test configuration to take into account the Primary and Secondary TX-RX spacing would be the responsibility of RAN5. However, effort should be made to avoid duplication of all RX tests and any additional tests should be minimised in order to confirm the UE support the additional TX-RX spacing

For operating bands with an unpaired DL part, the requirements only apply for carriers assigned in the paired part.

## 3.0	References

[3]	3GPP TR 36.870: "Study on New AWS 3/4 Band for LTE".

## Proposal for TR36.749v0.0.0

---- Start of TP -------

# 8	Study of E-UTRA specific issues

## 8.1	UE specific (stand-alone band)

### 8.1.1	Filter data

### 8.1.2	UE TX requirements

### 8.1.3	UE RX requirements

#### 8.1.3.1	UE REFSENS

The reference sensitivity is impacted by the duplex filter insertion loss in the Rx path since this directly increases the overall noise figure of the receiver.

The AWS-3/4 band plan would require to support two TX to RX spacing, however due to the large duplex gap the performance differences between the two TX/RX carriers (295/300MHz) are expected to be insignificant. This means the minimum performance requirements for the band can remain consistent regardless of the TX/RX frequency or duplex spacing choice [3]. Changes to TS 36.101 are proposed in Table 8.1.2.1-1 and Table 8.1.3.1-2

Table 8.1.2.1-1: Reference sensitivity QPSK PREFSENS (Table 7.3.1-1 of TS 36.101)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 4 | -104,7 | -101,7 | -100 | -97 | -95,2 | -94 | FDD |
| [XX] | [ ] | [ ] | [ ] | [ ] | [ ] |  | FDD |

NS_03 need to be signalled for REFSENS

Table 8.1.3.1-2: Network signalling value for reference sensitivity (Table 7.3.1-3 of TS 36.101)

| E-UTRA Band | Network Signalling value |
| --- | --- |
| Band [XX] | NS_03 |

Editor’s note: how to address the requirements for the DL-only part needs further consideration.

##### 8.1.3.1.1	Uplink configuration

Given the large 295/300 MHz TX-RX separation for Band [XX], the uplink configuration for reference sensitivity can be fully allocated. The proposed test configuration to take into account the Primary and Secondary TX-RX spacing would be the responsibility of RAN5. However, effort should be made to avoid duplication of all RX tests and any additional tests should be minimised in order to confirm the UE support the additional TX-RX spacing

For operating bands with an unpaired DL part, the requirements only apply for carriers assigned in the paired part.