3GPP TSG-RAN WG4 Meeting #78	   					       R4-160011

St. Julian’s, Malta, 15 - 19 Feb, 2016

Source:	China Telecom

Title:	TP: Clarifications on the link level evaluation parameters

Agenda Item:	6.6.2

Document for:	Approval

Introduction

This contribution provides a text proposal for TR 36.884 to:

- Correct the AWGN power levels for different channel bandwidths, according to TS 36.141.

- Clarify some of the phase-II link level evaluation parameters, mainly on the parameters which are different from phase-I link evaluation, based on the previous discussion in [1]-[3].

Reference

[1]	R4-155202, Huawei, China Telecom, “Meeting minutes for BS MMSE-IRC ad hoc,” RAN4 #76, Aug 2015.

[2]	R4-156618, China Telecom, “Meeting minutes for BS MMSE-IRC receiver ad hoc,” RAN4 #76bis, Oct 2015.

[3]	R4-156866, ZTE, China Telecom, Huawei, Alcatel-Lucent, Nokia Networks, Samsung, “Way forward on phase-II test parameters for BS MMSE-IRC in synchronous network,” RAN4 #76bis, Oct 2015.

Text Proposal

<< Start of text proposal >>

# 7	Link performance characterization

## 7.1	Parameters for link level evaluation

The common parameters for link level evaluation are given in Table 7.1-1. The evaluation cases with specific parameters are given in Table 7.1-2. And the performance under both MMSE and MMSE-IRC receivers should be provided for comparison of the performance.

For link level simulation in Phase I, the simulation results of throughput versus SINR for both MMSE-IRC and MMSE receivers, and SINR gain of MMSE-IRC over MMSE at the 70% relative throughput will be provided. The ideal simulation results will be provided without the impairments.

Table 7.1-1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | 2 |
|  | AWGN power level for 10MHz channel bandwidth | dBm / 9MHz | -83.5 |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for synchronous case |  | Well aligned: no timing delay and frequency offset between the serving UE and interfering UEs |
| Frequency hopping, TTI bundling |  |  | Disable |

Table 7.1-2: Cases for link level evaluations

| Case Num | PRB allocation/ Band width | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- | --- |
| 1 | 50 PRB/10MHz | 6 | (EPA5, EPA5) | 1x2 Low | (-1.11, -10.91) |
| 2 | 50 PRB/10MHz | 6 | (EPA5, EPA5) | 1x2 Low | (-0.43, -13.78) |
| 3 | 50 PRB/10MHz | 15 | (EPA5, EPA5) | 1x4 Low | (-1.11, -10.91) |
| 4 | 50 PRB/10MHz | 15 | (EPA5, EPA5) | 1x4 Low | (-0.43, -13.78) |
| 5 | 50 PRB/10MHz | 20 | (EPA5, EPA5) | 1x8 Low | (-1.11, -10.91) |
| 6 | 50 PRB/10MHz | 20 | (EPA5, EPA5) | 1x8 Low | (-0.43, -13.78) |
| 7 | 50 PRB/10MHz | 6 | (EVA70, EVA70) | 1x2 Low | (-1.11, -10.91) |
| 8 | 50 PRB/10MHz | 6 | (EVA70, EVA70) | 1x2 Low | (-0.43, -13.78) |
| 9 | 50 PRB/10MHz | 15 | (EVA70, EVA70) | 1x4 Low | (-1.11, -10.91) |
| 10 | 50 PRB/10MHz | 15 | (EVA70, EVA70) | 1x4 Low | (-0.43, -13.78) |
| 11 | 50 PRB/10MHz | 20 | (EVA70, EVA70) | 1x8 Low | (-1.11, -10.91) |
| 12 | 50 PRB/10MHz | 20 | (EVA70, EVA70) | 1x8 Low | (-0.43, -13.78) |

<< Unchanged text omitted >>

8	BS demodulation performance requirements

For PUSCH, enhanced demodulation performance requirements will be defined for verification of BS receiver’s ability on inter-cell interference suppression.

- 8.1	Parameters for link level evaluation

For deriving the enhanced demodulation requirements (clause 8 in 36.104) in phase II, generally the phase-I link level evaluation parameters in clause 7.1 can be re-used, with some exceptions / clarifications as follows:

- For uplink 2Rx cases, when the explicit interferer number is increased from 1 to 2, the additional MMSE-IRC gain over MMSE is not significant, thus it was decided to model only 1 explicit interferer for 2Rx cases. 2 explicit interferers are modeled for 4Rx and 8Rx cases.

- To check the per-PRB interference covariance estimation, it was decided to use ETU channel with high delay spread for the interferers. For cases with EPA5 and EVA70 as serving channel, ETU5 and ETU70 are respectively used as the interfering channel.

- In clause 7, 12 evaluation cases are introduced for synchronous network. To limit the test efforts, the number of synchronous network test case is reduced to 6 for each channel bandwidth.

- Different DM-RS base sequences are configured for the serving UE and interfering UEs, and both group hopping and sequence hopping are disabled.

The common parameters for phase-II link level evaluation are given in Table 8.1-1. The phase-II evaluation cases with specific parameters are given in Table 8.1-2.

Table 8.1-1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | 1 or 2 |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for synchronous case |  | Well aligned: no timing delay and frequency offset between the serving UE and interfering UEs |
| Frequency hopping, TTI bundling |  |  | Disable |
| HARQ combining |  |  | Incremental redundancy |
| Redundancy version sequence |  |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| Maximal number of HARQ transmissions (including 1st transmission and re-transmissions) |  |  | 4 |
| Bandwidth |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |
| Resource allocation |  |  | Full PRB allocation |
| DM-RS configuration |  |  | The serving UE, interfering UE 1 and interfering UE 2 are served by cells with cell id #0, 1, 2 respectively. For the wanted PUSCH and interfering PUSCHs, =0, =0 and =0. |

Table 8.1-2: AWGN power level

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |
| 3 | -88.7dBm / 2.7MHz |
| 5 | -86.5dBm / 4.5MHz |
| 10 | -83.5dBm / 9MHz |
| 15 | -81.7dBm / 13.5MHz |
| 20 | -80.4dBm / 18MHz |

Table 8.1-3: Cases for link level evaluations

| Case Num | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- |
| 1 | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) |
| 2 | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) |
| 3 | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) |
| 4 | 6 | (EVA70, ETU70) | 1x2 Low | (-1.11, N/A) |
| 5 | 15 | (EVA70, ETU70) | 1x4 Low | (-1.11, -10.91) |
| 6 | 20 | (EVA70, ETU70) | 1x8 Low | (-1.11, -10.91) |

<< End of text proposal >>