3GPP TSG-RAN WG4 Meeting #78 	R4-160091

St Julian’s, Malta, 15 – 19 February 2016

Source:	DISH Network

Title:	TP for TR36.749v.0.0.0 Section 8.1.3.2 (UE RX Requirements for operating bands with an unpaired spectrum)

Work item:	LTE_AWS_3_4

Agenda item:	8.9.1

Document for:	Approval

## Introduction

This document provides a TP for TR36.749v.0.0.0 Section 8.1.3.2 (UE RX Requirements for operating bands with an unpaired spectrum). (pCR)

## Introduction

<Text to be added>

#### 2.1	RX Requirements for operating bands with an unpaired spectrum

Similar to B66 which is an operating band with an unpaired spectrum, the requirements for standalone (non-CA operation) would only apply for the paired part of the band [XX]. Therefore, the existing text in 3GPP TS36.101 in the following subclauses “will also will also be applicable for this new band [XX];

7.4.1		Minimum requirements [Maximum input level]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.5.1		Minimum requirements [Adjacent Channel Selectivity]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.1.1		Minimum requirements [In band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.2.1		Minimum requirements [Out of band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.3.1		Minimum requirements [Narrow band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.7.1		Minimum requirements [Spurious response]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.8.1.1		Minimum requirements [Wide band intermodulation]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

## 3.0	References

[3]	3GPP TR 36.870: "Study on New AWS 3/4 Band for LTE".

## Proposal for TR36.749v0.0.0

---- Start of TP -------

# 8	Study of E-UTRA specific issues

## 8.1	UE specific (stand-alone band)

### 8.1.1	Filter data

### 8.1.2	UE TX requirements

### 8.1.3	UE RX requirements

#### 8.1.3.1	UE REFSENS

#### 8.1.3.2	RX Requirements for operating bands with an unpaired spectrum

Similar to B66 which is an operating band with an unpaired spectrum, the requirements for standalone (non CA operation) would only apply for the paired part of the band [XX]. Therefore, the existing text in 3GPP TS36.101 in the following subclauses “will also will also be applicable for this new band [XX];

7.4.1		Minimum requirements [Maximum input level]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.5.1		Minimum requirements [Adjacent Channel Selectivity]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.1.1		Minimum requirements [In band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.2.1		Minimum requirements [Out of band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.6.3.1		Minimum requirements [Narrow band blocking]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.7.1		Minimum requirements [Spurious response]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….

7.8.1.1		Minimum requirements [Wide band intermodulation]

…..   For operating bands with an unpaired DL part as noted in Table 5.1-1 (E-UTRA operating bands), the requirements only apply for carriers assigned in the paired part….