3GPP TSG RAN WG4 Meeting #78	R4- 160273

Santa Rosa, USA, 8-12 October 2012

Agenda Item:	6.7.4.2

Source: 	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Title: 	RRM Test Cases for Cat-M UEs

Document for:	Discussion

# Introduction

The RRM requirements related to the Release 13 work item, “Further LTE Physical Layer Enhancements for MTC” [1] are being completed. A list of Cat-M RRM requirements were endorsed in R4-77AH-IoT meeting [2-7]. In this paper, we discuss the RRM test cases needed for verifying the RRM requirements for Cat-M UEs supporting Cat-M UEs with a list of test cases proposed to be introduced in release 13. A time plan for the completion of this work is also proposed in this paper.

Note: There are still some open issues related to Cat-M RRM. However, it should not prevent RAN4 from working the tests in parallel. There is not necessary to wait for the solution of all open issues to start RRM test cases.

# Cat-M RRM Test Cases

Cat-M RRM performance requirements are being introduced in TS 36.133 Rel-13 based on the endorsed CRs [2-7]. In Rel-13, Cat-M RRM performance requirements do not include RSRQ, inter-frequency measurements, positioning measurements, and Cat-M UEs do not support new features such as carrier aggregation, dual connectivity, etc. On the other hand, Cat-M UEs may operate in normal coverage enhanced coverage in idle state, and configured as CEModeA or CEMode B in connected state. The Cat-M RRM performance requirements are defined for two scenarios in idle and connected states. Thus, for Cat-M RRM test cases, both scenarios need to be considered in idle and connected states

In Table 1, we provide the list of RRM test cases that should be considered.

Table 1: The List of RRM Test Cases for Cat-M UEs (FD-FDD, HD-FDD, TDD)

| Section | Title | Comments |
| --- | --- | --- |
| A.4.2 | Cell Re-Selection |  |
|  | Intra frequency cases | Normal coverage: Existing intra-frequency measurement requirements in section 4.2.2 still apply. Current test cases may be reused. Enhanced coverage: New test cases may need to be developed for the new requirements. |
| A.6 | RRC Connection Control |  |
|  | Intra-frequency RRC Re-establishment | The Cat-M UEs are expected to meet the exiting performance requirements. New test cases need to be defined due to the change of the PRACH channel New test cases for both CEModeA and CEModeB. |
| A.6.2 | Random Access |  |
|  | Contention Based Random Access Test | Cat-M random access procedure is not the same as legacy UEs. The changes include the PRACH resource selection based on RSRP measurements and the repetition of the preamble transmission. New test cases for both CEModeA and CEModeB. |
| A.7.1 | UE Transmit Timing |  |
|  | – UE Transmit Timing Accuracy Tests | The Cat-M UEs are expected to meet the exiting performance requirements. The parameters in the current test cases are not suitable due to the changes of the control channel New test cases for both CEModeA and CEModeB. |
| A.7.2 | UE Timing Advance |  |
|  | UE Timing Advance Adjustment Accuracy Test | The Cat-M UEs are expected to meet the exiting performance requirements. The parameters in the current test cases are not suitable due to the changes of the control channel New test cases for both CEModeA and CEModeB. |
| A.7.3 | Radio Link Monitoring |  |
|  | Radio Link Monitoring Test for Out-of-sync | New test cases for both CEModeA and CEModeB due to new performance requirements |
|  | Radio Link Monitoring Test for In-sync | New test cases for both CEModeA and CEModeB due to new performance requirements |
|  | Radio Link Monitoring Test for Out-of-sync in DRX | New test cases for both CEModeA and CEModeB due to new performance requirements |
|  | Radio Link Monitoring Test for In-sync in DRX | New test cases for both CEModeA and CEModeB due to new performance requirements |
| A.8.1 | E-UTRAN FDD Intra-frequency Measurements |  |
|  | intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells | New test cases for both CEModeA and CEModeB due to new performance requirements |
|  | intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with/without DRX | New test cases for both CEModeA and CEModeB due to new performance requirements |
| A.9.1 | RSRP Accuracy |  |
|  | FDD/TDD RSRP Intra frequency case | New test cases for both CEModeA and CEModeB due to new performance requirements |

# Time Plan for Cat-M RRM Tests

In terms of time plan for the tests, we suggest to have two phases. The first phase focuses on test cases for normal coverage/CEModeA, and the second phase focuses on extended coverage/ CEModeB.

The proposed timeline is given below:

- RAN4#78 (Feb 2016)

  - Agree the Cat-M RRM Test List and

- RAN4#78-bis (April 2016)

  - Agree the CRs for normal coverage/CEModeA

- RAN4#79 (May 2016)

  - Agree the CRs for extended coverage/CEModeB

# Conclusions

In this paper we discussed Cat-M RRM test cases, and provided a list of test cases to be developed for verifying the RRM performances of Cat-M UEs. It was proposed to have two phases in the development of the Cat-M RRM test cases with the first phase focusing on test cases for normal coverage/CEModeA, and the second phase on extended coverage/CEModeB.

# References

- RP-141660,	New WI proposal: further LTE physical layer enhancements for MTC, Ericsson, Nokia Networks

- R4-77AH-IoT-0124, RRM requirements for eMTC in IDLE mode in section 4, Nokia Networks, Alcatel Lucent, Ericsson, Huawel, HiSilicon, Intel, Verizon

- R4-77AH-IoT-0123, Measurement requirements for Rel-13 MTC UE under normal coverage, Ericsson, Alcatel-Lucent, Intel, CATT, Huawei, HiSilicon, Verizon

- R4-77AH-IoT-0149, Measurement requirements for Rel-13 MTC UE under enhanced coverage, Ericsson, Alcatel-Lucent, Huawei, HiSilicon CATT, Intel Corporation, Verizon

- R4-77AH-IoT-0126, Measurement accuracy requirements for Rel-13 MTC UE, Ericsson, Alcatel-Lucent, Huawei, HiSilicon, Intel Corporation, CATT, Verizon

- R4-77AH-IoT-0127, Radio link monitoring for Rel-13 MTC UE, Ericsson, Intel Corporation, Alcatel-Lucent, Huawei, HiSilicon, CATT, Verizon

- R4-77AH-IoT-0129, Reference configuration for Rel-13 MTC, Ericsson, Intel Corporation, Alcatel-Lucent, CATT, Huawei, HiSilicon, Verizon, Nokia Networks, NTTDOCOMO