3GPP TSG-RAN WG4 #78	R4-160368

Malta, Malta, 15th Feb– 20 Feb 2015

Source: 	Ericsson

Title:	General test setup and requirements applicability for LAA demodulation

Agenda Item:	6.13.4

Document for:	Discussion

# Introduction

In this paper, the general setup for LAA demodulation is discussed, and the requirements applicability is also discussed.

# System setup for LAA demodulation

## Bandwidth combination

For the bandwidth combination, in RF, the following combinations have been approved for LAA in RAN4#77 meeting:

- Some CA combinations including B46 up to 2CC (e.g. CA_1A-46A) [1]

- CA_1A-46C [2]

- CA_1A-46D [3]

- CA_1A-46E [4]

- CA_3A-46C [5]

- CA_3A-46D [6]

- CA_3A-46E [7]

Based on this approved combination, we can see that up to 4 LAA Scells can be configured in Rel-13 and one primary cell can be configured. Hence, in demodulation, the performance requirements shall cover up to 4 LAA Scells in addition to one primary cell.

Proposal  1: The performance requirements shall cover up to 4 LAA Scells and one primary cell

## CRS structure configuration

For CRS structure configuration, in RAN1 and RAN2, they define new RRC signaling indicates which subframe has one or two symbol CRS structure or all symbols CRS structure. For one or two symbol CRS structure, there are only one or two CRS symbols will be transmitted in the available subframe. For all symbols CRS structure, all OFDM symbols with CRS will be transmitted.

For CRS-based transmission, all symbols CRS structure is the pre-condition. For DMRS-based transmission, it is not necessary to configure all symbols CRS structure. With one or two CRS structure in LAA, we can increase the throughput and reduce the interference in practical system. Hence, we have the following proposal:

Proposal  2: One or two CRS symbols structure is configured for DMRS-based transmission mode and all CRS symbols structure is configured for CRS-based transmission mode

For PDCCH, since CRS is needed for its decoding. But for ePDCCH, the CRS is not a must. Hence, for control channel, we can have the following configuration:

Proposal  3: One or two CRS symbols structure is configured for ePDCCH performance and all CRS symbols structure is configured for PDCCH performance

## Antenna configuration

For antenna configuration, 2x2 can be set as a basic setup. It can be used for both (e)PDCCH demodulation and PDSCH demodulation.  For 4x2 case, more and more operators want to have. It is better to have at least one test case to cover 4x2 cases as well. In order to reduce the test number, one possible way to cover 4x2 case is to introduce 4x2 case for one PDSCH transmission mode, for example TM4 case.

Proposal  4: 2x2 can be set as the basic setup for all demodulation and CSI, and it is better to have one PDSCH test case to cover 4x2 case (i.e., TM4).

## Transmission mode

According to RAN1 agreements in [8], they have the following agreements:

- TM1, TM2, TM3, TM4, TM8, TM9, and TM10 are supported on LAA SCell from RAN1 specification point of views

    - TM10 supports at least QCL type A

        - FFS: QCL type B

    - PQI bits are still present in DCI format 2D

    - Fallback transmission schemes are supported

For RAN4, one option is cover all the transmission modes; another option is prioritize some transmission modes. Since CRS-based transmission and DMRS-based transmission mode have fundamental different demodulation procedure. Hence, both CRS-based transmission and DMRS-based transmission scheme shall be covered in LAA demodulation. Further, for CRS-based transmission scheme, as a minimum set, it is better to cover both close loop MIMO transmission scheme and open loop scheme. For close loop MIMO transmission scheme, we can consider TM4 as the baseline. For open loop MIMO transmission scheme, we can consider TM3.

Proposal  5: Both CRS-based transmission scheme(s) and DMRS-based transmission scheme shall be covered in LAA demodulation.

Proposal  6: For CRS-based transmission schemes, TM3 is configured for 2x2 case and TM4 is configured for 4x2 case; For DMRS-based transmission scheme, TM9 is configured for 2x2.

## DRS configuration

Since UE may use DRS for measurements and synchronization, DRS is configured. The periodicity can be 40 ms.

## Reference receiver

For receiver, we have MMSE-MRC, MMSE-IRC and other advanced receiver, such as receiver with CRS-IM capability, NAIC, receiver with control channel IC. As minimum performance requirements, we can use non-advanced receiver. Since MMSE-IRC is used widely, we prefer to use MMSE-IRC as the reference receiver.

Proposal  7: MMSE-IRC receiver is set as the reference receiver in LAA demodulation

## Interference modelling

In LAA, the interference is not only from 3GPP system, it also comes from WiFi system. For legacy MMSE-IRC, the interference is assumed stable, and in all test, the aggressor cell are LTE system. The WiFi interference characteristic is different from LTE interference characteristic. It may impact on the UE performance. Hence, it is better to introduce WiFi interference in the test.

# Applicability of the requirements to different CA combinations

For LAA demodulation requirements, it is very similar to traditional CA performance requirements. Hence, we can reuse the basic general principle for traditional CA performance requirements. In CA performance requirements, we have the following general principle

- The demodulation performance requirements are band agnostic and band combination agnostic

- Single carrier performance for multiple CA configurations is the pool with all single carrier perf for all bandwidths

For LAA demodulation, the only difference is two performance pools may be configured, one pool is for license carrier and one pool is for unlicensed carrier.

One example is shown in Table-X and Table-Y. In the table X, the performance for license carrier (primary carrier) is given, and in the Table-Y, the performance for unlicensed carrier (unlicensed secondary carrier) is given.

Table-X: Single carrier performance (Frame type 1 and Frame type 2) for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [-1.7] |
| … | … | … | … | … | … | … |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [-1.7] |

Table-Y: Single carrier performance (Frame type 3) for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 20MHz | R.X FDD | OP.X FDD | EVA5 | 1x2 Low | 70 | X |

Table-Z: Performance requirements for different CA combinations

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 10MHz + 20MHz | Requirements for Frame type 1 or 2 is as specified in Table-X per CC Requirements for Frame type 3 is as specified in Table Table-Y per CC | ≥5 |

In Table-Z, the performance for different CA combination is given. If the component carrier is license carrier, the performance requirements of Table-X will be applied. If the component carrier is unlicensed carrier, the performance requirements of Table-Y will be applied.

For the performance pool of license carrier, we can reuse what we have defined in 36.101. For the performance pool of unlicensed carrier, new performance requirements will be defined.

Proposal  8: The performance requirements are band agnostic and band combination agnostic. Two performance pools will be defined for LAA, one pool is for license carrier, and one pool is for unlicensed carrier. The pool for license carrier can be reused

# Test metric

In legacy demodulation test, the SNR at the 70% of the peak throughput is used for the metric definition. In legacy test, the peak throughput is predicted. However, in unlicensed carrier, the control or data channel transmissions are subjected to LBT. Hence, the peak throughput is unpredicted. Hence, this metric cannot be reused. We can replace this metric with the SNR at the 70% of the user throughput. The user throughput is defined as:

User throughput  = amount of datatime needed for the amount of data

For the control channel, the BLER metric can be reused.

Proposal  9：Update the test metric for PDSCH with user throughput rather than peak throughput, and reuse the BLER metric for the control channel.

# Conclusion

In this paper, we give our view on general test setup and also discuss the requirements applicability for LAA demodulation. We have the following proposals:

Proposal  1: The performance requirements shall cover up to 4 LAA Scells and one primary cell

Proposal  2: One or two CRS symbols structure is configured for DMRS-based transmission mode and all CRS symbols structure is configured for CRS-based transmission mode

Proposal  3: One or two CRS symbols structure is configured for ePDCCH performance and all CRS symbols structure is configured for PDCCH performance

Proposal  4: 2x2 can be set as the basic setup for all demodulation and CSI, and it is better to have one PDSCH test case to cover 4x2 case (i.e., TM4).

Proposal  5: Both CRS-based transmission scheme(s) and DMRS-based transmission scheme shall be covered in LAA demodulation.

Proposal 6: For CRS-based transmission schemes, TM3 is configured for 2x2 case and TM4 is configured for 4x2 case; For DMRS-based transmission scheme, TM9 is configured for 2x2.

Proposal 7: MMSE-IRC receiver is set as the reference receiver in LAA demodulation

Proposal 8: The performance requirements are band agnostic and band combination agnostic. Two performance pools will be defined for LAA, one pool is for license carrier, and one pool is for unlicensed carrier. The pool for license carrier can be reused

Proposal 9: Update the test metric for PDSCH with user throughput rather than peak throughput, and reuse the BLER metric for the control channel.

# Reference

- R4-158443, Introduction of RF requirements for LAA operation, Ericsson, Nov. 2015

- R4-158270, TP for TR 36.853-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_C), NTT DOCOMO, INC. Nov, 2015

- R4-158271, TP for TR 36.854-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_D), NTT DOCOMO, INC. Nov, 2015

- R4-158272, TP for TR 36.857-13: LTE Advanced Carrier Aggregation of Band Combination (1A+LAA_E), NTT DOCOMO, INC. Nov, 2015

- R4-158412, TP for TR 36.853-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_C), NTT DOCOMO, INC. Nov, 2015

- R4-158267, TP for TR 36.854-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_D), NTT DOCOMO, INC. Nov, 2015

- R4-158268, TP for TR 36.857-13: LTE Advanced Carrier Aggregation of Band Combination (3A+LAA_E), NTT DOCOMO, INC. Nov, 2015

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #83, 15th-22th Nov. 2015.