3GPP TSG-RAN 3GPPRAN4-#78	R4-161139

St. Julian’s, Malta, 15 – 19 February, 2016

Source: 	Nokia Networks, Alcatel-Lucent, Ericsson

Title: 	Updated simulation assumptions for RLM under normal and enhanced coverage

Agenda Item:	3.2

Document for:	Approval

# Introduction

RLM simulation assumptions for Rel-13 eMTC UEs were agreed in RAN4#77 [1]. At that time, some of the M-PDDCH parameters, especially DCI Formats and contents were not finalized. In this document, we propose some updates of the RLM simulation assumptions based on RAN1 agreed CRs [2, 3, 4].

# Link Simulation Assumptions

In [1], two tables are used to present the assumptions for in-sync and out-of-sync RLM simulation assumption. The main difference of the two tables is the DCI formats: DCI format for M-PDCCH order for PRACH was assumed for in-sync simulation while DCI formats M1A/M1B (now named as DCI format 6-1A/6-1B in [2,3,4]) for PUSCH were assumed for out-of-sync simulation. The reason behind the difference was that it was assumed DCI formats for M-PDCCH order and DCI formats M1A/M1B might have different payload sizes.

In [2], RAN1 agreed now that the same payload size will be used for DCI formats for M-PDCCH order and DCI formats M1A/M1B to make UE implementation simpler. Thus, there is no need to have sepearte simulation for the in-sync and out-of-sync RLM. Therefore, we suggest combine the two tables into one for clarity.

For RLM, we may need to simulate the maximum coverage enhancement with largest repetition level and aggregation level. The largest repetition level (R) is now defined as 256 instead of 128 [3]. Therefore, we will need to include R=256 in simulation.

In addition, an FFS parameter M-PDCCH starting subframe periodicity was included in [1]. This parameter, however, should have no impact on M-PDCCH performance investigation, and thus we suggest removing this FFS parameter from the simulation assumption to avoid confusion.

Furthermore, the number of PRB pairs for M-PDCCH was described as “depending on aggregation level” in the previous simulation assumption [1]. It would be better to clarify the dependency of the number of PRB pairs with the aggregation levels based on TS 36.213.

Based on above discussion, we suggest revising simulation assumptions as shown in the following table.

Table 1: Proposal for M-PDCCH transmission parameters for in-sync (IS) and out-of-sync (OOS)

| Parameter | Value for CE Mode A | Value for CE Mode B |
| --- | --- | --- |
| DCI format | DCI format 6-1A | DCI format 6-1B . |
| System Bandwidth | 10 MHz | 10 MHz |
| Channel model | AWGN ETU30 EPA5 | AWGN ETU1 EPA1 |
| Antenna configuration: 2 cases | 1x1 and 2x1 | 1x1 and 2x1 |
| Number of information bits | FDD: 28 TDD: 31 | FDD: 18 TDD: 18 |
| Antenna correlation | Low | Low |
| Repetition level | {2, 4} | {64, 128, 256} |
| Aggregation level (ECCE) | {4 , 8, 16} | {8, 16, 24} |
| Starting OFDM symbols | 2 | 2 |
| DRX | OFF | OFF |
| Frequency hopping | OFF | OFF |
| Number of PRB pairs for M-PDCCH | 2 for Aggregation level ={4,8} 4 for Aggregation level ={4,8,16} (see Table 9.1.5-1a in TS 36.213) | 4 for Aggregation level ={8,16} 2+4 for Aggregation level =24 (see Table 9.1.5-2a and Table 9.1.5-2b in TS 36.213) |
| Transmission type configured to UE for M-PDCCH | Distributed | Distributed |
| DMRS scrambling sequence initialisation parameter for UE-SS | PCID=1 | PCID=1 |
| UE timing error | [0] us | [0] us |
| UE frequency error | [50] Hz | [100] Hz |

Note: other aggregation levels and repetitions are not precluded.

# Performance Metric

The aim of the simulation is to identify whether the following RLM parameters need to be changed and if so then to what extent:

- For IS: aggregation level (CCE) and Repetition level (R), SNR levels for Qin/Qout

- For OOS: aggregation level (CCE), and Repetition level (R), SNR levels for Qin/Qout

Note: Companies are also encouraged to discuss the RLM requirements including L1 in-sync and out-of-sync evaluation periods based on the simulation evaluation results.

# References

- R4-158163, “Simulation assumptions for RLM under normal and enahnced coverage”, Ericsson, Alcatel-Lucent, Nokia Networks

- R1-157916, “Introduction of Rel-13 features of eMTC in 36.212”, Huawei

- R1-157926, “Introduction of further LTE Physical Layer Enhancements for MTC”, Motorola Mobility

- R1-157918, “Introduction of LC/CE MTC”, Ericsson