3GPP TSG-RAN WG4 Meeting #78BIS	R4-161634

St. San Jose del Cabo, Mexico, 11th – 15th April, 2016

Agenda Item:	7.12.3

Source: 	Intel Corporation

Title: 	Discussion on CSI reporting in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. In [2], it suggests that new CSI requirements should be specified and considered for the final high speed solutions to enhance the downlink demodulation performance, if needed. In this contribution, we provide our analysis on the CSI requirements, particularly on the CQI reporting issue, for high speed scenarios.

- Channel correlation coefficient w.r.t. Δt

Given that a high speed train (HST) is traveling at speed of 350km/h and the carrier frequency is 2.69GHz, the maximum Doppler shift can be up to fd,max = 872Hz. Generally, the channel coherence time is inversely related to the maximum Doppler shift, approximately expressed as 1/fd,max. Therefore, the coherence time for a HST is approximately 1ms. However, the CQI delay requires 8ms or more as specified in TS 36.101 Section 9.3 [3]. Fig.1 compares the “2-path” HST SFN channel [4] correlation coefficients w.r.t. Δt (i.e. channel correlation between time instant t and t+Δt) for fd,max = 5Hz and 872Hz, respectively.

a. fd,max = 5Hz                                                                                                 b. fd,max = 872Hz

Fig.1 Comparison of Correlation Coefficient w.r.t. ΔT for maximum Doppler shift fd,max = 5Hz and 872Hz

Fig.1 shows that, if the maximum Doppler shift is small (e.g. 5Hz), the channel coherence coefficient approximately keeps constant within a relatively long time period, such as 10ms. However, if the maximum Doppler shift is large (e.g. 872Hz), both the magnitude and angle of the coherence coefficient fluctuate rapidly in a considerably large range.

- Observation: CQI delay requirements of 8ms and more cannot be suitable for the HST scenario under current consideration.

- Conclusion

In this contribution, we provide our analysis on the CSI requirements, particularly on the CQI reporting issue, for high speed scenarios.

- Observation: CQI delay requirements of 8ms and more cannot be suitable for the HST scenario under current consideration.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO INC., Huawei, HiSilicon, Dec. 2015.

[2] RP-161252, “Way forward on UE performance enhancement under SFN channel”, Huawei, HiSilicon, NTT DoCoMo, Ericsson, Qualcomm, Intel, February 2016.

[3] TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”, 3GPP LTE, Rel. 12, May, 2015.

[4] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.