3GPP TSG-RAN WG4 Meeting #78Bis 	R4-162093

San Jose Del Cabo, Mexico, 11 – 15 April 2016

Source:	DISH Network

Title:	TS 36.101 Section 5 operating bands (clean up)

Work item:	TE12

Agenda item:	4.2.1

Document for:	Discussion

## Introduction

This is a discussion document to “clean up” section 5 (Operating bands and channel arrangement) for release 14.

## Background

Sub-clause 5.5 is the first introduction of operating bands in TS36.101 and provides the UL/DL frequency band information;

## Some observations of this sub-clause which covers 13 pages:

- High level reference

  - All the band specific requirements are defined in terms of operating band in the associated sub-clauses.

  - The requirements in this sections are used in other specifications (BS) and other working groups (R5) The information from this section is used in many outside forums i.e. ITU and regulatory bodies

- Duplication of information in sub-clause 5.5

  - The UL and DL frequency ranges is initially specified in sub-clause 5.5 and this information is repeated unnecessarily in sub-clause 5.5A (operating bands for CA) and 5.5B (operating bands for Dual Connectivity).

  - In the case of sub-clause 5.5B ( operating bands for UL-MIMO) duplication is avoided by referring to sub-clause 5.5

  - In the case of 5.5D (Operating bands for ProSe) the duplex direction is different and hence it is important to indicate the UL/DL information

The level of duplication (highlighted in yellow) is shown below in the case of inter-band CA operating bands (up to three CA bands) for Band 1 only!  As more bands are added i.e. standalone bands, CA bands, CA + unlicensed spectrum, MTC and NB-IoT bands this level of UL/DL frequency range duplication adds the possibility of errors, complexity of tabular information and should be avoided when necessary.

## Proposal

A way forward has been proposed to improve the specification clarity and remove some duplication. To address this there are two proposals for consideration

#### 3.1 	Option 1

To consider CA only and in this case remove some of the CA tables and add a note Details of CA band combinations and channel bandwidth are specified in subclause 5.6A.1.

Observations

- This proposal to refer to subclause 5.6A.1 only helps to address the inter band CA case and does not address intra-band (since there is no linkage to the corresponding E-UTRA band in this Table 5.6A-1 .For example in table below the E-UTRA band number is included (yellow highlight) for inter-band but not in the intra-band table.

.

So this means some E-UTRA band information would be specified in subclause 5.5A and others in 5.6A.1

- This proposal does not consider other features like dual connectivity, ProSe and NB-IoT so this would be inconsistent in terms of specification methodology

Summary: This proposal, although having merit, may cause confusion since this creates a separate path for inter-band CA compared with intra-band and other features like dual connectivity are not addressed. Hence, this may reduce the clarity of the specification, particularly to outside R4 delegates

#### 3.2 	Option 2

This would retain the existing approach of indicating the specified CA combination (like the BS spec) but remove duplication of the UL/DL operating band information so as to compress the table as shown below for the intra-band, inter-band CA (3 band) and Dual connectivity (3 bands).

Observations

- This reduces the number of rows by removing the duplicate information i.e. the frequency range information which is already specified in Table 5.5 but indicates the linkage of CA bands to E-UTRA bands

- This proposal would also introduce a common approach for intra-band CA, inter-band CA, Dual connectivity, ProSe and new features like NB-IoT bands

- There is no impact on other specifications since the Table numbering and band numbering is retained

Summary: The main advantage in maintaining the existing structure references to other specification, forums, ITU and directly indicate which CA band combinations are agreed in the specification.

## Conclusion

Considering the objective of the way forward which is to clean up the UE specification, it is proposed that option 2 is agreed as this removes duplication of information but more importantly retains the current structure of the specification relating to section 5 (Operating bands). A clean version of the section 5 is provided in Annex A which can be used as part of the “way forward” CR

## Annex A (Proposed draft CR)

## This would retain the existing approach of indicating the specified CA combination but remove duplication of the UL/DL operating band information

## --------------------- SECTION 5 --------------------------

5.5 		Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive / UE transmit |  |  | Downlink (DL) operating band BS transmit / UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 61 | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD2 |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD2 |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD8,9 |
| … |  |  |  |  |  |  |  |
| 64 | Reserved |  |  |  |  |  |  |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD4 |
| 67 | N/A |  |  | 738 MHz | – | 758 MHz | FDD2 |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |
| NOTE 1: Band 6 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. NOTE 8: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 9: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. |  |  |  |  |  |  |  |

## 5.5A Operating bands for CA

E-UTRA carrier aggregation is designed to operate in the operating bands defined in Tables 5.5A-1, 5.5A-2, 5.5A-2a and 5.5A-3.

Table 5.5A-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_1 | 1 |
| CA_2 | 2 |
| CA_3 | 3 |
| CA_5 | 5 |
| CA_7 | 7 |
| CA_8 | 8 |
| CA_12 | 12 |
| CA_23 | 23 |
| CA_27 | 27 |
| CA_38 | 38 |
| CA_39 | 39 |
| CA_40 | 40 |
| CA_41 | 41 |
| CA_42 | 42 |
| CA_66 | 66 |

Table 5.5A-2: Inter-band CA operating bands (two bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3 | 1 | 3 |  |  |  |
| CA_1-3-3 | 1 | 3 | 3 |  |  |
| CA_1-5 | 1 | 5 |  |  |  |
| CA_1-7 | 1 | 7 |  |  |  |
| CA_1-8 | 1 | 8 |  |  |  |
| CA_1-11 | 1 | 11 |  |  |  |
| CA_1-18 | 1 | 18 |  |  |  |
| CA_1-19 | 1 | 19 |  |  |  |
| CA_1-20 | 1 | 20 |  |  |  |
| CA_1-21 | 1 | 21 |  |  |  |
| CA_1-26 | 1 | 26 |  |  |  |
| CA_1-28 | 1 | 28 |  |  |  |
| CA_1-40 | 1 | 40 |  |  |  |
| CA_1-41 | 1 | 41 |  |  |  |
| CA_1-42 | 1 | 42 |  |  |  |
| CA_1-46 | 1 | 46 |  |  |  |
| CA_2-4 | 2 | 4 |  |  |  |
| CA_2-4-4 | 2 | 4 | 4 |  |  |
| CA_2-5 | 2 | 5 |  |  |  |
| CA_2-2-5 | 2 | 2 | 5 |  |  |
| CA_2-7 | 2 | 7 |  |  |  |
| CA_2-12 | 2 | 12 |  |  |  |
| CA_2-2-12 | 2 | 2 | 12 |  |  |
| CA_2-13 | 2 | 13 |  |  |  |
| CA_2-2-13 | 2 | 2 | 13 |  |  |
| CA_2-17 | 2 | 17 |  |  |  |
| CA_2-28 | 2 | 28 |  |  |  |
| CA_2-29 | 2 | 29 |  |  |  |
| CA_2-30 | 2 | 30 |  |  |  |
| CA_2-46 | 2 | 46 |  |  |  |
| CA_3-5 | 3 | 5 |  |  |  |
| CA_3-3-5 | 3 | 3 | 5 |  |  |
| CA_3-7 | 3 | 7 |  |  |  |
| CA_3-8 | 3 | 8 |  |  |  |
| CA_3-3-8 | 3 | 3 | 8 |  |  |
| CA_3-19 | 3 | 19 |  |  |  |
| CA_3-20 | 3 | 20 |  |  |  |
| CA_3-26 | 3 | 26 |  |  |  |
| CA_3-27 | 3 | 27 |  |  |  |
| CA_3-28 | 3 | 28 |  |  |  |
| CA_3-31 | 3 | 31 |  |  |  |
| CA_3-38 | 3 | 38 |  |  |  |
| CA_3-40 | 3 | 40 |  |  |  |
| CA_3-41 | 3 | 41 |  |  |  |
| CA_3-42 | 3 | 42 |  |  |  |
| CA_3-46 | 3 | 46 |  |  |  |
| CA_4-5 | 4 | 5 |  |  |  |
| CA_4-4-5 | 4 | 4 | 5 |  |  |
| CA_4-7 | 4 | 7 |  |  |  |
| CA_4-4-7 | 4 | 4 | 7 |  |  |
| CA_4-12 | 4 | 12 |  |  |  |
| CA_4-4-12 | 4 | 4 | 12 |  |  |
| CA_4-13 | 4 | 13 |  |  |  |
| CA_4-4-13 | 4 | 4 | 13 |  |  |
| CA_4-17 | 4 | 17 |  |  |  |
| CA_4-27 | 4 | 27 |  |  |  |
| CA_4-28 | 4 | 28 |  |  |  |
| CA_4-29 | 4 | 29 |  |  |  |
| CA_4-4-29 | 4 | 4 | 29 |  |  |
| CA_4-30 | 4 | 30 |  |  |  |
| CA_4-4-30 | 4 | 4 | 30 |  |  |
| CA_4-46 | 4 | 46 |  |  |  |
| CA_5-7 | 5 | 7 |  |  |  |
| CA_5-12 | 5 | 12 |  |  |  |
| CA_5-13 | 5 | 13 |  |  |  |
| CA_5-17 | 5 | 17 |  |  |  |
| CA_5-25 | 5 | 25 |  |  |  |
| CA_5-29 | 5 | 29 |  |  |  |
| CA_5-30 | 5 | 30 |  |  |  |
| CA_5-38 | 5 | 38 |  |  |  |
| CA_5-40 | 5 | 40 |  |  |  |
| CA_7-8 | 7 | 8 |  |  |  |
| CA_7-12 | 7 | 12 |  |  |  |
| CA_7-20 | 7 | 20 |  |  |  |
| CA_7-22 | 7 | 22 |  |  |  |
| CA_7-28 | 7 | 28 |  |  |  |
| CA_7-40 | 7 | 40 |  |  |  |
| CA_7-42 | 7 | 42 |  |  |  |
| CA_7-42-42 | 7 | 42 | 42 |  |  |
| CA_7-46 | 7 | 46 |  |  |  |
| CA_8-11 | 8 | 11 |  |  |  |
| CA_8-20 | 8 | 20 |  |  |  |
| CA_8-40 | 8 | 40 |  |  |  |
| CA_8-41 | 8 | 41 |  |  |  |
| CA_8-42 | 8 | 42 |  |  |  |
| CA_8-42-42 | 8 | 42 | 42 |  |  |
| CA_11-18 | 11 | 18 |  |  |  |
| CA_12-25 | 12 | 25 |  |  |  |
| CA_12-30 | 12 | 30 |  |  |  |
| CA_18-28 | 18 | 28 |  |  |  |
| CA_19-21 | 19 | 21 |  |  |  |
| CA_19-28 | 19 | 28 |  |  |  |
| CA_19-42 | 19 | 42 |  |  |  |
| CA_20-31 | 20 | 31 |  |  |  |
| CA_20-32 | 20 | 32 |  |  |  |
| CA_20-38 | 20 | 38 |  |  |  |
| CA_20-40 | 20 | 40 |  |  |  |
| CA_20-42 | 20 | 42 |  |  |  |
| CA_20-42-42 | 20 | 42 | 42 |  |  |
| CA_20-67 | 20 | 67 |  |  |  |
| CA_21-42 | 21 | 42 |  |  |  |
| CA_23-29 | 23 | 29 |  |  |  |
| CA_25-26 | 25 | 26 |  |  |  |
| CA_25-41 | 25 | 41 |  |  |  |
| CA_26-41 | 26 | 41 |  |  |  |
| CA_28-40 | 28 | 40 |  |  |  |
| CA_28-41 | 28 | 41 |  |  |  |
| CA_28-42 | 28 | 42 |  |  |  |
| CA_29-30 | 29 | 30 |  |  |  |
| CA_38-40 | 38 | 40 |  |  |  |
| CA_39-41 | 39 | 41 |  |  |  |
| CA_41-42 | 41 | 42 |  |  |  |
| CA_41-46 | 41 | 46 |  |  |  |
| CA_42-46 | 42 | 46 |  |  |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination. |  |  |  |  |  |

Table 5.5A-2a: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3-5 | 1 | 3 | 5 |  |  |
| CA_1-3-7 | 1 | 3 | 7 |  |  |
| CA_1-3-8 | 1 | 3 | 8 |  |  |
| CA_1-3-19 | 1 | 3 | 19 |  |  |
| CA_1-3-20 | 1 | 3 | 20 |  |  |
| CA_1-3-26 | 1 | 3 | 26 |  |  |
| CA_1-3-28 | 1 | 3 | 28 |  |  |
| CA_1-3-40 | 1 | 3 | 40 |  |  |
| CA_1-3-42 | 1 | 3 | 42 |  |  |
| CA_1-5-7 | 1 | 5 | 7 |  |  |
| CA_1-5-40 | 1 | 5 | 40 |  |  |
| CA_1-7-8 | 1 | 7 | 8 |  |  |
| CA_1-7-20 | 1 | 7 | 20 |  |  |
| CA_1-7-28 | 1 | 7 | 28 |  |  |
| CA_1-8-11 | 1 | 8 | 11 |  |  |
| CA_1-8-40 | 1 | 8 | 40 |  |  |
| CA_1-11-18 | 1 | 11 | 18 |  |  |
| CA_1-18-28 | 1 | 18 | 28 |  |  |
| CA_1-19-21 | 1 | 19 | 21 |  |  |
| CA_1-19-28 | 1 | 19 | 28 |  |  |
| CA_1-19-42 | 1 | 19 | 42 |  |  |
| CA_1-21-42 | 1 | 21 | 42 |  |  |
| CA_2-4-5 | 2 | 4 | 5 |  |  |
| CA_2-2-4-5 | 2 | 2 | 4 | 5 |  |
| CA_2-4-4-5 | 2 | 4 | 4 | 5 |  |
| CA_2-4-7 | 2 | 4 | 7 |  |  |
| CA_2-4-12 | 2 | 4 | 12 |  |  |
| CA_2-4-13 | 2 | 4 | 13 |  |  |
| CA_2-4-29 | 2 | 4 | 29 |  |  |
| CA_2-4-30 | 2 | 4 | 30 |  |  |
| CA_2-5-12 | 2 | 5 | 12 |  |  |
| CA_2-2-5-12 | 2 | 2 | 5 | 12 |  |
| CA_2-5-13 | 2 | 5 | 13 |  |  |
| CA_2-5-29 | 2 | 5 | 29 |  |  |
| CA_2-5-30 | 2 | 5 | 30 |  |  |
| CA_2-7-12 | 2 | 7 | 12 |  |  |
| CA_2-12-30 | 2 | 12 | 30 |  |  |
| CA_2-29-30 | 2 | 29 | 30 |  |  |
| CA_3-5-40 | 3 | 5 | 40 |  |  |
| CA_3-7-8 | 3 | 7 | 8 |  |  |
| CA_3-7-20 | 3 | 7 | 20 |  |  |
| CA_3-7-28 | 3 | 7 | 28 |  |  |
| CA_3-8-40 | 3 | 8 | 40 |  |  |
| CA_3-19-42 | 3 | 19 | 42 |  |  |
| CA_3-7-38 | 3 | 7 | 38 |  |  |
| CA_3-28-40 | 3 | 28 | 40 |  |  |
| CA_3-41-42 | 3 | 41 | 42 |  |  |
| CA_4-5-12 | 4 | 5 | 12 |  |  |
| CA_4-4-5-12 | 4 | 5 | 12 |  |  |
| CA_4-5-13 | 4 | 5 | 13 |  |  |
| CA_4-5-29 | 4 | 5 | 29 |  |  |
| CA_4-5-30 | 4 | 5 | 30 |  |  |
| CA_4-4-5-30 | 4 | 4 | 5 | 30 |  |
| CA_4-7-12 | 4 | 7 | 12 |  |  |
| CA_4-12-30 | 4 | 12 | 30 |  |  |
| CA_4-4-12-30 | 4 | 12 | 30 |  |  |
| CA_4-29-30 | 4 | 29 | 30 |  |  |
| CA_4-4-29-30 | 4 | 4 | 29 | 30 |  |
| CA_7-8-20 | 7 | 8 | 20 |  |  |
| CA_7-20-38 | 7 | 20 | 38 |  |  |
| CA_19-21-42 | 19 | 21 | 42 |  |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination. |  |  |  |  |  |

Table 5.5A-2b: Inter-band CA operating bands (four bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1-3-5-40 | 1 | 3 | 5 | 40 |  |
| 1-3-7-8 | 1 | 3 | 7 | 8 |  |
| 1-3-7-28 | 1 | 3 | 7 | 28 |  |
| 1-3-8-40 | 1 | 3 | 8 | 40 |  |
| 1-3-19-42 | 1 | 3 | 19 | 42 |  |
| 1-19-21-42 | 1 | 19 | 21 | 42 |  |
| 2-4-5-12 | 2 | 4 | 5 | 12 |  |
| 2-4-5-29 | 2 | 4 | 5 | 29 |  |
| 2-4-5-30 | 2 | 4 | 5 | 30 |  |
| 2-4-7-12 | 2 | 4 | 7 | 12 |  |
| 2-4-12-30 | 2 | 4 | 12 | 30 |  |
| 2-4-29-30 | 2 | 4 | 29 | 30 |  |

Table 5.5A-3: Intra-band non-contiguous CA operating bands (with two sub-blocks)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_2-2 | 2 |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_5-5 | 5 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_40-40 | 40 |
| CA_41-41 | 41 |
| CA_42-42 | 42 |
| CA_66-66 | 66 |

## 5.5B	Operating bands for UL-MIMO

E-UTRA UL-MIMO is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5B-1: Void

## 5.5C	Operating bands for Dual Connectivity

E-UTRA dual connectivity is designed to operate in the operating bands defined in Table 5.5C-1.

Table 5.5C-1: Inter-band dual connectivity operating bands (two bands)

| E-UTRA DC Band | E-UTRA Band (Table 5.5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DC_1-3 | 1 | 3 |  |  |  |
| DC_1-5 | 1 | 5 |  |  |  |
| DC_1-7 | 1 | 7 |  |  |  |
| DC_1-8 | 1 | 8 |  |  |  |
| DC_1-19 | 1 | 19 |  |  |  |
| DC_1-21 | 1 | 21 |  |  |  |
| DC_1-42 | 1 | 42 |  |  |  |
| DC_2-4 | 2 | 4 |  |  |  |
| DC_2-5 | 2 | 5 |  |  |  |
| DC_2-12 | 2 | 12 |  |  |  |
| DC_2-13 | 2 | 13 |  |  |  |
| DC_3-5 | 3 | 5 |  |  |  |
| DC_3-7 | 3 | 7 |  |  |  |
| DC_3-8 | 3 | 8 |  |  |  |
| DC_3-19 | 3 | 19 |  |  |  |
| DC_3-20 | 3 | 20 |  |  |  |
| DC_3-26 | 3 | 26 |  |  |  |
| DC_4-5 | 4 | 5 |  |  |  |
| DC_4-7 | 4 | 7 |  |  |  |
| DC_4-12 | 4 | 12 |  |  |  |
| DC_4-13 | 4 | 13 |  |  |  |
| DC_4-17 | 4 | 17 |  |  |  |
| DC_5-7 | 5 | 7 |  |  |  |
| DC_5-12 | 5 | 12 |  |  |  |
| DC_7-20 | 7 | 20 |  |  |  |
| DC_7-28 | 7 | 28 |  |  |  |
| DC_19-21 | 19 | 21 |  |  |  |
| DC_39-41 | 39 | 41 |  |  |  |

Table 5.5C-2: Inter-band dual connectivity operating bands (three bands)

| E-UTRA DC Band | E-UTRA Band (Table 5.5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1-3-19 | 1 | 3 | 19 |  |  |  |
| DC_1-19-21 | 1 | 19 | 21 |  |  |  |

## 5.5D	Operating bands for ProSe

E-UTRA ProSe is designed to operate in the operating bands defined in Table 5.5D-1.

Table 5.5D-1 E-UTRA ProSe operating band

| E-UTRA ProSe Band | E-UTRA Operating Band | ProSe UE transmit |  |  | ProSe UE receive |  |  | ProSe Duplex Mode | ProSe Direct |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  | Disc. | Comm. |
| 2 | 2 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | HD | Yes |  |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1710 MHz | – | 1785 MHz | HD | Yes | Yes |
| 4 | 4 | 1710 MHz | – | 1755 MHz | 1710 MHz | – | 1755 MHz | HD | Yes |  |
| 7 | 7 | 2500 MHz | – | 2570 MHz | 2500 MHz | – | 2570 MHz | HD | Yes | Yes |
| 14 | 14 | 788 MHz | – | 798 MHz | 788 MHz | – | 798 MHz | HD | Yes | Yes |
| 20 | 20 | 832 MHz | – | 862 MHz | 832 MHz | – | 862 MHz | HD | Yes | Yes |
| 26 | 26 | 814 MHz | – | 849 MHz | 814 MHz | – | 849 MHz | HD | Yes | Yes |
| 28 | 28 | 703 MHz | – | 748 MHz | 703 MHz | – | 748 MHz | HD | Yes | Yes |
| 31 | 31 | 452.5 MHz | – | 457.5 MHz | 452.5 MHz | – | 457.5 MHz | HD | Yes | Yes |
| 41 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | HD | Yes |  |
| 68 | 68 | 698 MHz | – | 728 MHz | 698 MHz | – | 728 MHz | HD | Yes | Yes |

E-UTRA ProSe is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5D-2.

Table 5.5D-2 Inter-band E-UTRA ProSe / E-UTRA operating bands

| E-UTRA ProSe Band Note 1 | E-UTRA band / E-UTRA CA band Note 2 |
| --- | --- |
| 2 | 4 |
|  | CA_2-4Note 3 |
| 28 | 1 |
|  | CA_1-28Note 3 |
| NOTE 1: As specified in Table 5.5D-1 NOTE 2: As specified in Table 5.5-1 and Table 5.5A-2 NOTE 3: Applies when E-UTRA uplink is assigned to one E-UTRA band and ProSe operation is restricted to the uplink frequencies paired with either PCC or SCC. NOTE 4: The concurrency for E-UTRA ProSe Direct Discovery with E-UTRA uplink/downlink applies after allowing for any transmission and/or reception gap requested by the UE. |  |