3GPP TSG-RAN WG4 Meeting #78bis 	R4-162133

# San José del Cabo, Mexico, 11-15 April, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3417 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New reference measurement channels for FS3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-04-01 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test cases for FS3 require new FS3 specific reference measurement channels and OCNG patterns. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FS3 specific reference measurement channels for PDSCH and PCFICH/PDCCH/PHICH as well as and OCNG patterns for FS3 are presented. 20 MHz system bandwidth is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will refer to unspecified reference measurement channels and OCGN patters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.3.1.1.x, A.3.1.2.y, A.3.2.2.z and A.3.2.2.å |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***Start of change***

#### A.3.1.1.x	FS3

Table A.3.1.1.x-1: PDSCH Reference Measurement Channels for FS3 with TDD UL/DL configuration1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.xx FS3 |
| Channel bandwidth | MHz | 20 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks (Note 4) |  | 24 |
| Uplink-Downlink Configuration (Note 5) |  | 1 |
| Special Subframe Configuration (Note 6) |  | 6 |
| Allocated subframes per Radio Frame |  | 6 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 | Bits | 2088 |
| For Sub-Frame 5 | Bits | 2088 |
| For Sub-Frame 0 | Bits | 2088 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 1032 |
| Number of Code Blocks per Sub-Frame (Note 7) |  | 1 |
| For Sub-Frames 4,9 |  | 1 |
| For Sub-Frame 5 |  | 1 |
| For Sub-Frame 0 |  | 1 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 |
| Binary Channel Bits Per Sub-Frame |  |  |
| For Sub-Frames 4,9 | Bits | 6624 |
| For Sub-Frame 5 | Bits | 6480 |
| For Sub-Frame 0 | Bits | 5928 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 3696 |
| Max. Throughput averaged over 1 frame | Mbps | 1.0416 |
| Note 1: 2 symbols allocated to PDCCH for 10 MHz channel BW. 3 OFDM symbols allocated to PDCCH for 5MHz channel BW. 4 symbols allocated to PDCCH for 1.4 MHz channel BW. For special subframe (1 & 6) only 2 OFDM symbols are allocated to PDCCH for all bandwidths. Note 2: Reference signal, synchronization signals and PBCH allocated as defined in TS 36.211 [16]. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 36.213 [3]. Note 4: Allocation is located in the middle of bandwidth. Note 5: As per Table 4.2-2 in TS 36.211 [16] Note 6: As per Table 4.2-1 in TS 36.211 [16] Note 7: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 8: PDSCH allocation applies only to subframes not configured as PRS subframes. |  |  |

***End of change***

***Start of change***

### A.3.1.2.y 	FS3

Table A.3.1.y.2-1: PCFICH/PDCCH/PHICH Reference Channel for FS3

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.yy FS3 |
| Channel bandwidth |  |  | MHz | 20 |
| Number of transmitter antennas |  |  |  | 1 |
| Control region OFDM symbolsNote1 |  |  | symbols | 2 |
| Aggregation level |  |  | CCE | 8 |
| DCI Format |  |  |  | Note 3 |
| Cell ID |  |  |  | Note 4 |
| Payload (without CRC) |  |  | Bits | Note 5 |
|  |  | Note 1: The control region consists of PCFICH, PHICH and PDCCH. Note 2: DCI formats are defined in TS 36.212. Note 3: DCI format shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. Note 5: Payload size shall depend upon the test configuration. Note 6: Only 2 OFDM symbols for special subframes 1 and 6. Note 7: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. |  |  |

***End of change***

***Start of change***

#### A.3.2.2.z	OCNG FS3 pattern z: outer resource blocks allocation in 20 MHz

Table A.3.2.2.z-1: OP.z FS3: OCNG FS3 Pattern z for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe) Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 – 37 | 0 | 0 | 0 | Table A.3.2.1.7-2 | Note 2 |
| 62 – 99 | 0 | 0 | 0 |  |  |
| Note 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes. Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated The parameteris used to scale the power of PDSCH. Note 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration defined in Table 4.2-2 in 3GPP TS 36.211 [16]. Note 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in section 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

Table A.3.2.2.z-2: OP.y FS3: OCNG FS3 Pattern z for special subframe configuration with 5ms downlink-to-uplink switch-point periodicity

| Allocation | CP length | Relative power level [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Special subframe configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |
|  |  | Control region OFDM symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| 0 – 37 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| 62 – 99 | N | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| Note 1: Special subframe configurations are defined in Table 4.2-1 in TS 36.211 [16]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

***End of change***

***Start of change***

#### A.3.2.2.å	OCNG FS3 pattern å: full bandwidth allocation in 20 MHz

Table A.3.2.2.å-1: OP.å FS3: OCNG FS3 Pattern å for 5ms downlink-to-uplink switch-point periodicity

| Allocation | Relative power level [dB] |  |  |  | PDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe (Note 1) |  |  |  |  |
|  | 0 | 5 | 3 , 4, 8, 9 and 6 (as normal subframe)Note 3 | 1 and 6 (as special subframe) Note 3 |  |
| 0 – 99 | 0 | 0 | 0 | 0 | Note 2 |
| Note 1: The allocation of any PDSCH with or without SIB1 applies only to the subframes not configured as PRS subframes Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. Note 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in 3GPP TS 36.211 [16]. Note 4: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in section 7.1 in 3GPP TS 36.213. |  |  |  |  |  |

***End of change***