3GPP TSG-RAN WG4 Meeting #78bis 	R4-162219

San Jose del Cabo, Mexico, 11 – 15 April, 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR |  | rev |  | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE requirements due to introduction of 10MHz CBW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-3-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | New channel bandwidth of 10MHz is introduced for LAA from Rel-13. This CR introduces RF TX requirements for 10MHz for Rel-13 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE RF TX requirements for 10MHz channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for 10MHz channel bandwidth are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <Start of Changes>

### 5.6.1	Channel bandwidths per operating band

a)	The requirements in this specification apply to the combination of channel bandwidths and operating bands shown in Table 5.6.1-1. The transmission bandwidth configuration in Table 5.6.1-1 shall be supported for each of the specified channel bandwidths. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 5.6.1-1: E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 |  |  | Yes | Yes | Yes | Yes |
| 2 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 3 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 4 | Yes | Yes | Yes | Yes | Yes | Yes |
| 5 | Yes | Yes | Yes | Yes1 |  |  |
| 6 |  |  | Yes | Yes1 |  |  |
| 7 |  |  | Yes | Yes | Yes3 | Yes1, 3 |
| 8 | Yes | Yes | Yes | Yes1 |  |  |
| 9 |  |  | Yes | Yes | Yes1 | Yes1 |
| 10 |  |  | Yes | Yes | Yes | Yes |
| 11 |  |  | Yes | Yes1 |  |  |
| 12 | Yes | Yes | Yes1 | Yes1 |  |  |
| 13 |  |  | Yes1 | Yes1 |  |  |
| 14 |  |  | Yes1 | Yes1 |  |  |
| ... |  |  |  |  |  |  |
| 17 |  |  | Yes1 | Yes1 |  |  |
| 18 |  |  | Yes | Yes1 | Yes1 |  |
| 19 |  |  | Yes | Yes1 | Yes1 |  |
| 20 |  |  | Yes | Yes1 | Yes1 | Yes1 |
| 21 |  |  | Yes | Yes1 | Yes1 |  |
| 22 |  |  | Yes | Yes | Yes1 | Yes1 |
| 23 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 24 |  |  | Yes | Yes |  |  |
| 25 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 26 | Yes | Yes | Yes | Yes1 | Yes1 |  |
| 27 | Yes | Yes | Yes | Yes1 |  |  |
| 28 |  | Yes | Yes | Yes1 | Yes1 | Yes1, 2 |
| 30 |  |  | Yes | Yes1 |  |  |
| 31 | Yes | Yes1 | Yes1 |  |  |  |
| ... |  |  |  |  |  |  |
| 33 |  |  | Yes | Yes | Yes | Yes |
| 34 |  |  | Yes | Yes | Yes |  |
| 35 | Yes | Yes | Yes | Yes | Yes | Yes |
| 36 | Yes | Yes | Yes | Yes | Yes | Yes |
| 37 |  |  | Yes | Yes | Yes | Yes |
| 38 |  |  | Yes | Yes | Yes3 | Yes3 |
| 39 |  |  | Yes | Yes | Yes3 | Yes3 |
| 40 |  |  | Yes | Yes | Yes | Yes |
| 41 |  |  | Yes | Yes | Yes | Yes |
| 42 |  |  | Yes | Yes | Yes | Yes |
| 43 |  |  | Yes | Yes | Yes | Yes |
| 44 |  | Yes | Yes | Yes | Yes | Yes |
| 45 |  |  | Yes | Yes | Yes | Yes |
| 46 |  |  |  | Yes |  | Yes |
| … |  |  |  |  |  |  |
| 64 | Reserved |  |  |  |  |  |
| 65 |  |  | Yes | Yes | Yes | Yes |
| 66 | Yes | Yes | Yes | Yes | Yes | Yes |
| 68 |  |  | Yes | Yes | Yes |  |
| NOTE 1: 1 refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 2: 2 For the 20 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz NOTE 3: 3 refers to the bandwidth for which the uplink transmission bandwidth can be restricted by the network for some channel assignments in FDD/TDD co-existence scenarios in order to meet unwanted emissions requirements (Clause 6.6.3.2). |  |  |  |  |  |  |

## <Next Section>

### 7.3.1A	Minimum requirements (QPSK) for CA

<Unchanged text omitted>

For band combinations including operating band 46 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA and Table 7.3.1A-0eB.

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-46A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | [-100.7]4 | [-97.7]4 | [-95.9]4 | [-94.7]4 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3A-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_4A-46A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-46A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | [-100.7]4 | [-97.7]4 | [-95.9]4 | [-94.7]4 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_41A-46A | 41 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | FDD |
|  |  |  |  | [-100.7]4 | [-97.7]4 | [-95.9]4 | [-94.7]4 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is TBD NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: The requirements do not apply when there is at least one individual RE within the uplink transmission bandwidth of the lower band for which the 2nd/3rd/4th transmitter harmonic is within the downlink transmission bandwidth of the higher band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = FFS MHz for the configuations listed. |  |  |  |  |  |  |  |  |

## <Next Section>

### 7.5.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.5.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46, the requirements specified in subclause 7.5.1 are replaced by the requirements in Table 7.5.1A-0a with test parameters in Table 7.5.1A-0b and Table 7.5.1A-0c.

Table 7.5.1A-0a: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | ACS | dB |  |  |  | 33 |  | 27 |

Table 7.5.1A-0b: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | REFSENS+44.5dB |  | REFSENS +39.5dB |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

Table 7.5.1A-0c: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm |  |  |  | -56.5 |  | -50.5 |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of unplink operation, the transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

## <Next Section>

#### 7.6.1.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.1.1 for each component carrier while all downlink carriers are active. For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The requirements for the component carrier configured in the operating band without uplink operation are specified in Table 7.6.1.1A-0, Table 7.6.1.1A-0a and Table 7.6.1.1A-0b.

Table 7.6.1.1A-0: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 29, 32, 67 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

Table 7.6.1.1A-0a: In band blocking parameters for additional operating bands for carrier aggregation

| E-UTRA band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 (NOTE 3) | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  | 30+0.0025 |  | 30+0.0125 |
|  | FIoffset, case 2 | MHz |  |  |  | 50+0.0125 |  | 50+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

## <Next Section>

### 7.8.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.8.1.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46, the requirements specified in subclause 7.8.1.1 are replaced by the requirements in Table 7.8.1-1A-0.

Table 7.8.1.1A-0: Wide band intermodulation

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 |  |  |  | 9 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  |  | 20 |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  |  | -BW/2-30/+BW/2+30 |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 (TBD) |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>