3GPP TSG-RAN WG4 Meeting #78bis	                                         R4-162255

San Jose del Cabo, Mexico, 11 - 15 April, 2016

Source: 	Huawei, Hisilicon

Title: 	TP for Rel-14 2UL/2DL TR 36.714-02-02: Co-existence and dTib and dRib for CA_2DL_2A-7A_2UL_2A-7A

Agenda Item:	7.6.1

Document for:	Approval

# Background

This contribution provides a TP for TR 36.714-02-02 to capture co-existence and ∆TIB and ∆RIB for 2UL/2DL CA_2DL_2A-7A_2UL_2A-7A.

# References

[1] RP-16xxxx, “New WID: Basket WI for LTE Advanced Inter-band CA Rel-14 for 2DL/2UL”, RAN #71

# Text Proposal

<Start of Text Proposal for TR 36.714-02-02>

## 6.x	CA_2DL_2A-7A_2UL_2A-7A_BCS0

### 6.x.3 Co-existence issues

Table 6.X.3-1 lists B2+B7 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.X.3-1: Band 2 and Band 7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1130 | 1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9620 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1180 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8430 | 8090 | 5140 | 4830 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12190 | 9900 | 10210 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4010 | 3680 | 730 | 410 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

Based on the table above there are no harmonics or IMD’s that fall on top of own RX band.

2UL inter-band CA UE can have impacts on other systems such as WiFi, Bluetooth and GNSS due to its harmonics and intermodulations. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.X.3-2: 2UL B2+B7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | NO |  |  |
| Galileo | 1559 | - | 1591 | NO |  |  |
| GLONASS | 1591 | - | 1610 | NO |  |  |
| GPS | 1563 | - | 1587 | NO |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | NO | US/Europe |  |
|  | 2400 | - | 2494 | NO | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonic, IMD4 |
|  | 5150 | - | 5350 | NO | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd harmonic, IMD4 |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonic, IMD4 |

### 6.x.4	∆TIB and ∆RIB values

For UL inter-band CA_2-7, the same ∆TIB and ∆RIB can apply as in respective 1UL-2DL case.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-7A | 2 | 0.5 |
|  | 7 | 0.5 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-7A | 2 | 0 |
|  | 7 | 0 |

<End of Text Proposal>