3GPP TSG-RAN WG4 Meeting#78 bis    						     							  R4-162274

# San Jose del Cabo, Mexico,  11-15 April 2016

Title:	impact of B41 HPUE on CA_B3_B41

Source:	CMCC

Agenda Item:	8.1.1

Document for:	Approval

- Introduction

In RAN4#78, a WF was approved [1] that some open issues need to be discussed in the following meeting.

  - Adopt 1 dB ACLR tightening as one option as the basis for continuing work into the next meeting on MPR/A-MPR determination.

  - Companies to provide system coexistence simulation results based on expanded cell in aggressor network vs. conventional cell size in victim network.  China Telecom to provide specific simulation parameters by 03/15/16.

  - Companies are encouraged to provide simulation results with more aggressive power control parameters for information.

  - Companies to evaluate the impact of HPUE to CA with specific examples of B3+B41 with and without Pcell operation in B41.  In CA configuration, Pcell max Tx power is 23 dBm.  Impact to B7 and B38 to be assessed.

In this paper, we discuss the impact of B41 HPUE to CA_B3_B41

- Discussion

For high power UE in B41, high power PA and high power enduring B41 filter will be introduced. In the following table the data sheet of high power PA and normal one from one component vendor are listed.

Table 2-1 PA parameters

|  | Max output power | Tx noise in B3 | PA Gain |
| --- | --- | --- | --- |
| Vendor A Model 1 | 28dBm | -135dBm/Hz | 30.5dB |
| Vendor A Model 2 ( High Power) | 30.3dBm | -135dBm/Hz | 26.3dB |

And in Table 2-2 filter data sheet from another vendor are shown, and we can find that the isolation for high power one is even better.

Table 2-2 filter parameters

|  | B41 band filter attenuation @ B3 Tx (dB) Min | B41 band filter attenuation @ B3 Rx (dB) Min | Insertion loss | Max Input Power |
| --- | --- | --- | --- | --- |
| Vendor B | 23 dB | 19 dB | 3.9dB | 31dBm |
| Vendor B (High Power) | 25 dB | 20 dB | 3.8dB | 33dBm |

To evaluate the impact of HPUE to CA_B3_B41, three aspects need to be considered: noise from Tx thermal, noise induced by Tx leakage through Rx IP2 and noise induced by Rx reciprocal mixing.

Noise from B41 Tx thermal: total noise floor at PA output should be the sum of transceiver noise floor at PA output and PA noise floor. In table 2-1, we can see that both two PA have the same noise floor, and the gain for high power PA is even lower. Hence, with similar isonlation from B41 Tx to B3 Rx, high power PA will not cause larger total noise floor at PA output.

Noise induced by Tx leakage through Rx IP2: since CA is not in the scope of this SI, even high power UE operating in CA mode, Pcell max Tx power is always 23 dBm, i.e. the PA output power should keep the same as power class 3 mode. In addition, Rx IP2 value and cross band isolation almost are not affected so much due to high power design.  Therefore, noise induced by Tx leakage through Rx IP2 will not be increased.

Noise induced by Rx reciprocal mixing: this noise depends on Rx LO phase noise and cross band isolation, thus it will not be increased.

- Conclusion

In this paper, three aspects are analysed for the impact of HPUE to  CA_B3_B41. Base on the comparison between high power component and normal one. Introducing HPUE will cause no impact, and requirement can keep the same.

- Reference

[1] R4-161488	Band 41 HPUE Way ForwardQualcomm Incorporated, Sprint, China Telecom, China Unicom, CMCC

***************** Start of the TP to TR 36.886 **********************************************

- 5	Deployment and Coexistence Studies

- 5.1 	General

- 5.2	Band 41

- 5.2.1	Carrier aggregation of Band 3 + Band 41

For high power UE in B41, high power PA and high power enduring B41 filter will be introduced. Following table is we list the data sheet of high power PA and normal one from one component vendor.

Table 5.2.1-1 PA parameters

|  | Max output power | Tx noise in B3 | PA Gain |
| --- | --- | --- | --- |
| Vendor A Model 1 | 28dBm | -135dBm/Hz | 30.5dB |
| Vendor A Model 2 ( High Power) | 30.3dBm | -135dBm/Hz | 26.3dB |

And in Table 2-2 filter data sheet from another vendor are shown, and we can find that the isolation for high power one is even better.

Table 5.2.1-2 filter parameters

|  | B41 band filter attenuation @ B3 Tx (dB) Min | B41 band filter attenuation @ B3 Rx (dB) Min | Insertion loss | Max Input Power |
| --- | --- | --- | --- | --- |
| Vendor B | 23 dB | 19 dB | 3.9dB | 31dBm |
| Vendor B (High Power) | 25 dB | 20 dB | 3.8dB | 33dBm |

To evaluate the impact of HPUE to CA_B3_B41, three aspects need to be considered: noise from Tx thermal, noise induced by Tx leakage through Rx IP2 and noise induced by Rx reciprocal mixing.

Noise from B41 Tx thermal: total noise floor at PA output should be the sum of transceiver noise floor at PA output and PA noise floor. In table 2-1, we can see that both two PA have the same noise floor, and the gain for high power PA is even lower. Hence, with similar isonlation from B41 Tx to B3 Rx, high power PA will not cause larger total noise floor at PA output.

Noise induced by Tx leakage through Rx IP2: since CA is not in the scope of this SI, even high power UE operating in CA mode, Pcell max Tx power is always 23 dBm, i.e. the PA output power should keep the same as power class 3 mode. In addition, Rx IP2 value and cross band isolation almost are not affected so much due to high power design.  Therefore, noise induced by Tx leakage through Rx IP2 will not be increased.

Noise induced by Rx reciprocal mixing: this noise depends on Rx LO phase noise and cross band isolation, thus it will not be increased.

***************** End of the TP to TR 36.802 ***********************************************