3GPP RAN WG4 Meeting #78 bis                         	R4-162552

# San Jose del Cabo, Mexico, 11 – 15 April, 2016

Source: 	Sony

Title: 	WF on REFSENS for NB IOT

Agenda Item:	6.13.3

Document for:	Approval

# Introduction

In 3GPPRAN4-NB-IOT in Budapest and 3GPPRAN4#78 at Malta REFSENS for NB IOT was a discussed [1], [2]. A proposal was shown in [1]. The topic has been discussed on the RAN4 IOT mail reflector after the meetings. Two options on way forward have been discussed:

Option 1

Re-use the existing (legacy) REFSENS table and apply the same relaxation for single RX as was agreed for cat 0. Then there would be a scaling from the 5MHz value of 10log10(1RB/25RB) = -14dB. The assumption in this case is that the same SNR for NB IOT as for legacy LTE applies. There is also a band-dependant factor in legacy LTE whose relevance needs to be discussed since only half duplex is considered.

Option 2

The alternative method would be to start all over: determine required SNR (e.g. by simulation), estimate the noise figure for the receiver, and agree on an adequate implementation and production margin. The noise figure of the receiver need to be calculated in antenna connector and may have some frequency dependence (e.g. above/below 1GHz or something). Probably there has to be an agreement on possible reference architectures (SAW filter, SAW-less or maybe both) in order to do the NF calculation.

In this contribution we will elaborate further on REFSENS for NB IOT using the Option 2 approach.

# Discussion

In legacy LTE PREFSENS is calculated as

PREFSENS dBm= -174dBm + 10log10B+NF + IM+ Rb+ SNR

where

-174dBm	Noise floor at room temperature in a 50Ω load

B		Operating bandwidth in Hz

NF		Noise figure in dB of the receiver in antenna connector

IM		Implementation margin in dB

Rb		Band-dependent relaxation due to the ratio between duplexer distance and band gap in dB

SNR		Required Signal to Noise Ratio in baseband in dB

## Operating BW B

NB-IOT is 1RB in downlink =180kHz = 52.6dBHz

Scaling factor from 1.4MHz BW is 10*log10 1RB6RB = -7.8dB

Scaling factor from 5MHz BW is 10*log10 1RB25RB = -14dB

## Noise Figure NF

In legacy LTE a NF = 9dB is used with some exceptions (e.g. B12,13,17) where a higher NF is used. In addition a correction factor for TX self-mixing in the receiver is applied for 3MHz and 1.4MHz BW. This factor is not applicable for HD FDD. For a modern design a NF of 9dB might be on the high side, however, 9dB NF for NB IOT will allow for a low power and low cost implementation and will therefore be used in this document.

## Implementation margin IM

An IM of 2.5dB is used in Legacy LTE. Among other things the IM should also account for production variations. In order to achieve a good production yield the same IM could be used in NB IOT and thus allowing for a low total cost.

## Band dependant relaxation Rb

Rb is introduced to compensate for centre frequency versus temperature drift in SAW based duplex filters. Rb is shown in Table  1 as agreed in rel 8 [3].

Table  1.  Band-dependent relaxation Rb

| Duplexer distance/Band gap | Rb [dB] |
| --- | --- |
| value ≤ 1.5 | 0 |
| 1.5 < value ≤ 2 | 1 |
| 2 < value ≤ 4 | 2 |
| value > 4 | 3 |

For a SAW-less design in HD (half duplex) operation Rb does not apply. However, in order to enable implementation flexibility, the possibility to use high volume, low cost SAW filters would be highly beneficial (e.g. like those SAW filters originally designed for secondary RX). Therefore we propose that Rb should be reused from rel 8.

In the event of lower RX band than the TX band (e.g. B13 and B20) there are additional SAW design constraints. In addition to that there are constrains for B13 due to the vicinity of B14.  Therefore, 1dB is added to B20, and 2dB to B13.

## Signal to Noise Ratio SNR

SNR is the operating SNR in dB to meet the 95% of maximum throughput or 5% BLER under reference measurement channel (RMC). For legacy LTE SNR= -4dB is used for deriving REFSENS and is based on 2 Rx ports where the same power, as specified in Table 7.3.1-1 in TS 36.101, is applied at each port. In case of 1 Rx the SNR was assumed -1dB without any further investigation (cat 0 and cat M1). Regarding NB IOT we cannot assume the same SNR since there is relatively lower payload size and the usage of TBCC instead of turbo coding as in legacy LTE. A simulated SNR is depicted in Figure 1 assuming practical channel estimation. Simulation assumptions are listed in Table 2 for both in-band and standalone / guard-band cases. The REFSENS conformance test is a conductive test where the channel is AWGN. The channel estimation in the demodulator in NB IOT is only based on very few reference symbols (NB-RS). Therefore a practical channel estimate seems to more reflect the REFSENS conformance test scenario.

According to the simulations, the BLER of 5% can be obtained at SNR=1.5dB for in-band and SNR=1.9dB for standalone/guard-band operation.

Figure 1. BLER versus SNR for NB IOT with perfect channel estimation

Table 2 . Simulation assumptions In-band and standalone/guard-band cases

| Parameter | Unit | In-band Case | Standalone/Guardband Case |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 10 |
| Antenna configuration |  | SISO | SISO |
| Allocated resource blocks |  | 1 | 1 |
| Subcarriers per resource block |  | 12 | 12 |
| Allocated subframes per Radio Frame |  | 10 | 10 |
| Channel Coding |  | TBCC | TBCC |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of HARQ Processes | Processes | 8 | 8 |
| Maximum number of HARQ transmissions |  | 1 | 1 |
| Information Bit Payload per Sub-Frame |  | 56 | 88 |
| Transport block CRC | Bits | 24 | 24 |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |
| Binary Channel Bits Per Sub-Frame |  | 236 | 320 |
| Channel estimation |  | Practical | Practical |
| Frequency error |  | None | None |
| Timing error |  | None | None |

# REFSENS proposal

Table 4 shows REFSENS NB IOT In-Band and Stan-Alone/Guard-Band cases where the above estimations are used for the agreed bands for NB IOT

Table 4. Reference sensitivity for HD-FDD NB-IOT PREFSENS

| E-UTRA Band | REFSENS NB IOT, In-Band [dBm] | REFSENS NB IOT, Stand-Alone/Guard Band [dBm] | Duplex Mode |
| --- | --- | --- | --- |
| 1 | -108.4 | -108 | HD-FDD |
| 3 | -105.4 | -105 | HD-FDD |
| 5 | -106.4 | -106 | HD-FDD |
| 8 | -105.4 | -105 | HD-FDD |
| 12 | -106.4 | -106 | HD-FDD |
| 13 | -106.4 | -106 | HD-FDD |
| 17 | -107.4 | -107 | HD-FDD |
| 18 | -108.4 | -108 | HD-FDD |
| 19 | -108.4 | -108 | HD-FDD |
| 20 | -105.4 | -105 | HD-FDD |
| 26 | -105.4 | -105 | HD-FDD |
| 28 | -105.4 | -105 | HD-FDD |

# Conclusion

In this contribution we have presented a method for deriving REFSENS for NB IOT according to the Option 2 approach as previously discussed at the RAN4 IOT mail reflector.

REFSENS values for the agreed NB IOT bands are proposed for In-Band respectively Stan-Alone/Guard-Band case.

# References

R4-77AH-IoT-0059	“NB-IOT UE RF aspects”, Nokia Networks

R4-160644		“UE receiver requirements for NB-IOT”,Ericsson

R4-080696		“TP 36.101: REFSENS and associated requirements”, Ericsson