3GPP TSG-WG4 Meeting #78bis	R4-162634

# San Jose Del Cabo, Mexico, 11th – 15th April 2016

Agenda item: 	6.13.5

Source: 	Qualcomm Incorporated

Title: 	NB-IOT RSRP Simulation Results

Document for:	Discussion

# 1		Introduction

In RAN4#78, the following was captured in the WF [2]:

- RAN4 is to study RRM measurement performance using following options:

  - Option 1: Measurement based on NB-RS signals

    - To be used as simulation baseline only

  - Option 2: Measurement based on synchronization signals (e.g. NB-SSS)

    - provided that NB-SSS are available for measurement

  - Option 3: Measurement based on combination of NB-RS and NB-SSS

    - provided that NB-SSS are available for measurement

  - Other options are not precluded

The CDF curves are to be provided for:

- Estimated RSRP

- Estimated RSRQ

- Delta RSRP   = (estimated RSRP – ideal RSRP) 	[dB]

- Delta RSRQ  = (estimated RSRQ – ideal RSRQ) [dB]

This papers presents RSRP simulation results for NB-IOT based on the agreed simulation assumptions

# 2	Simulation Assumptions

| Parameters |  | Value | Comments |
| --- | --- | --- | --- |
| Measurement bandwidth |  | 1 resource blocks | RSRP measured over 1 RB |
| System bandwidth |  | 1 resource blocks |  |
| L1 measurement period |  | 50ms, 100ms, 200 ms, 400 ms, 800 ms | Even further increased measurement period can be considered to evaluate the measurement performance |
| L3 filtering |  | Disabled |  |
| Antenna configuration | In-band Guard-band | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern |
| Power boosting for NB-IOT | In-band Guard-band | 0 dB | NB-IoT is transmitted at 0dB or 6dB higher power than the MBB LTE cell to which it is associated. |
| Mobility |  | Stationary UEs, |  |
| Channel model |  | AWGN | Models stationary device |
| Measurement type |  | NB-RS only based, NB-PSS based, Combination of NB-RS and NB-PSS |  |
| CP length |  | Normal |  |
| Ec/Iot |  | -20 dB, …, 10 dB | AWGN noise |
| Frequency error modelling |  | 50 Hz | With respect to reference cell |

## 2.1	Simulator setup

The 180KHz NB-IOT signal consists of random QPSK data symbols with CRS and NB-RS tones inserted at appropriate locations and NB-PSS symbols at subframe#5 of each frame. This signal is sampled at 1.92MHz sampling rate. The total Tx power is set to unity. SNR is varied by adjusting the AWGN noise power. As defined in [2], the ideal RSRP is the true value that does not include any channel estimation noise but uses the same sampling rate as for the estimated RSRP. So ideal RSRP expected for this simulation setup is unity power i.e. 0dBm. Consequently, the delta RSRP would be the estimated RSRP minus one. Simulations are run for 100 iterations and the estimated measurements (RSRP) from all the iterations is shown as boxplot which captures certain percentiles from the CDF of the measurements.

# 3	Estimated RSRP Results

The boxplots for estimated RSRP for different SNRs and averaging lengths are shown below. The lower edge of each box shows the 25th-percentile value, the upper edge of the box shows the 75th percentile value, the black dotted line with horizontal caps show the range of values observed when the simulations was run for 100 iterations. The red line inside box shows the median (50th percentile) value and the red dot/square in the box shows mean value. The markers ‘+’ indicate points which are outliers.

NBRS_RP implies NBRS based RSRP measurement, PSS_RP implies NB-PSS based RSRP measurement and NBRSplusPSS_RP implies RSRP measurement based on averaging scaled NBRS and NB-PSS. NBRS_RP is measured once per slot and averaged coherently over slots in time, PSS_RP is measured 10 times per frame and averaged coherently per frame and over multiple frames in time. NBRSplusPSS_RP is measured once per frame and averaged coherently over multiple frames in time.

The following are the observations from the plots below:

Observation 1: At lower SNR and lower averaging, the measurement variance is high and the measurement mean is higher than ideal/expected value while at high SNR and higher averaging, the measurement variance is small and measurement mean is close to expected/ideal value.

Observation 2: PSS_RP and NBRSplusPSS_RP have smaller measurement variance compared to NBRS_RP

Observation 3: The delta between the PSS_RP mean value and the ideal PSS_RP is the smallest compared to the measurement delta for NBRS_RP and NBRSplusPSS_RP.

## 3.1	SNR = 10dB with Frequency Error = 50Hz

## 3.2	SNR = -10dB with Frequency Error = 50Hz

## 3.3	SNR = -20dB and Frequency Error = 50Hz

# 4	Conclusions

This paper has presented the boxplots showing CDF for SNR=-20, -10dB and 10dB at frequency error=50Hz. The following are the observations from the simulations:

Observation 1: At lower SNR and lower averaging, the measurement variance is high and the measurement mean is higher than ideal/expected value while at high SNR and higher averaging, the measurement variance is small and measurement mean is close to expected/ideal value.

Observation 2: PSS_RP and NBRSplusPSS_RP have smaller measurement variance compared to NBRS_RP

Observation 3: The delta between the PSS_RP mean value and the ideal PSS_RP is the smallest compared to the measurement delta for NBRS_RP and NBRSplusPSS_RP.

# 5	References

- TS36.133 v13.2.0, “Requirements for support of radio resource management”

- R4-161293, “Simulation assumption for RRM measurements for NB-IoT”, Ericsson, Nokia Networks, Alcatel-Lucent, Huawei, Hisilicon, Intel Corporation, ZTE, Qualcomm Incorporated