3GPP TSG-RAN WG4 Meeting #78bis	R4-162965

# San Jose del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	6.2	Base Station (BS) RF requirements for Active Antenna System (AAS)

6.2.3	Conformance requirements

Source:	Kathrein

Title:	Text proposal for chapter 10.3.2.1 (Test methods)

Document for:	Approval

# Introduction

Several test methodologies have been proposed in TR 37.840 [1]. The TR 37.842 [2] has to describe in sub-clause 10 the potential test methodologies for the AAS testing in detail.

In [3] a novel method for a one dimensional compact range chamber has been proposed. The method uses a one dimensional probe antenna for the generation of a quiet zone, where the radiated wave has a quasi plane wave character. In [4], we discussed the measurement uncertainties of our proposed method for multi column antennas in detail. We showed, that EIS can be determined with very high accuracy for multi column antennas.

It is proposed to approve this TP and include it in the TR.

# References

[1]	TR 36.840,  Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station

[2]	TR 37.842,  Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

[3]	R4-75AH-AAS-0004, One dimensional Compact Range Chamber, Kathrein

[4]	R4-157666, Multi-Column Antennas in One-Dimensional Compact Range Chamber, Kathrein

# Text proposal TR 37.842

10.3.2 OTA sensitivity

10.3.2.1 Test methods

##### 10.3.2.1.3 One Dimensional Compact Range Chamber

###### 10.3.2.1.3.1 Description

The principle of the measurement set up for OTA sensitivity is shown in Figure xx. It closely resembles the standard compact range chamber. But for the generation of the plane wave in the quiet zone the proposed method uses a special one dimensional probe instead of the standard set up comprising a concave mirror and a feed horn. The main advantage is a reduced chamber size and a simplified probe system replacing the standard mirror feed horn set up.

Figure xx: Proposed compact measurement set up for OTA sensitivity of AAS: one dimensional compact range chamber

The probe antenna is set up of a one dimensional array of antenna modules, connected by a special feed system. Optimized amplitude and phase settings in the feed system result in a plane wave in vertical direction. As no near field to far field transformation has to be carried out, it is not required to measure a whole set of azimuth and elevation angles.

###### Procedure

- Connect the feed system of the compact probe to the signal generator.

- Calibrate the test range, using a reference antenna with standard gain installed in the quiet zone of the probe and measure the path loss between signal generator and reference antenna.

- Position the device under test (DUT) in the quiet zone of the probe with its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the DUT is set to be aligned with testing system.

- Connect  the DUT to the measurement equipment.

- Align with the conformance test direction (D10.11)

- Set the signal generator to the manufacturer declared sensitivity (EIS) level plus the measured path loss.

- Measure the throughput and BER.of the DUT.

- Repeat test steps 2 to 6 for all declared beams and corresponding conformance steering directions..

###### 10.3.2.1.3.3 Uncertainty budget format

<Texts to be added>

###### 10.3.2.1.3.4 Uncertainty assessment

<Texts to be added>