3GPP RAN4 #79		R4-163262

Nanjing, China, 23-27 May, 2016

Title:	DRAFT LS to CTIA on performance requirement and methodology optimization work

Response to:	none

Source:	3GPP RAN WG4

To:	COST Action CA15104 (IRACON)

Cc:

Contact Person:

#### Name:	Anatoliy Ioffe

Tel. Number:	+1 503 712 4257

E-mail Address:	anatoliy.ioffe@intel.com

Attachments:	None

1. Overall Description:

3GPP RAN4 would like to update COST on the current status of the work associated with defining MIMO OTA performance requirements for UEs.

The receiver total radiated multi-antenna sensitivity (TRMS) has been defined as the MIMO OTA performance metric utilizing the multi-probe anechoic chamber (MPAC) testing methodology (Clauses 6.3.1.1 and 12.1 of TR 37.977 [7]).

The average TRMS of free space data mode portrait (FS DMP), free space data mode landscape (FSDML), and free space data mode screen up flat (FS DMSU), as defined in Annex E of TR 37.977, when measured at the mid channel shall be lower than the average TRMS requirements specified in subclauses 8.1.1.1 and 8.1.1.2 of TS 37.144.  The averaging shall be done in linear scale for the TRMS results at these DUT positions.  Two average TRMS quantities are calculated from sensitivity measurements at 70% and 95% throughput, respectively.  Average TRMS requirement are shown in the column “Average, 70” and “Average, 95” on the requirement tables.

TRMSaverage,70=10log3110SFS_DMP,7010+110SFS_DML,7010+110SFS_DMSU,7010

TRMSaverage,95=10log3110SFS_DMP,9510+110SFS_DML,9510+110SFS_DMSU,9510

Where

SMODE,x=10log12110PMODE,x,010+110PMODE,x,110+…+110PMODE,x,1110

Such that MODE is one of {FS_DMP, FS_DML, FS_DMSU}, x is one of {70, 95}, and {PMODE,x,0, …, PMODE,x,11} are the measured sensitivity values at each azimuth position.

If at most 1 azimuth position does not result in a defined measured sensitivity at 70% throughput, SMODE,70 can be calculated using 11 terms and scaled appropriately by setting the numerator of the term inside the logarithm to 11.  If at most 2 azimuth positions do not result in a defined measured sensitivity at 95% throughput, SMODE,95 can be calculated using 10 terms and scaled appropriately by setting the numerator of the term inside the logarithm to 10.  If more azimuth positions result in undefined values for measured sensitivity at the 70% and/or 95% throughput, then the TRMS requirement for the corresponding throughput levels has not been met by such a device.

3GPP RAN4 respectfully asks COST to take these developments into account in its own work on MIMO OTA research.

2. Actions:

None

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting#80		August 22-26, 2016				Gothenburg, Sweden

TSG-RAN4 Meeting#80bis	October 10-14, 2016				Ljubljana, Slovenia

4. References

[1] RP-160603 “New WI proposal: Radiated performance requirements for the verification of multi-antenna reception of UEs”