3GPP TSG-RAN WG4 Meeting #79	R4-163274

Nanjing, China, 23rd – 27th May, 2016

Agenda Item:	7.12.3

Source: 	Intel Corporation

Title: 	Network-assisted signaling for performance enhancement in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to specify higher-layer assistance signalling for UEs complying with the enhanced downlink demodulation performance requirements under the high speed scenarios.

In RAN4#78bis Meeting, a network-assisted signalling was suggested [2], which can be used to provide assistance and indicate a high speed train (HST) that it is entering/leaving a HST SFN area. Because of the different properties between the HST SFN channel model and conventional fading channels model [3]–[7], such as EVA channels, it would be beneficial for the UE to recognize its entering/leaving the HST SFN environment. Moreover, since there are two types of HST SFNs, namely Unidirectional and Bidirectional SFN, under current consideration [8]. It would be also beneficial for the UE to know which type of HST SFNs it is going to be under.

In this contribution, we continue the discussion on the network-assisted signalling to indicate that a train is entering/leaving a HST SFN area and the type of HST SFNs.

- Network-assisted signalling for HST SFN

Although, from the demodulation perspective, a UE may blindly detect its relative speed w.r.t. the base station (BS), it is still of importance for a UE to accurately know that it is running in a HST SFN environment, especially when it is begin to enter/leave such environment, so that it can make timely and effective responses. As a good example, the benefit for a UE knowing its exact HST SFN status can be justified by the considerable throughput improvement of HST enhanced UE, compared to a legacy UE [6]. Therefore, we suggest a network-assisted signalling to provide the HST SFN information to the HST UE.

Fig. 1 and 2 shows that when a train is entering/leaving the Unidirectional and Bidirectional HST SFN area, the non-SFN BS may send signals to the UE indicating that it is entering/leaving HST SFN area, as well as the type of HST SFNs it is under; alternatively, the signalling operation can be performed by SFN RRH/BS on the edge of the SFN area as well.

Fig. 1 Network-assisted signaling for entering/leaving Unidirectional HST SFN.

Fig. 2 Network-assisted signaling for entering/leaving Bidirectional HST SFN

Proposal: when a high speed train (HST) is entering/leaving the HST SFN area, the network entities, such as non-SFN and/or SFN RRH/BS on the edge of the SFN area may assist and signal UEs that they are entering/leaving HST SFN area, as well as the type of HST SFNs, i.e. Unidirectional or Bidirectional SFN. Then, the UE can take corresponding actions, such as advanced channel estimation for high speed scenarios.

- Conclusion

In this contribution, we discuss the network-assisted signalling to indicate that a train is entering/leaving a HST SFN area and the type of HST SFNs.

Proposal: when a high speed train (HST) is entering/leaving the HST SFN area, the network entities, such as non-SFN and/or SFN RRH/BS on the edge of the SFN area may assist and signal UEs that they are entering/leaving HST SFN area, as well as the type of HST SFNs, i.e. Unidirectional or Bidirectional SFN. Then, the UE can take corresponding actions, such as advanced channel estimation for high speed scenarios.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO INC., Huawei, HiSilicon, Dec. 2015.

[2] R4-161635, “Network assistance for performance enhancement in high speed scenarios”, Intel Corporation, April. 2016.

[3] R4-156999, “Discussion on UE demodulation performances for high speed train scenario with SFN deployment”, Intel Corporation, Nov. 2015.

[4] R4-157944, “Improvement in UE demodulation performance in SFN channel”, Qualcomm Incorporated, Nov. 2015.

[5] R4-156001, “UE demodulation performance evaluation of the SFN channel”, Ericsson, Oct. 2015.

[6] R4-155657, “Evaluation of UE demodulation performance under the identified SFN scenario”, Huawei, HiSilicon, Oct. 2015.

[7] TR-36.878, “Study on performance enhancements for high speed scenario in LTE”, Dec. 2015.

[8] RP-161252, “Way forward on UE performance enhancement under SFN channel”, Huawei, HiSilicon, NTT DoCoMo, Ericsson, Qualcomm, Intel, Feb. 2016.