3GPP TSG-RAN WG4 Meeting #79	   					       R4-163290

Nanjing, China, 23 - 27 May, 2016

Source:	China Telecom

Title:	TP on TR conclusions

Agenda Item:	6.4

Document for:	Approval

Introduction

This contribution provides a text proposal for TR 36.884 to add the TR conclusions.

Text Proposal

< Start of change>

10	Conclusions

This technical report has documented the RAN4 work on performance requirements of MMSE-IRC receiver for LTE BS. The major work includes the definition of reference receiver structures, determination of network parameters and targeted/interference channels, investigation of interference models and profiles, performance evaluation with link-level simulations, and introduction of BS demodulation performance requirements with synchronous and asynchronous interferences.

According to the link-level simulations in clause 7, it can be observed that with two dominant interferers modelled, the MMSE-IRC receiver can bring in the significant gain over MMSE receiver:

●	With respect to homogeneous network, for 1x2 cases the gain is about 3.1 dB; for 1x4 cases the gain is 4.4 dB ~4.7 dB; for 1x8 cases the gain is 4.9 dB ~5.0 dB.

●	With respect to heterogeneous network, for 1x2 cases the gain is 5.5 dB~5.9 dB; for 1x4 cases the gain is 7.5 dB~8.3 dB; for 1x8 cases the gain is 8.4 dB~8.9 dB.

Based on the extensive evaluations from interested companies, BS demodulation performance requirements for MMSE-IRC receiver with synchronous and asynchronous interferences are provided in clause 8.3 and 9.3 respectively.

< End of change >