3GPP TSG-RAN WG4 Meeting #79 	R4-163452

# Nanjing, China, 23-27 May, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3540 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifications on LAA cell identification and measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Cell detection and measurment requirements for FS3 have been discovered infeasible. Additionally, cell identification and measurement time is not restricted, and may in certain occasions allow too long interruptions in DRS transmission due to LBT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell identification and measurement requirement tables are updated according to agreements in RAN4#78bis. Additionally, a time window within which discovery signal occasions used for cell identification/measurements is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will remain infeasible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2, 8.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

### 8.11.2	CRS based discovery signal measurements

#### 8.11.2.1	E-UTRAN intra-frequency measurements

NOTE: The requirements in this section are applicable only for measurements on SCC following the frame structure type 3 [16].

The UE shall be able to identify new intra-frequency FS3 cells and perform measurements of identified intra-frequency cells without an explicit intra-frequency neighbour cell list containing physical layer cell identities. During the RRC_CONNECTED state the UE shall continuously measure identified intra-frequency cells and additionally search for and identify new intra-frequency cells.

##### 8.11.2.1.1	Requirements

###### 8.11.2.1.1.1	Requirements when no DRX is used

When no DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency cell within identification time Tidentify_intra_ FS3, which shall include detection of the cell and additionally performing a single measurement with measurement period Tmeasure_intra_FS3_CRS.

Tidentify_intra_FS3 is the intra-frequency period for cell identification as specified in Table 8.11.2.1.1.1-1,

Tmeasure_intra_FS3_CRS is the intra-frequency period for measurements as shown in Table 8.11.2.1.1.1-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions which are not available during Tidentify_intra_FS3 for cell detection at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions which are not available during Tmeasure_intra_FS3_CRS for the measurements at the UE due to the absence of the necessary radio signals.

Tidentify_intra_FS3 and Tmeasure_intra_FS3_CRS apply provided that discovery signals are present in at least [50] % of DMTC occasions between the first and last discovery signal occasion used for cell identification in Tidentify_intra_FS3 or measurements in Tmeasure_intra_FS3_CRS.

Table 8.11.2.1.1.1-1: Intra-frequency cell identification under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tidentify_intra_FS3 [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | ([6]+L) * TDMTC_periodicity |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | ([24]+L) * TDMTC_periodicity |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | ([2]+L) * TDMTC_periodicity |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | ([8]+L) * TDMTC_periodicity |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Section 9.1.18.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Section 9.1.18.3 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.11.2.1.1.1-1.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_FS3_CRS when no DRX is used. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_intra_FS3_CRS as shown in Table 8.11.2.1.1.1-2, when no DRX is in use. The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_intra_FS3_CRS.

Table 8.11.2.1.1.1-2: Intra-frequency measurement requirements under operation with frame structure 3

| H Ês/Iot |  |  | CRS measurement bandwidth [RB] |  | CRS Ês/Iot | Tmeasure_intra_FS3_CRS [ms] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot |  |  | <25 |  | [-6] ≤ CRS Ês/Iot | ([5]+M) * TDMTC_periodicity |  |
| [-6] ≤ SCH Ês/Iot < [0] |  |  | <25 |  |  | ([20]+M) * TDMTC_periodicity |  |
| [0] ≤ SCH Ês/Iot |  |  | 25 |  | [0] ≤ CRS Ês/Iot | ([1]+M) * TDMTC_periodicity |  |
| [-6] ≤ SCH Ês/Iot < [0] |  |  | 25 |  |  | ([4]+M) * TDMTC_periodicity |  |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |  |  |  |  |

The RSRP measurement accuracy for all measured cells shall be as specified in Section 9.1.18.2, and the RSRQ measurement accuracy for all measured cells shall be as specified in Section 9.1.18.3.

8.11.2.1.1.1.1	Measurement Reporting Requirements

8.11.2.1.1.1.1.1	Periodic Reporting

Reported RSRP and RSRQ measurements contained in periodically triggered measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

8.11.2.1.1.1.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurements contained in event triggered periodic measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in Section 8.11.2.1.1.1.1.3.

8.11.2.1.1.1.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurements contained in event triggered measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_FS3 defined in Section 8.11.2.1.1.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_FS3 defined in Section 8.11.2.1.1.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_FS3_CRS provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

###### 8.11.2.1.1.2	Requirements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency cell within identification time Tidentify_intra_FS3_DRX, which shall include detection of the cell and additionally performing a single measurement with measurement period Tmeasure_intra_FS3_CRS_DRX.

Tidentify_intra_FS3_DRX is the intra-frequency period for cell identification as shown in Table 8.11.2.1.1.2-1,

Tmeasure_intra_FS3_CRS_DRX is the intra-frequency period for measurements as shown in Table 8.11.2.1.1.2-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions during ON DURATION and which are not available during Tidentify_intra_FS3_DRX for cell identification at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_intra_FS3_CRS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

Tidentify_intra_FS3_DRX and Tmeasure_intra_FS3_CRS_DRX apply provided that discovery signals are present in at least [50] % of DMTC occasions between the first and last discovery signal occasion used for cell identification in Tidentify_intra_FS3_DRX or measurements in Tmeasure_intra_FS3_CRS_DRX.

Table 8.11.2.1.1.2-1: Intra-frequency cell identification under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tidentify_intra_FS3_DRX [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | ([6]+L)*Max(TDMTC_periodicity, DRX cycle length) |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | ([24]+L)*Max(TDMTC_periodicity, DRX cycle length) |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | ([2]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | ([8]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in Section 9.1.18.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Section 9.1.18.3 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.11.2.1.1.2-1.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_FS3_ CRS_DRX when DRX is used. If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_ intra_FS3_CRS_DRX as shown in Table 8.11.2.1.1.2-2, when DRX is in use. The UE shall be capable of p performing RSRP and RSRQ measurements for 3 identified intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_intra_FS3_CRS_DRX.

Table 8.11.2.1.1.2-2: Intra-frequency measurement requirements under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tmeasure_intra_FS3_CRS_DRX [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | ([5]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | ([20]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | ([1]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | ([4]+M)*Max(TDMTC_periodicity, DRX cycle length) |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

The RSRP measurement accuracy for all measured cells shall be as specified in Section 9.1.18.2, and the RSRQ measurement accuracy for all measured cells shall be as specified in Section 9.1.18.3.

8.11.2.1.1.2.1	Measurement Reporting Requirements

8.11.2.1.1.2.1.1	Periodic Reporting

Reported RSRP and RSRQ measurements contained in periodically triggered measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

8.11.2.1.1.2.1.2	Event-triggered Periodic Reporting

Reported RSRP and RSRQ measurements contained in event triggered periodic measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in Section 8.11.2.1.1.2.1.3.

8.11.2.1.1.2.1.3	Event Triggered Reporting

Reported RSRP and RSRQ measurements contained in event triggered measurement reports shall meet the requirements in Sections 9.1.18.2 and 9.1.18.3, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_FS3_DRX defined in Section 8.11.2.1.1.2. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_FS3_DRX defined in Section 8.11.2.1.1.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasuer_intra_FS3_CRS_DRX provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

#### 8.11.2.2	E-UTRAN inter-frequency measurements

Editor’s note: the requirements are TBD.

<< End of change >>

<< Start of change >>

### 8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation

#### 8.12.2.1	Introduction

The requirements in Section 8.12.2 shall apply for CRS based discovery signal measurements comprising RSRP and RSRQ measurements [4].

#### 8.12.2.2	Measurements of a secondary component carrier

A secondary component carrier may be activated and deactivated by MAC-CE commands as specified in [17]. The applicable performance requirements depend on whether the SCell on the corresponding frequency is actived or deactivated.

#### 8.12.2.3	Measurements of a secondary component carrier with active SCell

When the SCell is activated, measurement performance requirements for the frequency are those given in Section 8.11.2.1. If common DRX is in use, then the requirements for that secondary component carrier are given by the DRX requirements in Section 8.11.2.1.1.2, otherwise the non-DRX requirements are applicable. The applicable measurement accuracy requirements are in Section 9.1.19.

#### 8.12.2.4	Measurements of a secondary component carrier with deactivated SCell

This section defines the measurement requirements of a secondary component carrier with deactivated SCell based on the parameter measCycleSCell defined in TS 36.331 [2].

##### 8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used

When no DRX is in use the UE shall be able to identify a new detectable cell on a secondary component carrier within identification time Tidentify_SCC_FS3, according to the parameter measCycleSCell, which shall include detection of the cell and additionally performing a single measurement with measurement period Tmeasure_SCC_FS3_CRS.

Tidentify_SCC_FS3 is the time period for cell identification as specified in Table 8.12.2.4.1-1,

Tmeasure_SCC_FS3_CRS is thetime period for measurements as shown in Table 8.12.2.4.1-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions which are not available during Tidentify_SCC_FS3 for cell identification at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions which are not available during Tmeasure_SCC_FS3_CRS for the measurements at the UE due to the absence of the necessary radio signals.

NConfigured_SCell is the number of SCells configured at the UE activated or deactivated.

Tidentify_SCC_FS3 and Tmeasure_SCC_FS3_CRS apply provided that discovery signals are present in at least [50] % of DMTC occasions between the first and last discovery signal occasion used for cell identification in Tidentify_SCC_FS3 or measurements in Tmeasure_SCC_FS3_CRS.

Table 8.12.2.4.1-1: Intra-frequency cell identification on SCC under operation with frame structure 3 with deactivated SCell

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tidentify_SCC_FS3 [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | Max {([6]+L) * measCycleSCell, ([6]+L)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | Max {([24]+L) * measCycleSCell, ([24]+L)* TDMTC_periodicity*NConfigured_SCell} |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | Max {([2]+L) * measCycleSCell, ([2]+L)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | Max {([8]+L) * measCycleSCell, ([8]+L)* TDMTC_periodicity*NConfigured_SCell} |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

A cell shall be considered detectable when

-	RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	 SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.12.2.4.1-1.

The measurement period for deactivated SCell measurements is Tmeasure_SCC_FS3_CRS according to the parameter measCycleSCell shown in Table 8.12.2.4.1-1.

Table 8.12.2.4.1-2: Intra-frequency cell measurement requirements on SCC under operation with frame structure 3 with deactivated SCell

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tmeasure_SCC_FS3_CRS [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | Max {([5]+M) * measCycleSCell, ([5]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | Max {([20]+M) * measCycleSCell, ([20]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | Max {([1]+M) * measCycleSCell, ([1]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | Max {([4]+M) * measCycleSCell, ([4]+M)* TDMTC_periodicity*NConfigured_SCell} |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified cells on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_SCC_FS3_CRS.

The measurement accuracy for all measured cells shall be as specified in Section 9.1.19.

A UE may reconfigure the receiver bandwidth or turn on/off one of the RF chains when performing measurements on one or two SCCs with deactivated SCell. This may cause interruptions on PCell or an activated SCell that are specified in Section 7.8.

8.12.2.4.1.1	Measurement Reporting Requirements

8.12.2.4.1.1.1	Periodic Reporting

Reported measurements contained in periodically triggered measurement reports shall meet the requirements in Section 9.

8.12.2.4.1.1.2	Event-triggered Periodic Reporting

Reported measurements contained in event triggered periodic measurement reports shall meet the requirements in Section 9.

The first report in event triggered periodic CRS based measurement reporting shall meet the requirements specified in Section 8.12.2.4.1.1.3.

8.12.2.4.1.1.3	Event Triggered Reporting

Reported measurements contained in event triggered measurement reports shall meet the requirements in Section 9.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which is caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_SCC_FS3 defined in Section 8.12.2.4.1. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_SCC_FS3 defined in Section 8.12.2.4.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_SCC_FS3_CRS provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used an additional delay can be expected.

##### 8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 cell on a secondary component carrier within identification time Tidentify_SCC_FS3_DRX, according to the parameter measCycleSCell, which shall include detection of the cell and additionally performing a single measurement with measurement period Tmeasure_SCC_FS3_CRS_DRX.

Tidentify_SCC_FS3_DRX is the time period for cell identification as shown in Table 8.12.2.4.2-1,

Tmeasure_SCC_FS3_CRS_DRX is the time period for measurements as shown in Table 8.12.2.4.2-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions during ON DURATION and which are not available during Tidentify_SCC_FS3_DRX for cell identification at the UE due to the absence of the necessary radio signals,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_SCC_FS3_CRS_DRX for the measurements at the UE due to the absence of the necessary radio signals.

NConfigured_SCell is the number of SCells configured at the UE activated or deactivated.

Tidentify_SCC_FS3_DRX and Tmeasure_SCC_FS3_CRS_DRX apply provided that discovery signals are present in at least [50] % of DMTC occasions between the first and last discovery signal occasion used for cell identification in Tidentify_SCC_FS3_DRX or measurements in Tmeasure_SCC_FS3_CRS_DRX.

Table 8.12.2.4.2-1: Cell identification on SCC under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tidentify_SCC_FS3_DRX [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | Max {([6]+L) * Max{ measCycleSCell, DRX cycle length }, ([6]+L)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | Max {([24]+L) * Max{ measCycleSCell, DRX cycle length }, ([24]+L)* TDMTC_periodicity*NConfigured_SCell} |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | Max {([2]+M) * Max{ measCycleSCell, DRX cycle length }, ([2]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | Max {([8]+M) * Max{ measCycleSCell, DRX cycle length }, ([8]+M)* TDMTC_periodicity*NConfigured_SCell} |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

A cell shall be considered detectable when

-	RSRP related side condition given in Section 9.1.19 are fulfilled for a corresponding Band,

-	 SCH_RP|dBm is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.12.2.4.2-1.

The measurement period for deactivated scell measurements is Tmeasure_SCC_FS3_CRS_DRX according to the parameter measCycleSCell shown in Table 8.12.2.4.2-2.

Table 8.12.2.4.2-2: Measurement requirements on SCC under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] | CRS Ês/Iot | Tmeasure_SCC_FS3_CRS_DRX [ms] |
| --- | --- | --- | --- |
| [0] ≤ SCH Ês/Iot | <25 | [-6] ≤ CRS Ês/Iot | Max {([5]+M) * Max{ measCycleSCell, DRX cycle length }, ([5]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | <25 |  | Max {([20]+M) * Max{ measCycleSCell, DRX cycle length }, ([20]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [0] ≤ SCH Ês/Iot | 25 | [0] ≤ CRS Ês/Iot | Max {([1]+M) * Max{ measCycleSCell, DRX cycle length }, ([1]+M)* TDMTC_periodicity*NConfigured_SCell} |
| [-6] ≤ SCH Ês/Iot < [0] | 25 |  | Max {([4]+M) * Max{ measCycleSCell, DRX cycle length }, ([4]+M)* TDMTC_periodicity*NConfigured_SCell} |
| NOTE: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. |  |  |  |

The UE shall be capable of performing RSRP and RSRQ measurements for 3 identified cells on a secondary component carrier, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_SCC_FS3_CRS_DRX.

The measurement accuracy for all measured cells shall be as specified in Section 9.1.19.

A UE may reconfigure receiver bandwidth taking into account the SCell activation/deactivation status, and when making measurements of cells on one or two SCCs with deactivated SCell. This may cause interruptions (packet drops) to a PCell or an activated SCell. No interruptions while the On Duration timer is running shall be allowed when common DRX is used. The requirement considers only interruptions due to reconfiguration of the receiver bandwidth, and not due to other causes such as RF impairments or channel conditions.

8.12.2.4.2.1	Measurement Reporting Requirements

8.12.2.4.2.1.1	Periodic Reporting

Reported measurements contained in periodically triggered measurement reports shall meet the requirements in Section 9.

8.12.2.4.2.1.2	Event-triggered Periodic Reporting

Reported measurements contained in event triggered periodic measurement reports shall meet the requirements in Section 9.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in Section 8.12.2.4.2.1.3.

8.12.2.4.2.1.3	Event Triggered Reporting

Reported measurements contained in event triggered measurement reports shall meet the requirements in Section 9.

The UE shall not send any event triggered CRS based measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which is caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_SCC_FS3 defined in Section 8.12.2.4.2. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_SCC_FS3_DRX defined in Section 8.12.2.4.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_SCC_FS3_CRS_DRX provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used an additional delay can be expected.

<< End of change >>