3GPP TSG-RAN WG4 meeting #79	R4-163542

Nanjing, China, 23 – 27 May, 2016

Source:	Ericsson

Title:	TP to TR 36.714-03-01: coexistence analysis and UE requirements for CA_2A-7A-66A

Agenda item:	7.3.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-03-01 for introduction of 3DL/1UL CA combination CA_2A-7A-66A.

# 2. Text Proposal for TR 36.714-03-01

##### ---Start of changes---

## 6.x		CA_3DL_2A-7A-66A_1UL_BCS0

### 6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-7-66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-7A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

### 6.x.3	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 7 and Band 66 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.x.3-1 below:

Table 6.x.3-1: Band 2, Band 7 and Band 66 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1990 | 2110 | 2200 | 2620 | 2690 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1350 |  |  | 1525 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2350 |  |  | 2570 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2810 |  |  | 2960 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6660 |  |  | 6880 |  |  |

It can be seen from Table 6.x.3-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 7 + 66) may fall into the BS receive band of Band 7, Band 40 and Band 41. However, Band 40 is not intended for use in the same geographical area as Bands 2, 7 and 66.

### Therefore, except the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused by BS supporting 3 DL CA of Band (2 + 7 + 66) will fall into the BS receive band of Band 7 amongst bands operating in the same geographical area.

### 6.x.4	∆TIB and ∆RIB values

For deriving ΔTIB,c and ΔRIB,c values for Band (2 + 7 + 66) we use filter data based on a hexplexer implemented in FBAR technology, but also make a comparison to the 2DL band combinations Band (2 + 7), Band (2 + 66) and Band (4 + 7) already specified in view of other implementations. The first two of these 2DL combinations are also fallback modes of Band (2 + 7 + 66). The requirements for the 2DL fallback modes should not be more stringent than the corresponding 2DL combinations for UE supporting the 3DL combination Band (2 + 7 + 66), even though the filter technologies improve. Furthermore, most UE support a multiple of combinations so filter replication is a challenge.

The hexplexer considered also supports operation in the G-block above Band 2 and thus operation in Band 25, which is used in North America and parts of Latin America. Hence the requirements for Band (2 + 7 + 66) thus derived would also be relevant for Band 25 and Band (7 + 25 + 66) operation and a larger fleet of UEs, which also facilitates component reuse. In fact, nowadays many duplexers for Band 2 are also designed to support Band 25.

Figure 6.x.4-1 displays the broadband response of a hexplexer supporting both Band (2 + 7 + 66) and Band (7 + 25 + 66). The plots below represent the performance of typical filters at room temperature (25C). However, reading the plots 5 MHz outside the nominal frequency band corresponds fairly well to extended temperature conditions (ETC) for -20 to +85 C. The TX and RX passband responses for B2 (B25), B7 and B66 are shown in detail for frequencies up to 5 MHz outside the nominal frequency range in Figure 6.x.4-2, Figure 6.x.4-3 and Figure 6.x.4-4, respectively.

Figure 6.x.4-1: broadband response for a Band (7 + 25 + 66) FBAR hexplexer with B25TX (pink), B25RX (light blue), B7TX (brown), B7RX (navy) and B66TX (red) and B66RX (blue).

Figure 6.x.4-2: traces for B25TX/B2TX (left) and B25RX/B2RX (right) at room temperature.

Figure 6.x.4-3: traces for B7TX (left) and B7RX (right) at room temperature.

Figure 6.x.4-4: traces for B66TX (left) and B66RX (right) at room temperature.

In order to estimate the ΔTIB,c and ΔRIB,c we compare the maximum IL 5 MHz outside the passband for each band displayed in the figures above to the specified IL for commercial B2 and B4 and B7 duplexers also implemented in FBAR. The results are shown in Table 6.x.4-1 and Table 6.x.4-2 with the “IL quadplexer” obtained by reading 5 MHz outside the nominal frequency range in the figures above and the “IL duplexer” the quoted minimum performance at ETC for the corresponding duplexer (in production). The duplexer data for Band 66 represent estimates for a pre-commercial product. The estimated ΔTIB,c and ΔRIB,c are obtained by using the shared-pain rule (50% of the IL), the values in brackets are the maximum values for the existing 2DL combinations Band (2 + 7) and Band (2 + 66) shown in Table 6.x.4-3.

Table 6.x.4-1: TX insertion loss across temperature

| Inter-band CA Configuration | E-UTRA Band | IL quadplexer [dB] | IL duplexer [dB] | Estimated ΔTIB,c [dB] |
| --- | --- | --- | --- | --- |
| CA_2A-7A-66A | 2 (25) | 3.4 | 3.0 | 0.2 (0.5) |
|  | 7 | 3.1 | 3.0 | 0.1 (0.5) |
|  | 66 | 3.4 | 2.2 | 0.6 (0.5) |

Table 6.x.4-2: RX insertion loss across temperature

| Inter-band CA Configuration | E-UTRA Band | IL quadplexer [dB] | IL duplexer [dB] | Estimated ΔRIB,c [dB] |
| --- | --- | --- | --- | --- |
| CA_2A-7A-66A | 2 (25) | 3.8 | 3.8 | 0 (0.3) |
|  | 7 | 2.6 | 3.0 | 0 (0) |
|  | 66 | 2.8 | 2.6 | 0.1 (0.3) |

Table 6.x.4-3: Performance for other 2DL and 3DL combiantions used for deriving ΔTIB,c and ΔRIB,c values

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_2A-7A | 2 | 0.5 | 0 |
|  | 7 | 0.5 | 0 |
| CA_4A-7C | 4 | 0.5 | 0.5 |
|  | 7 | 0.5 | 0.5 |
| CA_2A-66A | 2 | 0.5 | 0.3 |
|  | 66 | 0.5 | 0.3 |
| CA_2A-4A-7A | 2 | 0.5 | 0.3 |
|  | 4 | 0.5 | 0.5 |
|  | 7 | 0.5 | 0.5 |

The requirement for a constituent band of a 2DL mode of the 3DL combination should not be more stringent than the requirement for the corresponding “standalone” requirements for a UE not supporting the 3DL combination. This leads to the requirements shown in Table 6.x.4-4 and Table 6.x.4-5.

Table 6.x.4-4: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| Ca_2A-7A-66A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |

Table 6.x.4-5: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| Ca_2A-7A-66A | 2 | 0.3 |
|  | 7 | 0 |
|  | 66 | 0.3 |

It is noted that these requirements are more stringent than those of Band (2 + 4 + 7) even though Band 66 has a wider passband than Band 4. Band (2 + 4 + 7) was specified with a larger ΔRIB,c for Band 4 and Band 7 such that the Band 4 + Band 7 fall-back mode would not be specified with tighter requirements than the standalone 2DL combination Band (4 + 7).

The requirements for Band (2 + 7 + 66) above would also be relevant for Band (7 + 25 + 66) that includes the G-block.

##### ---End of changes---