3GPP TSG RAN4 Meeting #79	R4-163620

Nanjing, China, 23 – 27 May, 2016

Title: 	MSD for 3+41 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	7.6.1

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 3+41 UL CA.

# 2	Discussion

During the 1UL/2DL CA_3A_41A it was agreed to use a local diplexer as implementation architecture [1]. We use the same assumption in our respective UL CA analysis. Basically the architecture, found from [1] with added 2UL capabilities looks as below.

3+41 UL CA has IMD4 relation 2*B41 UL-2*B3UL=B3 DL. For instance the frequencies listed in table below result in IMD4 on top of B3 DL. The final UL frequencies are still to be agreed, but the IMD magnitude analysis can still be conducted. In single carrier specifications, there is some desensitization allowed for B3 due to lack of cross-band isolation. In this contribution we have included also this mechanism to the analysis.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_3A_41A | 3 | 1740 | 5 | 25 | 1835 | TBD | FDD-TDD |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |

We analyze the amount of IMD using the following component linearity assumptions:

Antenna switch IP4					56

Duplexer IP4							55

B3 forward mixing IP4				28

B41 forward mixing IP4				28

B3 reverse mixing IP4				31

B41 reverse mixing IP4				31

B3 LNA IIP4							-6

B41 LNA IIP4						-6

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation TX-RX				65

RFIC Pout B3/B41					0

B3 duplexer TX-RX isolation		50

B3 duplexer TX rejection at B41	31

B3 duplexer RX rejection at B41	42

B41 filter attenuation at B3 TX		26

B41 filter attenuation at B3 RX		22

Diplexer IP4							55

Diplexer isolation						15

Close proximity mechanism:

PA noise at RX					  -138

RX IIP2								50

LO phase noise					  -155

RFIC noise at RX					  -155

Please note that we have accounted the 3dB difference in B41 TX power between single UL CA and 2UL CA in assumptions and analysis.

Based on parameters above the IMD4 is 11.6 excluding CF. The most dominating factors assuming these parameters are diplexer, antenna switch, and PA forward mixing. We did slightly modify the PA reverse and forward mixing values based on PA measurements. We did not change passive front-end component parameters.

# 3	Conclusion

MSD test points and IMD for UL CA B3+B41 was provided.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | IMD (dB) | Duplex mode |
| CA_3A_41A | 3 | 1740 | 5 | 25 | 1835 | 11.6 | FDD-TDD |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |

# 4	References

[1] R4-152493, "Way Forward on UE RF analysis for 3+41," China Unicom, CATR