3GPP TSG-RAN WG4 meeting #79	R4-163727

# Nanjing, China, 23 – 27 May, 2016

Source:	Ericsson

Title:	TP for TR 36.714-03-01 on co-existence analysis for 1A-7A-42A, 3A-7A-42A, 1A-20A-42A, 3A-20A-42A, 7A-20A-42A

Agenda item:				7.3.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-03-01 for co-existence analysis for 3DL/1UL CA combinations 1A-7A-42A, 3A-7A-42A, 1A-20A-42A, 3A-20A-42A, 7A-20A-42A.

# 2. Text Proposal for TR 36.714-03-01

##### ---Start of changes---

### 6.27.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 1 Band 7 and Band 42 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.27.2-1 below:

Table 6.27.2-1: Band 1, Band 7 and Band 42 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 2620 | 2690 | 3400 | 3600 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1130 |  |  | 1460 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2820 |  |  | 3150 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3850 |  |  | 4180 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 8130 |  |  | 8460 |  |  |

It can be seen from Table 6.27.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (1 + 7 + 42) may fall into the BS receive band of Bands 11,21 and 45. As B42 is a TDD band, simultaneous Tx/Rx for the same band will not happen. So the IMD and harmonic impact to B42 needs not to be considered

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 11, 21 or 45 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (1 + 7 + 42) BS transmitter does not share the same antenna with Bands 11, 21 or 45 BS receiver.

Therefore, it is recommended that Band (1 +7 + 42) BS transmitter should not share the same antenna with Bands 11, 21 or 45 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 11, 21 or 45 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### ---Text omitted---

### 6.28.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 3 Band 7 and Band 42 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.28.2-1 below:

Table 6.28.2-1: Band 3, Band 7 and Band 42 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1805 | 1880 | 2620 | 2690 | 3400 | 3600 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 825 |  |  | 1170 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2515 |  |  | 2860 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 4140 |  |  | 4485 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 7825 |  |  | 8170 |  |  |

It can be seen from Table 6.28.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 7 + 42) may fall into the BS receive band of Bands 5, 7, 8, 18, 19, 20, 26, 38 and 41. As B42 is a TDD band, simultaneous Tx/Rx for the same band will not happen. So the IMD and harmonic impact to B42 needs not to be considered

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 5, 7, 8, 18, 19, 20, 26, 38 and 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (3 + 7 + 42) BS transmitter does not share the same antenna with Bands 5, 7, 8, 18, 19, 20, 26, 38 and 41 BS receiver.

Therefore, it is recommended that Band (1 +7 + 42) BS transmitter should not share the same antenna with Bands 5, 7, 8, 18, 19, 20, 26, 38 and 41 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 5, 7, 8, 18, 19, 20, 26, 38 and 41 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### ---Text omitted---

### 6.29.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 1 Band 20 and Band 42 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.29.2-1 below:

Table 6.29.2-1: Band 1, Band 20 and Band 42 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 791 | 821 | 2110 | 2170 | 3400 | 3600 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 599 |  |  | 409 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2021 |  |  | 2311 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 4689 |  |  | 4979 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6301 |  |  | 6591 |  |  |

It can be seen from Table 6.29.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (1 + 20 + 42) may fall into the BS receive band of Bands 30, 31, 34 and 40. However, Band 30 is not intended for use in the same geographical area as Bands 1, 20and 42. Therefore, the focus here is on the 3rd order IMD products falling into Bands 31, 34 and 40. As B42 is a TDD band, simultaneous Tx/Rx for the same band will not happen. So the IMD and harmonic impact to B42 needs not to be considered

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 31, 34 and 40 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (1 + 20 + 42) BS transmitter does not share the same antenna with Bands 31, 34 and 40 BS receiver.

Therefore, it is recommended that Band (1 + 20 + 42) BS transmitter should not share the same antenna with Bands 31, 34 and 40 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 31, 34 and 40 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### ---Text omitted---

### 6.30.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 3 Band 20 and Band 42 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.30.2-1 below:

Table 6.30.2-1: Band 3, Band 20 and Band 42 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 791 | 821 | 1805 | 1880 | 3400 | 3600 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1004 |  |  | 699 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 2311 |  |  | 2616 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 4384 |  |  | 4689 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 5996 |  |  | 6301 |  |  |

It can be seen from Table 6.30.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 20 + 42) may fall into the BS receive band of Bands 5, 7, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 30, 38, 40, 41, 44 and 68. However, Band 13, 26, 27, 30 is not intended for use in the same geographical area as Bands 3, 20and 42. Therefore, the focus here is on the 3rd order IMD products falling into Bands 5, 7, 8, 12, 14, 17, 18, 19, 20, 28, 38, 40, 41, 44 and 68. As B42 is a TDD band, simultaneous Tx/Rx for the same band will not happen. So the IMD and harmonic impact to B42 needs not to be considered.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the 5, 7, 8, 12, 14, 17, 18, 19, 20, 28, 38, 40, 41, 44 and 68 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (3 + 20 + 42) BS transmitter does not share the same antenna with Bands 5, 7, 8, 12, 14, 17, 18, 19, 20, 28, 38, 40, 41, 44 and 68 BS receiver.

Therefore, it is recommended that Band (3 + 20 + 42) BS transmitter should not share the same antenna with Bands 5, 7, 8, 12, 14, 17, 18, 19, 20, 28, 38, 40, 41, 44 and 68 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 5, 7, 8, 12, 14, 17, 18, 19, 20, 28, 38, 40, 41, 44 and 68 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### ---Text omitted---

### 6.31.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 7 Band 20 and Band 42 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.31.2-1 below:

Table 6.31.2-1: Band 7, Band 20 and Band 42 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 791 | 821 | 2620 | 2690 | 3400 | 3600 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 189 |  |  | 89 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1501 |  |  | 1801 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 5199 |  |  | 5499 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6811 |  |  | 7111 |  |  |

It can be seen from Table 6.31.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (7 + 20 + 42) may fall into the BS receive band of Bands 3, 4, 9, 10, 24, 46 and 66. However, Bands 10 and 24 are not intended for use in the same geographical area as Bands 7, 20and 42. Therefore, the focus here is on the 3rd order IMD products falling into Bands 3, 4, 9, 24, 46 and 66. As B42 is a TDD band, simultaneous Tx/Rx for the same band will not happen. So the IMD and harmonic impact to B42 needs not to be considered

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 3, 4, 9, 24, 46 and 66 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (7 + 20 + 42) BS transmitter does not share the same antenna with Bands 3, 4, 9, 46 and 66 BS receiver.

Therefore, it is recommended that Band (7 + 20 + 42) BS transmitter should not share the same antenna with Bands 3, 4, 9, 24, 46 and 66 BS receiver if the aforementioned BS transmit configurations are used unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 3, 4, 9, 24, 46 and 66 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

##### ---End of changes---