nrameter in  SNR due to Baseband
nrameter in  SNR due to Baseband3GPP TSG-RAN WG4 Meeting #79                                                                         R4-163808

# Nanjing, China, 23rd -27th May, 2016

Source:	CMCC

Title:	ACS and blocking requirements for NB-IoT

Agenda Item:	          6.13.3.5

Document for:	Approval

- Introduction

RAN4 has agreed a WF on BS Rx requirement for NB-IoT [1]. ACS and blocking requirements for NB-IoT have some consensus after several meeting discussion. In this contribution, we discuss the open issues for ACS and blocking requirements.

- Discussion

2.1 ACS desensitivity

During the last meeting, the calculation of the ACS desensitivity has two suggestions. One proposal is based on 15 kHz and 3.75 kHz, the results are shown below [2].

“The ACS requirement shall consider single tone signal for 15 kHz and 3.75 kHz sub-carrier spacing.

BS noise floor for NB-IoT 15 kHz has following value (assuming 5 dB noise floor)

As agreed, the interferer mean power value PInterfererACS was set to -52 dBm..

Considering this, assuming again NB-IoT shall have similar performances (sensitivity) as LTE, and following LTE rationale as mentioned in [5]. We can then determine the required desensitisation for 15 kHz signal:

For 3.75 kHz, following the same rationale, it should then be set to 36.2 dB. ”

Another proposal is consider 180 kHz BW to calculate the ACS desensitivity [3].

Channel bandwidth : 180kHz

Interfering signal mean power: -52dBm

Pnoisefloor=-174dBm+10*log(180000)+5dB（Noise figure）=-116.5dBm

Desensitisation=19.5dB

The ACS desensitivity of two proposals can be seen as follows.

| BW of calculation | Desensitivity |
| --- | --- |
| Proposal1: 180 kHz | 19.5 dB |
| Proposal2: 15 kHz/3.75 kHz | 30.2 dB/36.2 dB. |

We think proposal 1 is more appropriate.

- ACS = 45 dB is based on the simulation assumption of 200 kHz BW both for victim channel and aggressor channel. If 15 kHz/3.75 kHz is used for calculation, equivalent ACS will not be 45dB anymore.

- Proposal 2 exceed agreed BS dynamic range, which is not reasonable.

- We should consider the function of baseband shaping filter, the impact on the single tone would be reduced.

For standalone operation, we proposed ACS requirement for NB-IoT standalone operation is as listed below.

Table 1 ACS requirement for NB-IoT standalone operation

| NB-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5 dB | -52 | ±100 | 200kHz NB-IoT signal |

Proposal#1: The ACS desensitivity should be no more than 19.5dB .

  - In-band blocking

For in-band and guard band operation, spectral density of noise rise caused by in-band blocking is the same, so desensitivity for NB-IoT and LTE are the same as well. Therefore, desensitivity for in-band blocking is proposed to keep as 6dB. Same requirement should also apply to standalone operation.

Proposal#2: Desensitivity for in-band blocking is proposed to be 6dB.

  - Narrow-band blocking

Regarding the large desensitisation of ACS, narrowband blocking requirement is proposed to guarantee the co-existence performance for standalone NB-IoT.

We prefer to reuse MSR narrow-band blocking requirement. Interfering signal for standalone narrow-band blocking requirement can use 3MHz LTE, and the interfering signal should be one PRB (180 kHz) with interfering signal mean power = -49 dBm as in table 2.

Table 2 Narrowband blocking requirement for standalone

| RAT of the carrier | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] |
| --- | --- | --- | --- |
| NB-IoT | PREFSENS + (3 - 6dB) | -49 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 |

Proposal#3: Interfering RB centre frequency offset of Narrow-band blocking for standalone is proposed to reuse MSR narrow-band blocking requirement.

For standalone operation, similar to ACS requirement rationale, dsensitivity for narrow-band blocking do not need to scale by 15 kHz or 3.75 kHz. If desensitivity is up to 30dB for 3.75 kHz [2], it surely degrades the performance of NB-IoT in field deployment.

As we discussed in last meeting, For E-UTRA narrowband blocking test, the wanted signal mean power is PREFSENS + 6dB; and for E-UTRA ACS test, the wanted signal mean power depends on the channel bandwidth. However, if we look at the MSR specification in TS 37.014 or GSM specification in TS45.005, the wanted signal mean power for GSM/EDGE is PREFSENS + 3dB [4]. Considering the implement margin, we suggest that desensitivity for narrow-band blocking requirement is proposed to be between 3 - 6dB.

Proposal#4: Desensitivity for narrow-band blocking is proposed to be between 3 – 6 dB.

- Conclusion

Proposal#1: The ACS desensitivity should be no more than 19.5dB.

Proposal#2: Desensitivity for in-band blocking is proposed to be 6dB.

Proposal#3: Interfering RB centre frequency offset of Narrow-band blocking for standalone is proposed to reuse MSR narrow-band blocking requirement.

Proposal#4: Desensitivity for narrow-band blocking is proposed to be between 3 - 6dB.

- Reference

[1] R4-162820, Way forward on BS RX requirements for NB-IoT, Huawei, Ericsson, CMCC

[2] R4- R4-78AH-0100, BS RF Rx ACS requirements, Ericsson

[3] R4- R4-78AH-0025, Discussion on remaining BS RX issues for NB-IoT, Huawei

[4] R4- R4-78AH-0113, ACS and blocking for standalone NB-IoT, CMCC