3GPP TSG-RAN WG4#79	 R4-164313

Nanjing, China, 23 - 27 May 2016

Agenda Item:	6.13.4.6

Source: 	Nokia

Title: 		NB-IoT Radio Link Monitoring

Document for:	Approval

# Introduction

The purpose of Radio Link Monitoring (RLM) is to monitor the downlink radio link quality of the serving cell(s). Legacy RLM requirements are defined based on monitoring the downlink cell-specific reference signals (CRS) quality and compares the estimated quality to the out-of-sync and in-sync thresholds Qout and Qin, which corresponding to 10% and 2% BLER of hypothetical PDCCH channels [1].

In WG4#78-NB-IoT, RLM for NB-IoT was discussed with the following proposals [2]:

- Proposal 1: Define a unified NB-IoT RLM requirements for all coverage levels in terms of the NPDCCH configurations (e.g., the number of the NPDCCH repetitions), i.e., not have separate  RLM requirements for different coverage levels;

- Proposal 2: Use the same NB-IoT RLM requirements for all UEs regardless configured on anchor carrier or non-anchor carrier;

- Proposal 3: Define the NB-IoT RLM requirements based only on N-RS in Rel-13.

- Proposal 4: Further discussion on whether to have separate NB-IoT RLM requirements for in-band and other operation modes.

While Proposal 3 was agreed, there were concerns on other proposals. During the meeting, it was proposed that [4]

- RAN4 is to specify minimum RLM requirements for different ranges of Rmax.

- Whether RAN4 is to specify different RLM requirements for the different deployment modes based on the simulations outcome is FFS.

- Whether there is any impact of power-boosting of NB-IoT NPDCCH/NRS RE compared to LTE CRS RE in in-band/guard band is FFS.

In this paper, we provide further discussion on above issues.

# Discussion

RLM requirements for different ranges of Rmax

In the endorsed CR for NB-IoT RLM, the NPDCCH transmission parameters were agreed that as shown in the following table:

Table 1 NPDCCH transmission parameters for out-of-sync and in-sync for NB-IoT UE

| Attribute | Out-of-sync | In-sync |
| --- | --- | --- |
| DCI format | Format N1 | Format N1 |
| Number of information bits | [22] bits | [22] bits |
| System Bandwidth | 200kHz | 200kHz |
| Antenna configuration | 1x1 or 2x1 [TBD] | 1x1 or 2x1 [TBD] |
| Maximum NPDCCH Repetition level | RmaxNote1 | Rmax/k Note1 and ‘k is [TBD] |
| Aggregation level | 2 | 2 |
| DRX | OFF | OFF |
| Deployment mode | [TBD] | [TBD] |
| NOTE 1: Rmax is a configurable parameter defined in TS 36.331[2]. |  |  |

Table 2 Mapping of Rmax and Qout and Qin Evaluation Period in non-DRX for HD-FDD NB-IoT UE

| RmaxNote1 | TEvaluate_Qout_NB-IoT | TEvaluate_Qin_NB-IoT |
| --- | --- | --- |
| RmaxNote1 ≤ [TBD] | TBD | TBD |
| RmaxNote1 > [TBD] | TBD | TBD |
| NOTE 1: Rmax is a configurable parameter defined in 36.331. |  |  |

As shown in above tables, the only differences between Out-of-sync and In-sync requirements are the parameters depending on the configured maximum repetitions.

For Table 1, the scale factor ‘k’ should be defined with enough SNR offset between Out-of-sync and In-sync thresholds. Based on our simulation [6], for NPDCCH with the same settings (e.g., the number of repetitions), there is about 5dB SNR difference between 10% NPDCCH BLER and 2% NPDCCH BLER. Consider the NB-IoT RSRP measurement error is defined larger than 5dB, it is clear that ‘k’ cannot be equal to 1. Also, based on our simulation results, every double of NPDCCH repetitions will generate about 2-3 dB SNR gap, depending on the fading channel assumption. In general, we will need to have the Out-of-sync and In-sync SNR offset to be larger than RSRP measurement error. On the other hand, the Qout and Qin thresholds cannot be too large gap, since if the gap between Qout and Qin thresholds is too large, the UE may never be able to re-enter in-synch state, and the UE will go into RLF whenever it declares Out-of-sync.

Proposal 1: Set Maximum NPDCCH Repetition level as Rmax /4 for the In-sync requirements;

NB-IoT RLM for Difference Operation Modes

A NB-IoT UE may work on one of the operation mode: in-band, guard-band and standalone mode. From RLM point of view, whether to define separate requirements for UE configured on different mode also depend on the whether there are significant performance differences in these operation modes.

- NPDCCH performance differences on different operation modes

For  NPDCCH, as mentioned before, the same DCI size for DL and UL is for all operation modes and all coverage cases. The main difference between the inband NB-PDCCH and the NB-PDCCH in other modes is that the first few OFDM symbols (1~3) are reserved for legacy LTE control region, and thus not available for inband NB-PDCCH, while all OFDM symbols are assumed to be available for NB-PDCCH in stand-alone and guard-band modes. Thus, the maximum performance difference between inband NPDCCH and the NPDCCH in other operation modes can be as high as 3/14(~20%), which is about 1dB difference. The impact of the 1dB difference on RLM requirements  may need further discussion, which also depends on the NRS measurement performance in NB-IoT. For example, if NRS measurement error can be small, e.g., 3 ~ 4 dB, then the 1dB NPDCCH performance difference will be significant; If, however,  NRS measurement error is much larger, e.g., > 6  dB, then the 1dB NPDCCH performance difference may not be so significant.

- NRS performance differences on different operation modes

As defined in RAN1, for In-band operation, NB-RS is present without condition. For Stand-alone and guard-band operations, only NB-RS is used. Thus, we would not expect significant NRS performance difference on different operation modes assume only NB-RS measurements are used for RLM,

The impact of the 1dB difference on RLM may depend on NRS measurement performance. Since the 1dB difference is much smaller than RSRP measurement error; we see no need to NB-IoT RLM for different operation modes.

Proposal 2: No need to separate RLM requirements for difference operation modes;

Impact of power-boosting of NB-IoT NPDCCH/NRS RE compared to LTE CRS

Since NB-IoT RLM is based only on the monitoring of NRS quality and the mapping between NRS quality to NPDCCH BLER, the power of CRS should have no impact on the NB-IoT RLM. Therefore, the power-boosting of NB-IoT NPDCCH/NRS RE compared to LTE CRS should have no impact on NB-IoT RLM.

Proposal 3: Power-boosting of NB-IoT NPDCCH/NRS RE compared to LTE CRS should have no impact on NB-IoT RLM.

# Summary

In this paper, we further discussed the NB-IoT RLM requirements. Based on the discussion, we have the following proposals:

Proposal 1: Set Maximum NPDCCH Repetition level as Rmax /4 for the In-sync requirements;

Proposal 2: No need to separate RLM requirements for difference operation modes;

Proposal 3: Power-boosting of NB-IoT NPDCCH/NRS RE compared to LTE CRS should have no impact on NB-IoT RLM.

# References

- 3GPP TS 36.133, E-UTRAN: Requirements for support of radio resource management

- R4-78AH-0164, RLM for NB-IoT UEs, Nokia

- R4-78AH-0221, Draft CR on Radio Link Monitoring for NB-IoT, Qualcomm

- R4-78AH-0220, WF on RLM for NB-IoT, Ericsson

- R4-16zzzz, RAN4#78AH-NB-IoT meeting report,

- R4-78AH-0167, Simulation Results for NB-IoT RLM, Nokia