3GPP TSG-RAN WG4 Meeting #79	R4-164348

# Nanjing, China, 23 - 27 May, 2016

Source:	Nokia

Title:	BS demodulation performance of TU600

Agenda item:	7.12.4

Document for:	Discussion

### 1	Introduction

Under the Rel-14 WI [1] of performance enhancement for high speed scenarios, there will be new BS performance requirements under ETU600, following the TR 36.878 [2].  The simulation assumptions are agreed in [3] to study PUSCH performance under ETU600.

In last RAN4 meeting, one WF [4] for BS MCS level is agreed to define BS ETU600 performance requirements with 16QAM rate-1/2 with 4Rx.  This contribution provides further details for the uplink ETU600 performance requirements.

### 2	FRC table for 16QAM rate 1/2

There is no legacy FRC table in RAN4 36.104 specifications to support 16QAM rate-1/2.  New FRC tables are needed for this MCS level.  In another WI on BS MMSE-IRC performance, two new FRCs are defined to support QPSK rate 0.36 and 16QAM rate ½ [5].  The new FRC table for 16QAM rate /12 is shown here based from [5].

Table A.13-1 FRC parameters for performance requirements (16QAM 1/2)

| Reference channel | A13-1 | A13-2 | A13-3 | A13-4 | A13-5 | A13-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 6 | 15 | 25 | 50 | 75 | 100 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 0.51 | 0. 50 | 0.50 | 0.49 | 0.50 | 0.49 |
| MCS index | 15 | 15 | 15 | 15 | 15 | 15 |
| Payload size (bits) | 1736 | 4264 | 7224 | 14112 | 21384 | 28336 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 2 | 3 | 4 | 5 |
| Coded block size including 12bits trellis termination (bits) | 5292 | 12876 | 10956 | 14220 | 16140 | 17100 |
| Total number of bits per sub-frame | 3456 | 8640 | 14400 | 28800 | 43200 | 57600 |
| Total symbols per sub-frame | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |

The ETU600 performance requirements can be defined based on the FRC table of 16QAM rate-1/2.

### 3	ETU600 simulation results

The simulation results shall be defined for all BW: 1.4, 3, 5, 10, 15, 20 MHz.  The simulation results are shown here:

| Bandwidth (MHz) | (Tx, Rx) configuration | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 1x4 | ETU 600Hz Low | A13-1 | 30% |  |
|  |  |  |  | 70% |  |
| 3 | 1x4 | ETU 600Hz Low | A13-2 | 30% |  |
|  |  |  |  | 70% |  |
| 5 | 1x4 | ETU 600Hz Low | A13-3 | 30% |  |
|  |  |  |  | 70% |  |
| 10 | 1x4 | ETU 600Hz Low | A13-4 | 30% | -3.6 |
|  |  |  |  | 70% | 4.2 |
| 15 | 1x4 | ETU 600Hz Low | A13-5 | 30% |  |
|  |  |  |  | 70% |  |
| 20 | 1x4 | ETU 600Hz Low | A13-6 | 30% |  |
|  |  |  |  | 70% |  |

A corresponding CR [6] is also presented to introduce the ETU600 performance requirements.

# Reference

[1]	RP-152263	“New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO, Huawei, HiSilicon, Dec 2015

[2]	3GPP TR 36.878	“Study on performance enhancements for high speed scenario in LTE”, Rel 13

[3]	R4-161255	“Way forward on high speed BS demodulation assumptions”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN4 #78

[4]	R4-163033	“Way forward on PUSCH ETU600 test”, Huawei, Nokia, Qualcomm,CATT, April 2016

[5]	R4-162794	“36.104 CR for BS MMSE-IRC receiver - FRC definitions”, China Telecom, Huawei, ZTE, April 2016.

[6]	R4-164349	“36.104 CR: Introduction of BS ETU600 performance requirements”, Nokia, April 2016